| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0454 WG11 Number: m24710 |
| --- | --- |

| Title: | AHG6: Test results on region adaptive ALF with 64 regions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Tomoo Yamakage, Takayuki Itoh, Takashi Watanabe, Takeshi Chujoh TOSHIBA 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, Japan Chia-Yang Tsai, Ching-Yeh Chen, Chih-Ming Fu, Yu-Wen Huang, Shawmin Lei MediaTek No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 In Suk Chong, Marta Karczewicz Qualcomm 5775 Morehouse Dr, San Diego, CA 92121, USA | Tel: Email: | Tomoo Yamakage +81-44-549-2152 {tomoo.yamakage, takayuki2.itoh, takashi39.watanabe, takeshi.chujoh}@toshiba.co.jp Shawmin Lei +886-3-5670766 ext. 25555 {chiayang.tsai, chingyeh.chen, chihming.fu, yuwen.huang, shawmin.lei}@mediatek.com Marta Karczewicz +1-858-658-5673 {ichong, martak}@qualcomm.com |
| Source: | TOSHIBA Corporation1, MediaTek2, Qualcomm3 |  |  |

_____________________________

# Abstract

There were two modes in ALF: region adaptive (RA) and block adaptive (BA) ALF in HM5/WD5. In AHG6, it is reported in JCTVC-I0157 that BA shows better coding efficiency than RA (0.2-0.3% on average) especially on Class A sequences. However, in comparison with RA, BA requires (1) additional feature computation to switch filters and (2) faster switching of filters (BA switches filters in each 4 x 4-block, while RA switches filters by some LCUs).

In this contribution, the maximum number of regions in RA is increased from 16 to 64. It is reported that even when the maximum number of regions in RA is increased, almost no additional gain is observed.

# Introduction

In this contribution, RA with up to 64 regions is tested in comparison with HM5-like RA (up to 16 regions). By increasing the maximum number of regions in RA, RA may switch filters in finer granularity especially with high resolution inputs.

# WD modifications

In RA mode in HM5, filter index array is derived by following:
1.	The variables xIdx and yIdx are derived as

regionTab[16] = { 0, 1, 4, 5, 15, 2, 3, 6, 14, 11, 10, 7, 13, 12, 9, 8 }

offset = 1 << (Log2MaxCUSize - 1)

xInterval = ( ( ( PicWidthInSamplesL + offset ) >> Log2MaxCUSize ) + 1 ) >> 2	(8 474)

yInterval = ( ( ( PicHeightInSamplesL + offset ) >> Log2MaxCUSize ) + 1 ) >> 2	(8 474)

xIdx = Min( 3, Floor( ( xC + x ) / ( xInterval << Log2MaxCUSize ) ) )

yIdx = Min( 3, Floor( ( yC + y ) / ( yInterval << Log2MaxCUSize ) ) )

2.	The filter index fIdx[ x, y ] with x, y = 0..(nS)-1 is derived as

fIdx[ x ][ y ] = regionTab[ ( yIdx << 2 ) + xIdx ]

In the 64 regions RA, filter index array is derived by following:
1.	The variables xIdx and yIdx are derived as

regionTab[64] = { 0,	3,	4,	5,	58,	59,	60,	63,
1,	2,	7,	6,	57,	56,	61,	62,
14,	13,	8,	9,	54,	55,	50,	49,
15,	12,	11,	10,	53,	52,	51,	48,
16,	17,	30,	31,	32,	33,	46,	47,
19,	18,	29,	28,	35,	34,	45,	44,
20,	23,	24,	27,	36,	39,	40,	43,
21,	22,	25,	26,	37,	38,	41,	42 }

offset = 1 << (Log2MaxCUSize - 1)

xInterval = ( ( ( PicWidthInSamplesL + offset ) >> Log2MaxCUSize ) + 2 ) >> 3

yInterval = ( ( ( PicHeightInSamplesL + offset ) >> Log2MaxCUSize ) + 2 ) >> 3

xIdx = Min( 7, Floor( ( xC + x ) / ( xInterval << Log2MaxCUSize ) ) )

yIdx = Min( 7, Floor( ( yC + y ) / ( yInterval << Log2MaxCUSize ) ) )

2.	The filter index fIdx[ x, y ] with x, y = 0..(nS)-1 is derived as

fIdx[ x ][ y ] = regionTab[ ( yIdx << 3 ) + xIdx ]

The syntax is changed as following:

| alf_param(){ |  |
| --- | --- |
| //... |  |
| for (i=1; i< 16 64; i++) |  |
| alf_filter_pattern[i] | u(1) |
| //... |  |
| } |  |

# Simulation results

Table 1 shows the simulation results. The CE2_Option2 software (see JCTVC-I0157) is used to comparison. The anchor uses RA in HM5, whereas the test uses RA with up to 64 regions. It is reported that almost no gain/loss is observed by increasing maximum number of regions in RA from 16 to 64.

# Patent rights declaration(s)

TOSHIBA corporation, MediaTek and Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

Table 1 Results for RA16 vs. RA64.

|  | All Intra HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.1% | 0.1% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | -0.2% | -0.1% | -0.1% |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access HE10 |  |  |
|  | Y | U | V |
| Class A | 0.0% | 0.1% | 0.1% |
| Class B | 0.0% | 0.1% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.1% | 0.1% |
| Class E |  |  |  |
| Overall | 0.0% | 0.1% | 0.1% |
|  | 0.0% | 0.1% | 0.1% |
| Class F | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B HE10 |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | 0.0% | 0.1% |
| Class C | 0.1% | 0.1% | 0.1% |
| Class D | 0.0% | -0.1% | 0.2% |
| Class E | 0.0% | -0.2% | -0.7% |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | -0.2% | -0.2% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay P HE10 |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | 0.1% | -0.3% |
| Class C | 0.1% | 0.1% | 0.4% |
| Class D | 0.0% | 0.4% | -0.1% |
| Class E | -0.3% | -0.2% | -0.2% |
| Overall | 0.0% | 0.1% | 0.0% |
|  | 0.0% | 0.1% | -0.1% |
| Class F | 0.2% | -0.5% | 0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |