| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0455 |
| --- | --- |

| Title: | Profiles and Levels in HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ajay Luthra 6420 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1 858 404 3470 aluthra@motorola.com |
| Source: | Motorola Mobility |  |  |

_____________________________

# Abstract

This contribution proposes creating three categories of the Profiles: low complexity, medium complexity with very good coding efficiency and a third profile with the highest coding efficiency with increased complexity. This will allow implementation of HEVC on a wide range of devices, from handhelds (e.g. smart phones, tablets, laptops), to very high end PCs, to set top boxes and TVs, to Blu-ray players. Those devices have very diverse implementation architectures, where some of them have ASICs based decoders, others use decoders with some hardware assists and many of them decode the video using software only decoders. In addition, as it is seen in AVC, there are three main application areas - distribution (broadcasting, streaming, etc), creation (contribution quality in studios) and storage (Blu-Ray etc). Even with in content creation area, there are two or three sub-parts utilizing different coding structures: Intra only, short GOP (IBIB ...) and long GOP (IBBPBBP ...). These applications need and can support widely varying attributes associated with the Level, e.g. maximum bit rates and buffer sizes etc. However, HEVC draft text (JCTVC-H1003) defines only one table (A-1) associated with the Levels for all those diverse applications. Due to the onion structure of the Level hierarchy, it becomes hard to support those wide ranges of applications with one table for all. It is therefore proposed to create more than one tables associated with the Levels.

# Introduction

The HEVC (High Efficiency Video Coding) standard consists of various coding tools each with varying complexity and coding gain. To promote interoperability Profiles and Levels are defined in the standard corresponding to the needs of various sets of applications. Devices used in those applications range from handheld devices (e.g. smart phones, tablets, laptops), to very high end PCs, to set top boxes and TVs, to Blu-ray players. Those devices have very diverse implementation architectures. Some of the devices may have ASICs based decoders, others use decoders with some hardware assists and many of them decode the video using software only decoders. Therefore, specification of only one profile, as in the current draft text (JCTVC-H1003), may not be satisfactory and may limit the adoption of HEVC standard.

Similarly, due to a wide range of applications ranging from distribution to consumers, to storage, to content creation, to very high quality content distribution etc., specification of only one table corresponding to the attributes associated with the Levels is not sufficient.

# Profiles

It is proposed to group the coding tools in HEVC in three different profiles – (1) Low complexity, (2) Medium complexity with very good coding efficiency and (3) Highest possible coding efficiency achievable by using the tools in HEVC with increased complexity.

## Profile 1 (Baseline) – Low Complexity

- Allows the use of up to 16x16 size transforms (4x4, 8x8 and 16x16).

- Allows up to 16x16 size prediction units including 4x4 (inter_4x4_enabled_flag equal to 1).

- Only De-blocking in-loop filter (SAO and ALF not allowed).

- Only progressively scanned or frame pictures are allowed.

- Weighted prediction shall not be applied to both P and B slices.

- Need to add constraint on maximum number of slices and tiles.

## Profile 2 – Medium Complexity

- Same as currently defined Main except the use of tiles is not allowed.

- Only progressively scanned or frame pictures are allowed.

- Need to add constraint on maximum number of slices.

### Profile 2a – Field pictures

- Create Profile 2a corresponding to field pictures.

- Same attributes as Profile 2 except allow pictures (progressively scanned or interlaced scanned) to be coded as fields.

- Profile 2a compliant decoder is required to decode the bit stream compliant with Profile 2 but not vice-versa.

## Profile 3 – Highest coding efficiency

- All transform sizes are allowed.

- All sizes of prediction units are allowed (including 4x4).

- All three in-loop filters – De-blocking, SAO and ALF – are allowed.

- NSQT, AMP allowed.

- Only progressively scanned or frame pictures are allowed.

- Consider inclusion of higher internal bit depth.

- Use of tiles is not allowed.

- Need to add constraint on maximum number of slices.

# Levels

## Changes in currently defined Levels

Level 3: This level is targeted towards Standard Definition as in AVC. However, looking forward QHD (960x540) will potentially become more popular Standard Definition, in comparison to for example 720x480x30i, in many applications involving Tablets and Smart phones. But, that is not included in the Level 3. Next Level 3.1 goes to QHDx60. Consider increasing the Max Luma Size and Max Luma Pixel rate in Table A-1 to also include QHDx30. Also, consider increasing maximum bit rate to 6 Mbps.

Level 4.2: It covers 1080p60. It has the maximum bit rate (MaxBR) of 30 Mbps. Today, with AVC, 1080p60 is targeted to be in the range of 12 to 15 Mbps for distribution to consumers. If we accept the 2x coding efficiency of HEVC, expected range of 1080p60 operation is less than 10 Mbps. Maximum bit rate of 30 Mbps looks to be too high for the applications associated with the distribution of entertainment TV to consumers via satellite, cable etc.  Considering that Level 4.3 allows one to go at higher rate (for the applications that may want to use higher rate), suggestion is to lower the maximum bit rate of Level 4.2 to 20 Mbps. This also implies lowering the rate of Level 4.1 (which may imply removing it).

Level 5.1: There is no difference between Level 5 and 5.1 except the maximum bit rate. It increases from 50Mbps to 100 Mbps, and yet the minimum compression ratio (MinCR) goes up from 6 to 8. The minimum compression ratio (MinCR) for 5.1 should be lowered to 4.

Level 5.2: This includes 4kx2kx60p - a targeted area for HEVC for SHDTV. It has the maximum bit rate of 150Mbps. That is too high for the applications associated with the distribution of entertainment TV to consumers via satellite, cable etc. Suggestion is to lower it to around 50Mbps or less. And, if needed, may be a new level 5.3 can be introduced for those applications that may need higher rate. This also implies re-looking at 5.1 (either removing it or going away from the Onion structure in the Levels also).

HEVC should consider adding MV range limit and equivalent of "Max number of motion vectors per two MBs" in AVC columns in Table A-1.

## More than one Level tables

As is also the case with AVC, there are three main classes of HEVC applications - distribution (broadcasting, streaming, etc), contribution/studio and storage (Blu-Ray etc). Even with in contribution side, there are two or three sub-parts: Intra only, short GOP (IBIB ...) and long GOP (IBBPBBP ...). These applications need and can support widely varying maximum bit rates. So, it will be worthwhile thinking about not having the same Level attributes (e.g. maximum bit rates, buffer sizes and maximum picture sizes) for all and to have different tables equivalent to A-1. Due to the Onion structure of the Level hierarchy, it becomes hard to support wide range of applications with one table A-1 for all. It is time to think about supporting more than one such Table and break the Onion model across those application sets. We started to move in that direction by having overloaded columns like MaxBR, MaxCPB etc. in AVC. But, that was mainly tied to Chroma resolution and bit depth. It did not take into consideration that various applications have different quality targets and different typical GOP structures. For example, in studios one may typically use I-only or IBIB… type structures, do more than one generation of compressions and have higher quality requirement leading to the bit rates significantly higher than those used for the distribution to consumers over satellite or cable. If we think this way, it will make compliance lot easier by not forcing one set of applications to support extremely high maximum bit rates needed by another set. Otherwise, it will lead to the deployment of the decoders that are not fully compliant with the standard (as happened in the past).

# Patent rights declaration(s)

Motorola Mobility may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).