| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0461 |
| --- | --- |

| Title: | On HEVC still picture coding performance |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jani Lainema Kemal Ugur | Email: | jani.lainema@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution provides information about the HEVC still picture coding performance. The intra coding of HEVC is reported to provide on average 56 % objective bitrate reduction over a widely utilized JPEG implementation. It is further reported that HEVC intra coding achieves a 34 % average objective bitrate reduction over the official JPEG-XR reference implementation. The differences are reportedly larger for Class F content (67 % and 48 % with respect to JPEG and JPEG-XR, respectively).

# Test set-up

The following software implementations were used to measure performance of HEVC, JPEG and JPEG-XR standards:

| Standard | Software implementation |
| --- | --- |
| HEVC | HM 6.0 |
| JPEG | ImageMagick 6.3.7 08/09/09 Q16 |
| JPEG-XR | ITU-T T.835 (01/10) reference implementation |

HM 6.0 was configured to use the Main profile settings, ImageMagick used its default settings for JPEG compression and the JPEG-XR was configured as follows:

| Switch | Explanation |
| --- | --- |
| -f YUV420 | YUV420 internal color format |
| -d | U/V quant derived from Y channel |
| -P 111 | Advanced profile |

JCT-VC common test conditions [1] were used with the following modifications: only first picture of each of the sequence was coded and 10-bit sequences were excluded from the tests. In addition the Class B sequences were excluded from the comparisons with JPEG-XR as the latest published JPEG-XR reference implementation (01/2010) was not able to handle the Class B resolution.

The quantization / quality parameters were selected according to the table below:

| Standard | Quality parameters used |
| --- | --- |
| HEVC | 22, 27, 32, 37 |
| JPEG | 85, 60, 30, 10 |
| JPEG-XR | 30, 50, 70, 80 |

# Test results

The tables below summarize the results comparing HEVC Main profile intra coding against the tested JPEG and JPEG-XR implementations:

Table 2.1: BD bitrate reduction provided by HEVC with respect to JPEG

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -47.3% | -58.6% | -60.3% |
| Class B | -55.8% | -62.8% | -63.7% |
| Class C | -57.1% | -61.3% | -60.1% |
| Class D | -55.2% | -53.0% | -59.4% |
| Class E | -63.4% | -65.0% | -62.8% |
| Overall | -56.3% | -60.2% | -61.4% |
|  | -56.3% | -60.1% | -61.3% |
| Class F | -66.6% | -57.8% | -56.6% |

Table 2.2: BD bitrate reduction provided by HEVC with respect to JPEG-XR

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -30.1% | -41.1% | -42.3% |
| Class B | N/A | N/A | N/A |
| Class C | -33.2% | -44.3% | -45.3% |
| Class D | -31.6% | -43.1% | -45.0% |
| Class E | -41.9% | -54.0% | -55.1% |
| Overall | -34.3% | -45.7% | -47.0% |
|  | -34.1% | -47.0% | -49.6% |
| Class F | -47.8% | -51.6% | -54.4% |

# Visual quality comparison

Some examples of the visual quality provided by the three codecs analysed in this study can be found below. Bitrates reported in Figures 1 and 2 assume the frame rate of the original sequence (60 fps for KristenAndSara, 30 fps for Racehorses). Figures 3 and 4 provide examples outside of the JCT-VC testset utilizing well known test images for still image compression [2][3].

Figure 1: Extracts of KristenAndSara, frame number 0. Left HEVC at 7.9 Mbps (0.14 bpp), middle JPEG-XR at 8.0 Mbps (0.14 bpp) and right JPEG coded picture at 8.2 Mbps (0.15 bpp).

Figure 2: Extracts of Racehorses_big, frame number 0. Left HEVC at 6.2 Mbps (0.51 bpp), middle JPEG-XR at 6.3 Mbps (0.52 bpp) and right JPEG coded picture at 6.7 Mbps (0.56 bpp).

Figure 3: Extracts of Lena. Left HEVC at 74 kbits (0.28 bpp), middle JPEG-XR at 76 kbits (0.29 bpp) and right JPEG coded picture at 80 kbits (0.31 bpp).

Figure 4: Extracts of Kodim07. Left HEVC at 131 kbits (0.33 bpp), middle JPEG-XR at 134 kbits (0.34 bpp) and right JPEG coded picture at 130 kbits (0.33 bpp).

# References

[1] F. Bossen, “Common test conditions,” JCT-VC document H1100, San Jose, March 2012.

[2] Lena image, as available at http://sipi.usc.edu/database/database.php?volume=misc

[3] Kodim07 image, as available at http://r0k.us/graphics/kodak/

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).