| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0462 |
| --- | --- |

| Title: | AHG4: Profile Requirements For Facilitation Of Parallel Tile Decoding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Stewart Worrall Aspex House, 44-45 Oxford Street, High Wycombe, Buckinghamshire, HP11 2DJ | Tel: Email: | +44 1494 558121 stewart.worrall@aspex-semi.com |
| Source: | Aspex Semiconductor |  |  |

_____________________________

# Abstract

The current CD defines a single restriction on tiles: tiles should be greater than 384 pixels in width. The current restriction can only be used by applications that include capability negotiation (e.g. videoconferencing), so that the encoded bitstream can be tailored to the number of cores on the encoder and decoder. To allow parallel decoding for streaming and broadcast applications, tighter restrictions are needed to facilitate flexible encoding and decoding with different numbers of cores. This contribution proposes some restrictions on tile sizes, which are intended to allow parallel decoding by as wide a range of applications as possible.

# Introduction

Communication applications will likely be able to exchange information on the number of available cores, allowing an encoder to choose a tile configuration to suit both encoder and decoder. For broadcast and streaming systems, this is not an option. Some of these issues are discussed in [1].

To enable parallel decoder support in a wide range of applications it is necessary to restrict tile sizes. To do this there are two obvious approaches, which can basically be categorised as using many tiles or few tiles:

- Set level related constraints mandating that a limited number of tiles be used at a particular level. This will allow decoder implementers to plan the level of parallelism based on the level that they intend to support. Although slightly different levels of parallelism can be exploited at the encoder and decoder, flexibility is severely limited. However, this is potentially the most compression efficient option, as it may be implemented using a limited number of tiles.

- Use enough tiles to allow encoders and decoders to flexibly process groups of tiles. This solution will allow significantly different levels of parallelism at the encoder and decoder. This is the most flexible solution, but reduces compression efficiency. This solution can either be set related to levels or can be set independently.

The first option will actually restrict parallelism, potentially to levels below those already exploited by real-time AVC encoders. Therefore, the second option seems to be the most practical, and the most future-proof (implementers may want to use many more cores in the future than are currently used). It also enables tiles to achieve a similar level of flexibility to Wavefront Parallel Processing.

Note that it is also preferable to avoid setting tile sizes/numbers with direct links to levels, as the level settings specify maximum values. This means that implementers may choose to work around the profiles and levels by encoding at a higher level than necessary for a particular resolution video, so that more tiles can be used.

# Profile Definitions

As demonstrated by the parallel processing efficiency results in [2], good efficiency can be achieved by both the encoder and decoder if the number of tiles is approximately equal to the number of Coding TreeBlock (CTB) lines (for CTB size of 64). Therefore, the proposed tile restrictions have been based on achieving a similar number of tiles to PicHeightInCtbs.

The proposed restrictions for the Main Profile are as follows:

uniform_spacing_flag = 1

ColumnWidthInLumaSamples[ i ] shall be greater than or equal to 384 for any i in the range of 0 to num_tile_columns_minus1, inclusive.

num_tile_columns_minus1 = Floor( PicWidthInCtbs/(6 * 26-Log2CtbSize ) ) - 1

The above restrictions constrain the tile width, now we need to constrain the tile height to get a similar number of tiles compared to the number of 64 CTB rows in a picture.

The following constraints shall be obeyed:

(num_tile_columns_minus1 + 1) * (num_tile_rows_minus1 + 1) < Ceil(pic_height_in_luma_samples / 64)

ColumnHeightInLumaSamples[ i ] shall be greater than or equal to 256 for any i in the range of 0 to num_tile_rows_minus1, inclusive.

In addition to these level independent constraints, the following constraint should be set: tiles shall only be used for Levels higher than or equal to 3.1.

Example tile column and rows specified by these constraints are shown in Table 1. The specified tile rows and columns are similar to those evaluated in [2].

Table 1: Example tile columns and rows specified by the proposed restrictions.

| Class | LumaWidth | LumaHeight | num_tile_columns | num_tile_rows |
| --- | --- | --- | --- | --- |
| A | 2560 | 1600 | 6 | 5 |
| B | 1920 | 1080 | 5 | 4 |
| C | 832 | 480 | 2 | 2 |
| E | 1280 | 720 | 3 | 3 |

# Conclusions

The proposed restrictions enforce a column width constraint, and ensure that a tile height is chosen that is sufficient to allow efficient parallel processing at different levels of parallelism. It is also proposed that tiles should not be used unless the Level is higher than or equal to 3.1.

# References

- W. Wan, T. Hellman, “Comments on specifying tile support in HEVC Profiles and Levels”, JCTVC-H0463, JCTVC 8th Meeting: San José, CA, USA, 1–10 February, 2012.

- S. Worrall, A. Wise, “AHG8: Inclusion Of Parallel Processing Schemes In The Main Profile”, JCTVC-I0153, JCTVC 9th Meeting: Geneva, CH, 27 April – 7 May 2012.

# Patent rights declaration(s)

Aspex Semiconductor does not have any current or pending patent rights relating to the technology described in this contribution.