| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0466 |
| --- | --- |

| Title: | Cross-check of Sign data hiding simplification and enhancement (JCTVC-I0326) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jun Xu 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4086 jun.xu@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

This document reports cross-verification for Qualcomm’s contribution JCTVC-I0326 on sign data hiding simplification and enhancement. The software implementation matches with the description in the contribution document. The simulation results match with the results provided by proponents in BD-rate numbers. Encoding and decoding time information is available for common test conditions.

# Introduction

In JCTVC-I0326, methods are proposed to simply or improve the sign data hiding technique in HM6.0. As a cross-checker, we studied the document and software implementation. Simulations are set up to verify the results for Method 1 and Method 2.

## Method 1

Method 1 in JCTVC-I0326 simplified HM6.0 by changing the condition for sign data hiding. Method 1 uses the last nonzero position directly instead of the distance between the first and last nonzero coefficients. The software follows the description of the algorithm Method 1. Changes are made in TComTrQuant.cpp, TDecSbac.cpp and TEncSbac.cpp.

## Method 2

Method 2 is based on Method 1. In addition, Method 2 changes the hidden sign from the first nonzero coefficient to the last nonzero coefficient in each CG. The software follows the description of the algorithm Method 2.

# Results

For each method, 2 sets of results are tested. One is under common test conditions [1], the other is common test conditions with RDOQ off. All results match with proponents’ results in BD-rate numbers. Time information is only available for common test conditions, in which case encoding and decoding time is close to that of proponents’ results. Detailed results are available in accompanying excel files.

# Reference

- F. Bossen, “Common HM test conditions and software reference configurations”, JCTVC-H1100, San Jose, February 2011.