| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0469 |
| --- | --- |

| Title: | CE3: Cross-check of Subtest 3 on Parallelization and Throughput |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Nguyen Nguyen, Tianying Ji, Dake He and Gaëlle Martin-Cocher 295 Phillip St. Waterloo, ON, Canada N2L 3W8 | Tel: Email: | +1-519-8887465 nnguyen@rim.com tiji@rim.com dhe@rim.com gmartincocher@rim.com |
| Source: | Research In Motion Ltd. |  |  |

_____________________________

# Abstract

This document presents the results of the cross-check performed on CE3 Subtest 3 Parallelization and Throughput for JCTVC-H0228.  The updated software of JCTVC-H0228 punctures the template to allow two bins to be (de)coded in parallel, and provides a BD-rate gain of between 0.1% and 0.3%.  Software integrating JCTVC-H0228 and JCTVC-H0427 was used to study three and four bins parallelism.  For three bins parallelism, the BD-rate vs. HM-6.0 is between -0.1% and 0.2%, and for four bins parallelism, the BD-rate vs. HM-6.0 is between 0.2% and 0.4%.  Further analysis of JCTVC-H0228 shows that it may require additional memory accesses for significance flag context derivation.

# Introduction

JCTVC-H0228 [1] proposes a transform coefficient coding cleanup, which provides an average BD-rate gain over HM-5.0 of between 0.7% and 1.1%.  JCTVC-H0228 proposes:

- Using a template of neighbouring coefficients to derive contexts for significance, greater-than-1, greater-than-2 flags, and the Rice parameter in all TU sizes

- Making the contexts for significance, greater-than-1, greater-than-2 flags, and the Rice parameter dependent on the absolute levels of the coefficients in the template

- Using the same contexts for greater-than-1 and greater-than-2 flags

- Interleaving significance, greater-than-1, greater-than-2 flags, and remaining level information and doing a single scan pass

One of the concerns raised by experts at the 8th JCTVC meeting in San Jose was that in its current form, JCTVC-H0228 limits parallelism: only one bin can be (de)coded at a time.  The purpose of CE3 Subtest 3 on Parallelization and Throughput (as described in [2]) is to study how this can be improved, and how such improvements impact coding efficiency.

# Results and Analysis

## Two Bins Parallelism

The proponents of JCTVC-H0228 provided software in which JCTVC-H0228 is integrated into HM-6.0 and modified to introduce a hole in the template, depending on the scan order. This template hole removes the dependency of the contexts for the significance, greater-than-1 and greater-than-2 flags of a coefficient on the immediately preceding coefficient in reverse scan order.  As a result, the template hole better facilities parallel processing, allowing two bins to be (de)coded in parallel.

We verified that the provided software matches the text description of JCTVC-H0228, and that it introduces the intended hole in the template.  Our BD-rate results match those provided by the proponents.  For two bins parallelism, in the common test conditions [3], JCTVC-H0228 with the template hole modification provides an average BD-rate gain over HM-6.0 of between 0.1% and 0.3%.

## Three and Four Bins Parallelism

As part of CE3 Test 3 [4], the proponents of JCTVC-H0427 [5] provided software in which JCTVC-H0427 and JCTVC-H0228 are both integrated into HM-6.0.  JCTVC-H0427 modifies the template to allow either three or four significance flag contexts to be (de)coded in parallel.

We used this software to study the behaviour of JCTVC-H0228 for three and four bins parallelism.  We verified that the provided software matches its intention.  Our BD-rate results for class A, B, C, D and E sequences match those provided in CE3 Test 3.  For three bins parallelism, in the common test conditions, the average BD-rate of the integrated software vs. HM-6.0 is between -0.1% and 0.2%.  For four bins parallelism, in the common test conditions, the average BD-rate of the integrated software vs. HM-6.0 is between 0.2% and 0.4%.

The integrated software de-interleaves the bitplanes (i.e., significance, greater-than-1, greater-than-2 flags, etc.) and does multiple scan passes.  This increases complexity because for each coefficient, the sum of the neighbouring levels must be computed twice: once for significance flag context derivation and once for greater-than-1/greater-than-2 flag context derivation.

## Memory Complexity

Our analysis of JCTVC-H0228 reveals that it may have a problem with respect to memory complexity.

In HM-6.0, to determine the context for a given significance flag, the significance flags of up to five neighbouring coefficients must be referenced.  Since significance flags are binary, 32 bits are required for significance map context derivation in a given coefficient group.  On a 32-bit platform such as ARM, this amounts to one register.

On the other hand, in JCTVC-H0228, the context for a given significance flag depends on the magnitudes of up to five neighbouring coefficients.  If these magnitudes are stored as 16 bit integers, 512 bits are required for significance map context derivation in a given coefficient group.  Consequently, additional memory accesses may be required to (de)code each significance flag.

# References

- T. Nguyen, D. Marpe, and T. Wiegand, “Non-CE11: Proposed Cleanup for Transform Coefficient Coding”, JCTVC-H0228, JCT-VC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 8th Meeting, San Jose, February 2012.

- V. Sze, T. Nguyen, K. Panusopone, Y. Piao, J. Sole, “Description of Core Experiment (CE3): Coefficient scanning and coding”, JCTVC-H1103, JCT-VC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 8th Meeting, San Jose, February 2012.

- F. Bossen, “Common HM test conditions and software reference configurations”, JCTVC-G1200, JCT-VC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 7th Meeting, Geneva, November 2011.

- J. Sole, V. Seregin, W.-J. Chien, J. Chen, and M. Karczewicz, “CE3 test 3: Parallelization with bit-plane coding (H0228)”, JCTVC-I0324, JCT-VC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 9th Meeting, Geneva, April 2012.

- V. Seregin, J. Sole, and M. Karczewicz “Non-CE11:  Support modification for parallel calculation of significant map contexts”, JCTVC-H0427, JCT-VC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 8th Meeting, San Jose, February 2012.

# Patent rights declaration(s)

Research In Motion Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).