| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I00475 |
| --- | --- |

| Title: | Considerations for level definitions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Teruhiko Suzuki 2-10-1, Osaki, Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2740 teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

In CD of HEVC (JCTVC-H1003), the levels are defined with onion ring structure. In level 4.0 and higher, two levels are defined for the same picture size (Max luma sample rate). Only max bit rate is different for those two levels. In order to keep onion ring structure, bitrate for higher resolution video can not be defined lower than that for lower resolution video. However there could be a demand to send higher resolution video at low bitrate. This contribution proposes to consider such onion ring structure for level. Instead of defining two levels with different bitrate for the same picture size, this contribution propose to add a flag at SPS to indicate the bit rate is higher/lower than a level table.

# Problems

The current levels in HEVC are defined with onion ring structure, and higher level decoder needs to decode lower level bitstreams. Therefore, bitrate at higher level must be equal or higher than that at lower levels. However there could be a demand to send higher resolution video at low bitrate. For example, high end HDTV authoring systems need to support higher bitrate, while 4K distribution or broadcast can not use such high bit rate due to the bandwidth limitation. The following is my observation of the current level definitions.

- Since the standard specifies decoder spec, profile/levels should be defined based on the complexity of decoder. If decoder complexity is not so different, it is better to keep higher quality.

- For example, both level 4.2 and 4.3 support 1080@60p and bitrate are 30 Mbps and 50 Mbps respectively. The complexity of entropy coder is not so different between 30M and 50M. The complexity of CABAC is not so high compared with AVC.

- Bit rate of the current level 4.1, 4.3, 5.1 are defined as 2/3 of AVC. This was proposed by JCTVC-H0168.

- It is not necessary to put upper bound of the quality by standard. Especially the bit rate constraints at which picture quality become lower than existing standard, e.g. AVC, should not be introduced.

- In AVC, only HDTV is the format in which two bit rates are defined by level. Level 4.0 is for broadcasting and level 4.1 is for storage media, e.g. Blu-Ray Disc. However, the target bitrates for high end and affordable applications are overlapped for HDTV (30p/60p), 4K@30p in very near future.

The current HEVC level is defined by the following table A-1.

Table A-1 – Level limits

| Level | Max luma pixel rate MaxLumaPR (samples/sec) | Max luma picture size MaxLumaFS (samples) | Max bit rate MaxBR (1000 bits/s) | Min Compression Ratio MinCR | MaxDpbSize (picture storage buffers) | Max CPB size (1000 bits) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 552,960 | 36,864 | 128 | 2 | 6 | 350 |
| 2 | 3,686,400 | 122,880 | 1,000 | 2 | 6 | 1,000 |
| 3 | 13,762,560 | 458,752 | 5,000 | 2 | 6 | 5,000 |
| 3.1 | 33,177,600 | 983,040 | 9,000 | 2 | 6 | 9,000 |
| 4 | 62,668,800 | 2,088,960 | 15,000 | 4 | 6 | 15,000 |
| 4.1 | 62,668,800 | 2,088,960 | 30,000 | 4 | 6 | 30,000 |
| 4.2 | 133,693,440 | 2,228,224 | 30,000 | 4 | 6 | 30,000 |
| 4.3 | 133,693,440 | 2,228,224 | 50,000 | 4 | 6 | 50,000 |
| 5 | 267,386,880 | 8,912,896 | 50,000 | 6 | 6 | 50,000 |
| 5.1 | 267,386,880 | 8,912,896 | 100,000 | 8 | 6 | 100,000 |
| 5.2 | 534,773,760 | 8,912,896 | 150,000 | 8 | 6 | 150,000 |
| 6 | 1,002,700,800 | 33,423,360 | 300,000 | 8 | 6 | 300,000 |
| 6.1 | 2,005,401,600 | 33,423,360 | 500,000 | 8 | 6 | 500,000 |
| 6.2 | 4,010,803,200 | 33,423,360 | 800,000 | 6 | 6 | 800,000 |

# Proposal

Based on this observation, it may be difficult to keep onion ring structure for bitrate. Alternative approach to specify level is to introduce a kind of "constrainted_flag". For example, this flag is equal to 1, it means bitarte is N/M of table A-1. Then table A-1 can be simplified. The example of the syntax change is shown in the following table.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| constraint_bitrate_flag | u(1) |
| reserved_zero_7bits /* equal to 0 */ | u(8) |
| level_idc | u(8) |
| seq_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| … |  |

# Patent rights declaration(s)

Sony Corporation does not have any current or pending patent rights relating to the technology described in this contribution.