| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0476 |
| --- | --- |

| Title: | Cross-check of Simplified Alpha Bit-depth Restriction in Chroma LM Mode (JCTVC-I0178) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yi-Jen Chiu 2200 Mission College Blvd. Santa Clara CA USA 95054 | Tel: Email: | 408-765-9494 yi-jen.chiu@intel.com |
|  | Wenhao Zhang 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171657 wenhao.zhang@intel.com |
|  | Lidong Xu 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171425 lidong.xu@intel.com |
|  | Yu Han 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171388 yu.han@intel.com |
| Source: | Intel Corp. |  |  |

_____________________________

# Abstract

This contribution provides the cross-check results of the Simplified Alpha Bit-depth Restriction in Chroma LM Mode proposed by Hong Kong University of Science and Technology in JCTVC-I0178. Two schemes were proposed in JCTVC-I0178 and the second scheme was verified by the cross checker. The software provided by the proponent was checked and tested. The cross-check result confirms the result reported by proponent.

# Introduction

Integer implementation of LM mode in HM6.0 utilizes the following equations to calculate the linear parameter:

a3	= 	a2s < 1 ? 0 : Clip3( −215, 215−1, a1s*lmDiv + ( 1 << ( k1 − 1 ) ) >> k1 ) 	(1)
a	= 	a3 >> Max( 0, Log2( abs( a3) − 6 ) )	(2)

In equation (1), a3 is restricted into −215 to 215−1. And in equation (2), a3 is right shifted to generate a, which is a 7-bit signed integer. In order to avoid the max judgment in equation (2), JCTVC-I0178 proposed two schemes to simplify the calculation of a, by setting the last B bits of a3 to zero.

Scheme 1:

a3	= 	a2s < 1 ? 0 : Clip3( −215, 215−1, a1s*lmDiv + ( 1 << ( k1 − 1 ) ) >> k1 ) 	(3)
a4	= 	a3 & ( ~( 2B-1 ) ) 	(4)

a	= 	a4 >> Log2( abs( a4 ) − 6 ) 	(5)

Scheme 2:

a3	= 	a2s < 1 ? 0 : Clip3( −215-B, 215-B−1, a1s*lmDiv + ( 1 << ( k1 − 1 ) ) >> k1 ) 	(6)
a4	= 	a3 << B 	(7)

a	= 	a4 >> Log2( abs( a4 ) − 6 ) 	(8)

# Experimental results

The software package of scheme 2 obtained from the proponent was successfully compiled and tested under the common test conditions [1]. AI_HE10 and AI_MAIN + LMChroma were simulated to test the results when B is equal to 5, 6, 7 and 8. There was no mismatch between encoder and decoder. It was confirmed that the R-D performance produced by this cross-check perfectly matches the results provided by the proponent. The encoding time and decoding time are also confirmed. The summarized performance data is shown in Table 1-4. Detailed excel sheets are attached.

Table 1 Performance data of simplified alpha bit-depth restriction with B = 5

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.9% | -0.9% | 0.0% | -1.0% | -1.3% |
| Class B | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class C | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |
| Overall | 0.0% | -0.2% | -0.2% | 0.0% | -0.2% | -0.2% |
|  | 0.0% | -0.2% | -0.2% | 0.0% | -0.2% | -0.2% |
| Class F | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 2 Performance data of simplified alpha bit-depth restriction with B = 6

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -2.1% | -2.0% | 0.0% | -2.5% | -3.0% |
| Class B | 0.0% | -0.1% | -0.1% | 0.0% | -0.2% | -0.1% |
| Class C | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.1% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% | 0.1% |
| Overall | 0.0% | -0.5% | -0.4% | 0.0% | -0.6% | -0.6% |
|  | 0.0% | -0.4% | -0.4% | 0.0% | -0.6% | -0.6% |
| Class F | 0.0% | -0.1% | 0.1% | 0.0% | 0.1% | 0.3% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 3 Performance data of simplified alpha bit-depth restriction with B = 7

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -3.9% | -3.9% | 0.0% | -4.3% | -4.8% |
| Class B | 0.0% | -0.2% | -0.1% | 0.0% | -0.3% | -0.2% |
| Class C | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class D | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class E | 0.0% | 0.0% | 0.2% | 0.0% | -0.1% | 0.2% |
| Overall | 0.0% | -0.9% | -0.8% | 0.0% | -1.0% | -1.0% |
|  | 0.0% | -0.9% | -0.8% | 0.0% | -1.0% | -0.9% |
| Class F | 0.0% | 0.0% | 0.2% | 0.1% | 0.0% | 0.5% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

Table 4 Performance data of simplified alpha bit-depth restriction with B = 8

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -5.5% | -5.6% | 0.1% | -6.1% | -6.4% |
| Class B | 0.0% | -0.3% | -0.1% | 0.0% | -0.4% | -0.2% |
| Class C | 0.0% | 0.0% | 0.2% | 0.1% | 0.1% | 0.3% |
| Class D | 0.0% | -0.1% | 0.1% | 0.0% | -0.1% | 0.1% |
| Class E | 0.0% | 0.0% | 0.5% | 0.0% | 0.0% | 0.5% |
| Overall | 0.0% | -1.2% | -1.0% | 0.0% | -1.3% | -1.2% |
|  | 0.0% | -1.1% | -1.0% | 0.0% | -1.3% | -1.1% |
| Class F | 0.1% | 0.1% | 0.5% | 0.1% | 0.3% | 0.5% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

It was observed the coding gain of chroma components can be improved, especially for the Nebuta and Stemelocal in Class A.

# References

- F. Bossen, “Common HM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-H1100, February 2012.