| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0477 |
| --- | --- |

| Title: | AHG5 : Cross check of Qualcomm’s proposal JCTVC-I0325 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jungsun Kim, Byeongmoon Jeon #19 Yangjae-daero 11gil, Seocho-gu Seoul 137-130 Korea | Tel: Email: | +82-10-2201-9774 jungsun.kim@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This document reports the verification results of the proposal JCTVC-I0325 on AhG5: Change of threshold for NSQT significance map [1] submitted by Qualcomm. The proponents provide the patch of their source codes with the results. The crosschecker checked source codes and provided cross check results to the proponents. It succeeded and produced as same result as those the proponent provided. The spreadsheet contains the detailed data is accompanied.

# Problem statement and Methods Proposed

Currently coefficients in higher frequency region of transform unit use one common context. The region is defined by a threshold,

thredHighFreq = 3*(std::max(width, height)>>4).

Therefore the high frequency regions for non square transform units are as below:

Fig. 1 High frequency region shares common context in non square transform units

The proponent propose to change the threshold to

thredHighFreq = (3*(width + height))>>5

Therefore high frequency regions for non square transform unit is as below and the regions for square transform unit are not changed.

Fig. 2 Proposed high frequency region shares one common context in non square transform units

# Results

## Simulation environment

The encoding environment for cross-checking the method as following:

OS: Window XP 64 bit

CPU: i7 875K 2.93Hz

Memory : : DDR3 16G

The decoding environment for cross-checking is as following:

OS: Win XP 64 bit

CPU: i7 875K 2.93Hz

Memory: DDR3 16G

## Simulation conditions and results

The simulation follows the test conditions of [2]. There was no mismatch between reconstructed sequences at the encoder side and decoded sequences at the decoder. The results are same as the proponent provided. The spreadsheet contains the detailed data of the simulation is accompanied.

|  | Random Access HE10 |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.01% | -0.01% | -0.15% |  |  |  |
| Class B | 0.01% | -0.04% | -0.07% | -0.02% | 0.14% | 0.06% |
| Class C | 0.01% | -0.05% | -0.02% | 0.04% | 0.08% | 0.21% |
| Class D | 0.05% | 0.00% | -0.08% | -0.01% | 0.02% | -0.64% |
| Class E |  |  |  | -0.01% | -0.25% | -0.15% |
| Overall | 0.02% | -0.03% | -0.08% | 0.00% | 0.02% | -0.12% |
|  | 0.02% | -0.01% | -0.08% | 0.00% | 0.01% | -0.20% |
| Class F | -0.01% | -0.06% | 0.10% | -0.03% | -0.61% | -0.21% |
| Enc Time[%] | 101% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# References

- J. Sole, W.-J. Chien, J. Chen, M. Karczewicz, “AhG5: Change of threshold for NSQT significance map”, JCTVC-H00325, 9th JCT-VC Meeting, Geneva, CH, Apr. 2012

- F. Bossen, “Common HM test conditions”, JCTVC-H1100, 8th JCT-VC Meeting, San Jose, USA, Mar. 2012.