| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0482 |
| --- | --- |

| Title: | Crosscheck of JCTVC_I0257 - On Intensity Dependent Quantization in the HEVC codec |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Rik Allen, Vivek Gowri-Shankar Holmers Farm Way High Wycombe Bucks, UK | Tel: Email: | +44(0)1494602334 rallen@altera.com, vshankar@altera.com |
| Source: | Altera |  |  |

_____________________________

# Abstract

JCTVC-I0257 proposes a tool “Intensity Dependant Quantization” (IDQ) in which the quantization factor is modified according to a measure of local brightness to reduce bitrate in areas that the eye is less susceptible to. The supplied code, HM modifications and proposal were found to match, and for a subset of streams run to date, the objective figures match those supplied by the proponents.

In informal subjective viewing, many streams did appear not to have significant quality degradation as claimed in the proposal, but on some there were significant artefacts introduced by the IDQ.

# Introduction.

JCTVC-I0257 provides details of a tool “IDQ” which is claimed to provide significant bitrate reductions while producing streams with no subjective quality drop. The proponents do acknowledge an expected increase in BD Rate. The paper describes the tool, along with modifications to the standard, source code, and spreadsheet of results. Two experiments are described.

# Experiment 1

JCTVC-I0257 describes an experiment in which the proposed IDQ tool is compared with the normal HM_6.0 anchor. BD Rate figures are not provided since the proposal is expected to give subjective gains, but objective losses. Version 1 uploaded provides a list of streams that were subjectively viewed with the differences in PSNR and bitrate for each listed. The selection method for these streams was not disclosed.

Subsequent versions of the proposal indicated that all streams with a change in PSNR > 0.5dB were subjectively viewed. This is not the same set of streams as indicated in version 1 of the proposal – where all streams listed had a PSNR delta < 0.5dB.

A spreadsheet was provided giving the delta in PSNR and bitrate for all streams. The work to duplicate this is ongoing – the most recent indication is that the figures provided by the proponents are correct.

A selection of the encoded streams have been informally viewed to confirm the assertion that there is no degradation in quality. While most streams do appear to have no such degradation, some problems were noted e.g.:-

- lower left area of BQSquare, AI_Main, qp22, where the relatively bright area looses significant amounts of detail. At full speed this is manifested as a significant flickering effect in this. Also visible to some extent on the side of the tower later in the clip. This stream has a 7% bitrate reduction for 1.93dB increase in SNR, which is in the top 5% of streams in terms of SNR increase.

- Around the left hand table in BQSquare, AI_Main, qp37, there seems to be a colour shift between the anchor and the test.

- Blowing Bubbles, AI_Main, qp22, Some loss of detail in the seam on the sleeve in the lower left.

Figure 1 BQSquare AI_Main, QP 22, Frame 1, Anchor.

Figure 2 BQSquare AI_Main, QP 22, Frame 1, JCTVC_I0257.

The visible differences are most noticeable in bright areas. Further tuning of the IDQ parameters might improve this.

# Experiment 2

The second experiment described compares a non-normative technique for IDQ against the proposed method. It is understood that this is being independently cross checked by another party.

# Working draft.

The original WD text included a left shift by ( bitDepthMinus8 – 1 ), which would be negative in the case of 8 bit input. This was amended by the proponents.

The revised WD text appears to match the proposal.

# Code review

- The estimate of µ given in equation (1) is computed in TComTrQuant::computeIQPOffset and the code matches with equations (2), (3) and (4) although the proposal should be amended to properly define the case bitdepth==8 where no scaling is applied as intended. In the case of non-square TU, log2(MxN) seems to be computed with the assumption that M=4N or N=4M and it is not clear how future proof this code will be.

- The proposal states that the luminance component is used to drive the proposed IDQ. The code seems to be in agreement with this although it is a bit confusing because it was clearly written to also use chroma predictors initially. The WD should clearly identify predSample as being the luma samples and computeIdqAverage should be called only once (bitDepthMinus8 has been replaced with bitDepthY and can be removed).

- A distinct compilation flag was used to turn on the use of the IDQ at the TU level in the deblocking filter but the impact on reconstruction quality was not mentioned.

- The encoder and decoder were modified to write and parse the IDQ profile in the PPS as described in the proposal and updated WD.

# Observations

The proposed technique appears to require that inter or intra prediction be completed for a TU before inverse quantization.  In the case of inter blocks, this could potentially be pipelined in a hardware decoder, but for an intra block this would increase the latency around the reconstruction loop and potentially reduce throughput.

TU level calculation of the deblocking strength adds complexity to the decoder, perhaps substantially if the deblocking process is significantly decoupled from the reconstruction process.

# Patent rights declaration(s)

Altera does not have any current or pending patent rights relating to the technology described in this contribution.