| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0487 |
| --- | --- |

| Title: | On coefficient level remaining coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei-Jung Chien, Marta Karczewicz, Joel Sole, Jianle Chen 5775 Morehouse Dr. San Diego, CA 92130 USA | Tel: Email: | +1-858-845-1795 wchien@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

In the current coefficient level remaining coding of HEVC, an adaptive truncated Rice code (with parameter 0, 1, 2, 3, and 4) and a 0-th order Exponential Golomb Code is used to represent the remaining  of the absolute value of a quantized transform coefficient level. This contribution modified the binarization scheme by a unary code and a variable length code. The length of the variable length code depends on the unary code and a parameter (ranged also from 0 to 4). The simulation results showed no coding performance impact caused by the modifications.

# Introduction

In HM6.0 coefficient level coding, syntax element coeff_abs_level_remaining is used to represent the remaining of the absolute value of a quantized transform coefficient level.  The decoding process of  coeff_abs_level_remaining consists of an adaptive truncated Golomb-Rice code (Prefix0 and Prefix1) and a 0-th order Exp-Golomb code (EG0)(Suffix), as shown in Figure 1.  Suffix is only existed when coeff_abs_level_remaining is larger than a value,  cTRMax. The value, cTRMax, is what truncated Golomb-Rice code with a Rice-Golomb parameter, cRiceParam, could possibly represent.

Decode Prefix0

Decode Prefix1

ruiGoRiceParam > 0

Yes

Is cTRMax reached

Yes

Decode Suffix (EG0)

End

No

No

Figure 1. coeff_abs_level_remaining decoding process of HM6.0

# Proposed method

In this contribution, a simpler way for decoding coeff_abs_level_remaining is proposed compared to HM6.0. In the proposed method, coeff_abs_level_remaining is represented as a truncated unary code (Prefix) and a variable length code (Suffix), as shown in Table 2.1.  In table 2.1, a parameter, cParam, controls the length of the codeword structure. The update mechanism of cParam is the same as the update mechanism for cRiceParam in current HM6.0. The proposed method was also used in the coefficient level coding for HM3.0 LCEC.

Table 2.1 binarization of coeff_abs_level_remaining for cParam = k

|  | Prefix | Suffix (in bits) |
| --- | --- | --- |
| 0 ~ 1×2k-1 | 0 | k |
| 1×2k ~ 2×2k -1 | 10 | k |
| 2×2k ~ 3×2k -1 | 110 | k |
| 3×2k ~ 4×2k -1 | 1110 | k |
| 4×2k ~ 5×2k -1 | 11110 | k |
| 5×2k ~ 6×2k -1 | 111110 | k |
| 6×2k ~ 7×2k -1 | 1111110 | k |
| 7×2k ~ 8×2k -1 | 11111110 | k |
| 8×2k ~ 9×2k -1 | 111111110 | k |
| 9×2k ~ 11×2k -1 | 1111111110 | k +1 |
| 11×2k ~ 15×2k -1 | 11111111110 | k +2 |
| 15×2k ~ 23×2k -1 | 111111111110 | k +3 |
| … | … |  |

Table 2.2. shows the binarization of coeff_abs_level_remaining in HM6.0 (cRiceParam = 1) and the proposed method (cParam = 1). In HM6.0 and the proposed method, the existence of the suffix depends on the prefix.  However, the length of the suffix in HM 6.0 depends on the partially decoded results of the suffix, since it is an Exp-golomb code. The length of the suffix only depends on the prefix and this characteristic results in simpler decoder design.

Table 2.2 binarization of coeff_abs_level_remaining for cRiceParam = 1 and cParam = 1

| Value | HM6.0 |  |  | Proposed method |  |
| --- | --- | --- | --- | --- | --- |
|  | Prefix0 | Prefix1 | Suffix | Prefix | Suffix |
|  | Truncated Rice |  | EG0 |  |  |
| 0 | 0 | 0 |  | 0 | 0 |
| 1 | 0 | 1 |  | 0 | 1 |
| 2 | 10 | 0 |  | 10 | 0 |
| 3 | 10 | 1 |  | 10 | 1 |
| 4 | 110 | 0 |  | 110 | 0 |
| 5 | 110 | 1 |  | 110 | 1 |
| 6 | 1110 | 0 |  | 1110 | 0 |
| 7 | 1110 | 1 |  | 1110 | 1 |
| 8 | 11110 | 0 |  | 11110 | 0 |
| 9 | 11110 | 1 |  | 11110 | 1 |
| 10 | 111110 | 0 |  | 111110 | 0 |
| 11 | 111110 | 1 |  | 111110 | 1 |
| 12 | 1111110 | 0 |  | 1111110 | 0 |
| 13 | 1111110 | 1 |  | 1111110 | 1 |
| 14 | 1111111 | 0 |  | 11111110 | 0 |
| 15 | 1111111 | 1 | 0 | 11111110 | 1 |
| 16 | 1111111 | 1 | 100 | 111111110 | 0 |
| 17 | 1111111 | 1 | 101 | 111111110 | 1 |
| 18 | 1111111 | 1 | 11000 | 1111111110 | 00 |
| 19 | 1111111 | 1 | 11001 | 1111111110 | 01 |
| 20 | 1111111 | 1 | 11010 | 1111111110 | 10 |
| 21 | 1111111 | 1 | 11011 | 1111111110 | 11 |
| 22 | 1111111 | 1 | 1110000 | 11111111110 | 000 |
| 23 | 1111111 | 1 | 1110001 | 11111111110 | 001 |
| … | … | … | … | … | … |

# Simulation results

The proposed method is implemented on HM 6.0 and simulated under the common test conditions [1]. The performance is compared to HM6.0 anchor in terms of Bjøntegaard Delta (BD) Bit Rate. Table 3.1 and 3.2 summarize the experimental results for QP={22, 27, 32, 37} and QP={12, 17, 22, 27}, respectively.

Table 3.1 Simulation results for the proposed method at QP={22, 27, 32, 37}.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | 0.0% | 0.0% | 0.2% | 0.4% |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% | 0.0% |
| Class C | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | -0.1% |
| Class D | 0.0% | -0.2% | -0.1% | 0.0% | 0.1% | 0.2% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.3% | 0.2% | 0.0% | -0.2% | 0.1% |
| Class C | 0.0% | 0.3% | 0.0% | 0.0% | 0.2% | -0.1% |
| Class D | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | -0.4% |
| Class E | 0.0% | -0.6% | -0.1% | 0.1% | -0.3% | 0.0% |
| Overall | 0.0% | -0.1% | 0.1% | 0.0% | -0.1% | -0.1% |
|  | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% |
| Class F | -0.1% | -0.2% | 0.1% | 0.0% | -0.6% | 2.2% |
| Enc Time[%] | 98% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 3.2 Simulation results for the proposed method at QP={12, 17, 22, 27}.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class F | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 100% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | 0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | -0.1% | 0.1% |
| Class C | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class E | 0.0% | -0.5% | 0.1% | 0.0% | 0.2% | 0.9% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.2% |
| Class F | 0.0% | 0.0% | 0.1% | 0.0% | 0.6% | 1.1% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

The proposed method was also tested with [2] which modifies the Golomb-Rice parameter update scheme as suggested during the meeting. The table values g_auiUpTbl[5] ={3,6,12,24,0xfffff} are selected as in [2] (instead of {3,5,12,23,0xfffffff}) because can be represented as a simple formula. No more tables are needed (i.e., g_aauiGoRiceUpdate[5][24] is removed).

Table 3.3 Simulation results for the proposed method at QP={22, 27, 32, 37}.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class B | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class E | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Overall | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |
|  | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class F | -0.1% | -0.1% | -0.1% | -0.1% | -0.3% | -0.1% |
| Enc Time[%] | 102% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.1% | 0.0% | 0.0% | 0.2% | -0.2% |
| Class B | 0.0% | -0.1% | 0.1% | 0.0% | -0.1% | 0.0% |
| Class C | 0.0% | -0.1% | -0.2% | 0.0% | -0.2% | 0.0% |
| Class D | -0.1% | 0.0% | -0.2% | 0.1% | 0.0% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
|  | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
| Class F | -0.1% | -0.2% | -0.2% | -0.1% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.4% | 0.1% | 0.0% | 0.0% | -0.1% |
| Class C | 0.0% | 0.4% | -0.2% | 0.0% | 0.2% | 0.2% |
| Class D | 0.1% | 0.4% | -0.1% | 0.0% | 0.1% | -0.1% |
| Class E | 0.1% | -0.4% | 0.4% | 0.0% | 0.1% | 0.0% |
| Overall | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
|  | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class F | -0.1% | -0.6% | -0.6% | 0.0% | -1.4% | -0.1% |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

Table 3.4 Simulation results for the proposed method at QP={12, 17, 22, 27}.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class B | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class C | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class D | -0.2% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class E | -0.2% | -0.2% | -0.2% | -0.2% | -0.2% | -0.1% |
| Overall | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
|  | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class F | -0.2% | -0.1% | -0.2% | -0.1% | -0.2% | -0.1% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class F | -0.2% | -0.2% | -0.1% | -0.1% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 97% |  |  | 99% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class C | 0.0% | 0.0% | -0.2% | 0.0% | 0.2% | 0.1% |
| Class D | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 0.2% |
| Class E | 0.0% | 0.1% | -0.6% | 0.0% | -0.4% | 0.2% |
| Overall | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | 0.1% |
|  | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | 0.1% |
| Class F | -0.1% | -0.2% | -0.1% | -0.1% | 0.6% | -0.6% |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 98% |  |  | 97% |  |  |

- Conclusions

In this contribution, we propose to change to the binarization of coeff_abs_level_remaining by a unary code and a variable length code. The length of the variable length code depends on the unary code. The proposed scheme has no noticeable coding performance impact based on the simulation results. We recommend to adopt the proposed method to HEVC.

# References

[1] F. Bossen, “Common test conditions”, JCTVC-G1100, San Jose, Feb, 2012.

[2] C. Kim, J. Kim, J. H. Park, “Simplification of Golomb-Rice Parameter Update”, JCTVC-I0124, Geneva, April, 2012

# Patent rights declaration(s)

Qualcomm Inc. does not have any current or pending patent rights relating to the technology described in this contribution.