| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0490 |
| --- | --- |

| Title: | Combination of JCTVC-I0084 and JCTVC-I0298 on simplified MV scaling process for spatial AMVP derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongbing Lin Lingzhi Liu | Tel: Email: | +86 10 60612142 yblin@huawei.com Lingzhi.liu@huawei.com |
| Source: | HiSilicon Technologies |  |  |

_____________________________

# Abstract

This document reports test results of combination of JCTVC-I0084 and JCTVC-I0298. The combined method simplifies spatial AMVP derivation by removing the neighboring positions for the scaled MV candidate derivation. Furthermore, the proposed method also removes the dependency between the derivation of scaled MV candidate B and the derivation of the MV candidate A. The test results show coding efficiency of (0.0%, 0.0%, 0.0%) for (Y, U, V) in average for both random access and low-delay configurations under common test conditions.

# Introduction

The process of spatial AMVP candidate derivation in HM6 can be described as follows:

- Non-scaled MV candidate A derivation based on positions of A0, A1; If the non-scaled MV candidate  is not available, scaled MV candidate A derivation based on positions of A0, A1

- Non-scaled MV candidate B derivation based on positions of B0, B1, B2

- If the candidate A is not available, scaled MV candidate B derivation based on positions of B0, B1, B2

These neighbouring positions, i.e., A0, A1, B0, B1, B2, are show in Figure 1. It is noted that the scaled MV candidate B derivation in step 3 is coupled with the availability of the derived MV candidate A in step 1 [1].

Fig. 1. Neighboring positions for spatial AMVP derivation in HM6

In this meeting, both JCTVC-I0084 and JCTVC-I0298 discuss simplification of the scaled MV candidate derivation. JCTVC-I0298 proposes to remove the scaled MV candidate derivation in step 1 [2], while JCTVC-I0084 proposes to use one position for the scaled MV candidate A in step 1 and another one position for the scaled MV candidate B in step 3 [3].

In this document, combination of JCTVC-I0084 and JCTVC-I0298 are proposed to further simplify the spatial AMVP derivation by:

- removing the scaled MV candidate A derivation in step 1

- using only one position B1 for the scaled MV candidate B derivation in step 3

- Furthermore, the conditional check in step 3 is changed to check the availability of the MV candidate B, instead of the MV candidate A before performing the scaled MV candidate derivation. It also decouples the derivation of scaled MV candidate B from the derivation of MV candidate A.

As a result, the proposed spatial AMVP derivation becomes

- Non-scaled MV candidate A derivation based on positions of A0, A1

- Non-scaled MV candidate B derivation based on positions of B0, B1, B2

- if the non-scaled MV candidate B is not available, scaled MV candidate B derivation based on positions of B1

# Experimental results

The simulations of this document use HM6.0 software. The common test conditions and reference configurations specified in [4] are followed. The performance of the proposed method, including a), b) and c), is summarized in Table 1. In addition, we also test the combined simplifications of a) and b), as shown in Table 2. It is observed that both Table 1 and Table 2 show coding efficiency of (0.0%, 0.0%, 0.0%) for (Y, U, V) in average for both random access and low-delay configurations under common test conditions.

Table 1. Experimental results of the proposed combination of a), b) and c)

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.2% | 0.2% | 0.3% | 0.1% | 0.3% | 0.3% |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class C | 0.1% | 0.2% | 0.1% | 0.1% | 0.1% | 0.0% |
| Class D | 0.0% | 0.1% | 0.1% | 0.1% | 0.0% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% | 0.1% |
|  | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% | 0.1% |
| Class F | 0.1% | 0.1% | 0.1% | 0.1% | 0.2% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.1% | 0.0% | -0.1% | 0.0% | -0.2% |
| Class C | -0.1% | 0.1% | -0.1% | -0.1% | 0.1% | 0.1% |
| Class D | 0.0% | -0.3% | 0.2% | -0.1% | -0.4% | -0.3% |
| Class E | -0.1% | 0.1% | -0.3% | -0.1% | -0.6% | 0.2% |
| Overall | 0.0% | -0.1% | 0.0% | -0.1% | -0.2% | -0.1% |
|  | 0.0% | -0.1% | 0.0% | -0.1% | -0.2% | -0.2% |
| Class F | 0.1% | -0.2% | 0.3% | 0.1% | -0.2% | 0.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 99% |  |  |

Table 2. Experimental results of the proposed combination of a) and b)

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.2% | 0.3% | 0.3% | 0.1% | 0.3% | 0.2% |
| Class B | 0.0% | 0.1% | 0.2% | 0.0% | 0.0% | 0.1% |
| Class C | 0.1% | 0.2% | 0.2% | 0.0% | 0.0% | -0.1% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% |
|  | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% |
| Class F | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -0.4% | -0.2% | -0.1% | 0.0% | -0.2% |
| Class C | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | -0.3% | 0.9% | -0.1% | -0.4% | -0.5% |
| Class E | 0.0% | -0.3% | 0.4% | -0.1% | 0.0% | -0.1% |
| Overall | 0.0% | -0.2% | 0.2% | -0.1% | -0.1% | -0.2% |
|  | 0.0% | -0.2% | 0.2% | -0.1% | -0.1% | -0.2% |
| Class F | -0.1% | -0.1% | 0.0% | 0.2% | -0.2% | 1.3% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

# Conclusion and recommendation

The proposed methods simplify the process of spatial AMVP derivation by removing the neighboring positions for the scaled MV candidate derivation. Furthermore, the proposed method, including a), b) and c), also decouples the derivation of scaled MV candidate B from the derivation of the MV candidate A. Experimental results show the proposed method has no coding efficiency loss in average for both random access and low-delay configurations under common test conditions. It is recommended to adopt the proposal.

# References

- B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm, T. Wiegand, “High Efficiency Video Coding (HEVC) text specification draft 6,” JCT-VC Document, JCTVC-H1003, San Jose, US, Feb. 2012

- S. Fukushima, I.-K Kim, “Simplification of spatial motion vector scaling process,” JCT-VC document, JCTVC-I0298, Geneva, CH, April, 2012

- Y. Lin, L. Liu, “Reduced number of checking scaled MV candidates in AMVP list construction,” JCT-VC document, JCTVC-I0084, Geneva, CH, April, 2012

- F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-H1100, San Jose, US, Feb. 2012

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Proposed text changes to WD6

#### 8.5.2.1.6 Derivation process for motion vector predictor candidates

…

The motion vector mvLXA and the availability flag availableFlagLXA are derived in the following ordered steps:

- Let a set of two sample locations be (xAk, yAk), with k = 0, 1, specifies sample locations with xAk = xP − 1, yA0 = yP + nPSH and yA1 = yA0 - MinPuSize. The set of sample locations ( xAk, yAk ) represent the sample locations immediately to the left side of the left partition boundary and it’s extended line.

- Let the availability flag availableFlagLXA be initially set equal to 0 and the both components of mvLXA are set equal to 0.

- For ( xAk, yAk ) from ( xA0, yA0 ) to ( xA1, yA1 ) where yA1 = yA0 − MinPuSize, the following applies repeatedly until availableFlagLXA is equal to 1:

- If the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xAk ][ yAk ] is equal to 1 and the reference index refIdxLX[ xAk ][ yAk ] is equal to the reference index of the current prediction unit refIdxLX, availableFlagLXA is set equal to 1 and the motion vector mvLXA is set equal to the motion vector mvLX[ xAk ][ yAk ], refIdxA is set equal to refIdxLX[ xAk ][ yAk ] and ListA is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xAk ][ yAk ] (with Y = !X) is equal to 1 and PicOrderCnt( RefPicListLY( refIdxLY[ xAk ][ yAk ] ) ) is equal to PicOrderCnt( RefPicListLX( refIdxLX ) ), availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLY[ xAk ][ yAk ], refIdxA is set equal to refIdxLY[ xAk ][ yAk ] ,  ListA is set equal to LY and mvLXA is set equal to mvLXA.

-

-

-

-

-

The motion vector mvLXB and the availability flag availableFlagLXB are derived in the following ordered steps:

- Let a set of three sample location (xBk, yBk), with k = 0,1,2, specifies sample locations with xB0 = xP + nPSW, xB1 = xB0− MinPuSize , xB2 = xP − MinPuSize and yBk = yP − 1. The set of sample locations ( xBk, yBk ) represent the sample locations immediately to the upper side of the above partition boundary and its extended line.

- If yP−1 is less than yCtb, the following applies.

xB0 = (xB0>>Log2MinCbSize)<<Log2MinCbSize) + ((xB0>>Log2MinCbSize)&1)*(MinPuSize*2−1)	(8-6)
xB1 = (xB1>>Log2MinCbSize)<<Log2MinCbSize) + ((xB1>>Log2MinCbSize)&1)*(MinPuSize*2−1)	(8-7)
xB2 = (xB2>>Log2MinCbSize)<<Log2MinCbSize) + ((xB2>>Log2MinCbSize)&1)*(MinPuSize*2−1)	(8-8)

where MinPuSize = 1 << ( Log2MinCbSize − 1)

- Let the availability flag availableFlagLXB be initially set equal to 0 and the both components of mvLXB are set equal to 0.

- For ( xBk, yBk ) from ( xB0, yB0 ) to ( xB2, yB2 ) where xB0 = xP + nPSW, xB1 = xB0 − MinPuSize , and xB2 =  xP − MinPuSize, the following applies repeatedly until availableFlagLXB is equal to 1:

- If the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xBk ][ yBk ] is equal to 1, and the reference index refIdxLX[ xBk ][ yBk ] is equal to the reference index of the current prediction unit refIdxLX, availableFlagLXB is set equal to 1 and the motion vector mvLXB is set equal to the motion vector mvLX[ xBk ][ yBk ], refIdxB is set equal to refIdxLX[ xBk ][ yBk ] and ListB is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xBk ][ yBk ] (with Y = !X) is equal to 1 and PicOrderCnt( RefPicListLY( refIdxLY[ xBk ][ yBk ] ) ) is equal to PicOrderCnt( RefPicListLX( refIdxLX ) ), availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLY[ xBk ][ yBk ],  refIdxB is set equal to refIdxLY[ xBk ][ yBk ],  and ListB is set equal to LY.

- When availableFlagLXB is equal to 0, ( xBk, yBk ) is set equal to ( xB1, yB1 ) where xB1 = xP + nPSW - MinPuSize, the following applies:

- If the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xBk ][ yBk ] is equal to 1, availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLX[ xBk ][ yBk ], refIdxB is set equal to refIdxLX[ xBk ][ yBk ], ListB is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xBk ][ yBk ] (with Y = !X) is equal to 1, availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLY[ xBk ][ yBk ], refIdxB is set equal to refIdxLY[ xBk ][ yBk ], ListB is set equal to LY.

- When availableFlagLXB is equal to 1 and PicOrderCnt( RefPicListListB( refIdxB ) ) is not equal to PicOrderCnt( RefPicListLX( refIdxLX ) ), mvLXB is derived as specified below.

…