| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0491 |
| --- | --- |

| Title: | Crosscheck of CE1 Test4.2: Region-based syntax in APS mode (JCTVC-I0161) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Cross-check |  |  |
| Author(s) or Contact(s): | Madhukar Budagavi Texas Instruments Inc., USA | Tel: Email: | +1-214-4870-1861 madhukar@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This contribution presents results of cross-verification of CE1 Test 4.2 proposal of JCTVC-I0161 which uses syntax elements log2_sao_unit_width_minus6 and log2_sao_unit_height_minus6 to signal the size of SAO region for APS mode. By increasing the size of SAO region, this proposal reduces the memory required to store SAO offsets in APS mode. When compared to CE1 anchors, it is reported that BD-rate results of Y:0.0%, U: 0.2%, V:0.2% were observed for RA-Main and RA-HE10. Simulation results are reported to match those provided by proponents.

# Simulation results

Table below presents a summary of BD-Rate in comparison with CE1 anchors.

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 1.0% | 1.0% | 0.0% | 0.7% | 0.7% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.2% | 0.2% | 0.0% | 0.2% | 0.2% |
|  | 0.0% | 0.2% | 0.2% | 0.0% | 0.2% | 0.2% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  | 96% |  |  |
| Dec Time[%] | 99% |  |  | 96% |  |  |

# Comments

SAO offsets transmitted in APS need to be decoded and stored in memory. HM-6.0 allows for SAO offsets to be signalled every LCU in APS. When LCU size is 16x16, the memory requirements are large. So specifying a larger region for signalling offsets is a desirable feature to reduce memory storage requirements.

# Patent rights declaration(s)

Texas Instruments Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).