| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0493 |
| --- | --- |

| Title: | Crosscheck of Option 2 (RA) and independent ALF chroma control of AHG6: Baseline options for ALF (JCTVC-I0157) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Cross-check |  |  |
| Author(s) or Contact(s): | Madhukar Budagavi Texas Instruments Inc., USA | Tel: Email: | +1-214-4870-1861 madhukar@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Cross-check results

This document presents results of cross-verification of (1) AHG6 Option 2 Region based ALF (16 regions) where ALF filter is designed using previous frame statistics (RA_prev) and current frame statistics (RA_curr) and (2) when chroma ALF is independently controlled of luma (chroma_indep). Simulation results matched those provided by proponents.

Tables below presents a summary of BD-Rate in comparison with HM 6.0 LCU-based ALF anchors.

RA_curr:

|  | AI-HE10 | RA-HE10 | LB-HE10 | LP-HE10 |
| --- | --- | --- | --- | --- |
| Y | -0.3% | -0.7% | -0.4% | -0.6% |
| U | -2.4% | -0.7% | -2.6% | -4.3% |
| V | -2.8% | -0.2% | -2.7% | -4.2% |

RA_prev:

|  | AI-HE10 | RA-HE10 | LB-HE10 | LP-HE10 |
| --- | --- | --- | --- | --- |
| Y | -0.2% | -0.3% | -0.1% | -0.1% |
| U | -2.6% | -2.0% | -2.7% | -4.1% |
| V | -2.9% | -1.5% | -2.8% | -4.4% |

Independent control of ALF chroma:

BD-Rates matched.

# Comments

Option 2 is desirable simplification. The CD text size significantly reduces. Only one ALF filter set is signaled in APS. ALF is switched on and off at LCU level.

Independent control of chroma is also desirable instead of making chroma ALF switching dependent on luma ALF being on.