| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0495 |
| --- | --- |

| Title: | Cross-check of Intensity Dependent Quantization (JCTVC-I0257) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Glenn Van Wallendael Sebastiaan Van Leuven, Jan De Cock, Rik Van de Walle Gaston Crommenlaan 8 P.O. Box 201 9050 Ghent-Ledeberg, Belgium | Tel: Email: | +32 93314957 glenn.vanwallendael@ugent.be |
| Source: | Ghent University - IBBT |  |  |

_____________________________

# Abstract

This contribution provides a cross verification of the Intensity Dependent Quantization (JCTVC-I0257). The code reflects the description and test results are conforming to the tests conducted by the proponents. Additionally, a subjective evaluation was conducted. The difference between HM and the proposed technique could only be detected by experts who had knowledge about the way IDQ works. Video experts without this knowledge could not observe any difference.

# Code

The code is conforming the proponents description.

# Test conditions

## Encoding

Cluster computer with 167 computing nodes (IBM iDataPlex dx360M2), each of which contains:

- CPU: dual-socket quad-core Intel Xeon L5520 (Intel Nehalem microarchitecture, 2.27 GHz, 8 MB L3 cache per quad-core chip), thus 8 cores / node

- memory: 12 GB RAM (DDR3 PC3-8500S 1067 MHz)

- storage (local): one 156 GB 3.5" 15k RPM SAS disk

and 194 computing nodes (IBM HS 21 XM blade), each of which contains:

- CPU: dual-socket quad-core Intel Xeon L5420 (Intel Core microarchitecture, 2.5 GHz, 6 MB L2 cache per quad-core chip), thus 8 cores / node

- memory: 16 GB RAM (DDR2 FB-DIMM PC-5300 CL5)

- storage (local): one 146 GB 2.5" 10k RPM SAS disk

The encoder executables were generated with g++ 4.1.2.

## Decoding

- CPU: quad-core Intel Core 2 Q6600 (Intel Core microarchitecture, 2.40GHz, 8MB L2 cache)

- Memory: 4 GB RAM (DDR2 PC2-6400 400MHz)

## Display

Dell Ultrasharp U2212HM

- Resolution: 1920x1080

- Size: 22”

Barco LC-47

- Resolution: 1920x1080

- Size: 47”

# Subjective evaluation

An informal subjective evaluation of the IDQ is done by experts. There were 2 groups of experts (6 persons), one group with knowledge about the encoding technique (3 persons evaluating on the 2 different displays), the other group without knowledge of IDQ (3 persons only evaluating on the Barco display).

|  | No difference | HM better | IDQ better |
| --- | --- | --- | --- |
| Experts with IDQ information | 113 | 22 | 3 |
| Experts without IDQ information | 64 | 2 | 3 |
| Total | 177 | 24 | 6 |

It can be concluded that for experts with knowledge about the IDQ technique a small difference can be observed in some sequences. For example, in BQSquare, a slight quality degradation of the tiles on the bright floor could be noticed. In other sequences, when it was noticeable, it could only be perceived on small parts of the image where people know there would be dark or light regions.

For people who did not know that they had to look for IDQ related artefacts, it was impossible to distinguish the sequences. All these tests were conducted with video experts.