INTERNATIONAL ORGANIZATION FOR STANDARDIZATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC1/SC29/WG11

CODING OF MOVING PICTURES AND AUDIO

ISO/IEC JTC1/SC29/WG11

MPEG2012/m25028

28 April 2012, Geneva, CH

Title: 	HEVC Profile Signalling

Status:	Contribution

Authors: 	David Singer

Organization: 	Apple Inc.

Group: 	JCT-VC

# Introduction

I have been wondering if we can improve the profile signalling in HEVC.  In days of yore, profiles were simple: they 'nested' fully, and simple numerical comparison (my implemented profile is greater than or equal to the stream's required profile) determined playability.

No more with AVC; the profiles do not nest very much, as the following diagram shows. It shows which profiles are fully subsets of others.

CBP

BP

Extended

Main

ProgHi

High

Hi10

Hi422

Hi444Pred

Hi10Intra

Hi422Intra

Hi444Intra

ScalCBP

ScalBP

ScalConsHi

ScalHi

ScalHiIntra

StereoHi

MultiVHi

CAVLC444Intra

ConsProgHi

Figure 1: 'is fully a subset of' AVC profiles

We need a labelling scheme that:

- allows the committee to introduce new profiles, and older systems be able to work out which bitstreams they can decode when those new profiles are fully subsets of older existing profiles, even when they do not recognize the new profile identifier;

- allows bitstreams which don't use all the features of any given profile, and are as a consequence compatible with more than one, to be labelled with so that it's possible to deduce all the profiles that the bitstream is compatible with (the ones that don't automatically follow from the nesting structure).

Example of (a): say we build a device that can decode 'advanced X' profile, and later a 'restricted X' profile is introduced with a subset of the features in it. We want devices that know 'I can play advanced X' know that they can play streams labelled as 'restricted X' even though the profile_idc is, to them, unrecognized.

Example of (b): some profiles include tools that are rarely used, such as the error-resilience tools in Baseline. It should be possible to label a stream as 'I am a baseline stream' and also say 'but any Main, or High, or… decoder can also decode this stream, because I left out the features in Baseline that aren't in those profiles'.

Today we have a few bits of 'compatibility indicators' (the constraint_set flags) but that existing indicator + 4 bits system is not much machinery for this problem.

# Alternative signaling methods

## Methods that don’t work

- feature bit-map: simply decide on all the profilable features and have a bit-mask in the stream saying what features are used.  This fails on two counts (a) we'll never decide the complete list of features and (b) there are basically no profiles any more – we can name various values of the bit-mask, but anyone can mask how they like.

- number and list: do what the MP4 file format does with the ftyp: have a 'best viewed as' and a list 'also compatible with'.  We don't want variable-length list structures at this point in the SPS, and it takes too many bits.

- 8-bit number and bitmap: have a bitmap of 256 bits to say 'if the bit is 1, I am also compatible with the indicated profile'.  This is way too many bits.

## Methods that might work

- small number and bitmap: have a smaller value-space for profile_idc, have the actual value mean nothing, and have a bit mask for 'also compatible with'.  We only have 21 profiles in AVC, so profile_idc in the range 1-31 and a 32-bit mask is probably fine, and defensible.

- 8-bit number and family bit-map: the most compact, the most complex. Keep the 8-bit value for profile_idc, but instead of having a 256-bit bit-mask (too big), leverage the fact that the profiles do nest 'in families'.  This is worked out below.  Whether we could have arrived here, as we invented profiles and assigned values one by one, is open to doubt.  But it can be done.

So, working out (B).

I think a 'best viewed as' profile_idc makes sense; so then, what's a reasonable way to code the 'also compatible with' list?

Numeric comparison can be used when we know the profiles are in a nesting relationship (in a 'family'); that deals with the known-nesting of profiles.

We can group the profile indicator values into M (say 16) families of N (say 16) values. (0-15, 16-31, and so on). Within a family, support of label X means you automatically support all profiles with label Y<=X.  (8 families is not enough – AVC uses 9, as shown in the diagram above).

So, given a profile_IDC, you already know you support lower profiles in the same family. Then we introduce 16 bits to indicate what families you (also) support.  The bit indicator for the family of the signalled profile must be set; other family indicators can also be set.  For a stream labelled as profile_idc X, with a bit-position B set in the profile_family mask, a system can decode that stream if it supports a profile Y:

- Y is in the range [B*16, B*16+15] (i.e. a profile in that family)

- and Y >= (X rem 16) + B*16 (i.e. we support the same or a higher profile than required in that family)

This means that the profiles have to be numbered such that, in the above diagram, all the lines go up from a lesser modulus value to a greater modulus value.  The specification can indicate the bits that should be set because of these pre-defined nesting relationships ("bits 3, 6, and 7 MUST be set for profile_IDC X") – the lines in the diagram.

Here is the AVC profile structure fit into an 8-bit profile_IDC and 16 family-compatible bits.

| FAMILY |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
|  | 0-15 | 16-31 | 32-47 | 48-63 | 64-79 | 80-95 | 96-111 | 112-127 | 128-143 | 144-159 |
| 15 |  |  |  | ScalHi |  |  | Hi444Pred |  |  |  |
| 14 |  |  | ScalHIntra |  |  |  | Hi422 | Hi444Intra |  |  |
| 13 |  |  |  |  |  |  |  |  |  |  |
| 12 |  |  |  | ScalConsHi |  | StereoHi | Hi10 |  |  |  |
| 11 |  |  |  |  |  |  |  |  |  |  |
| 10 |  |  |  |  | MultiVHi |  |  | Hi422Intra |  |  |
| 9 |  |  |  |  |  |  |  |  |  |  |
| 8 |  | Extended |  | ScalBP |  |  | High | Hi10Intra |  | CAVLC444Intra |
| 7 |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  | ProgHi |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |
| 4 |  | BP |  | ScalCBP |  |  | ConsProgHi |  | Main |  |
| 3 |  |  |  |  |  |  |  |  |  |  |
| 2 |  | CBP |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |

Not shown in the diagram are the cases where a stream that is compatible with profile A can often be compatible with B (example (b) cases); imagine they are shown with dotted lines.  We'd want such potential 'example b' lines also to go up from a lower to a higher modulus value.

This makes picking profile_idc values more of an art, for the committee, to be sure.  But it makes the profile labeling and compatibility determination uniform and deterministic.

I think that this system with only 16 families has too much constraint on the committee ‘getting it right as we go’. A system with 8 profiles per family and 32 families could be used; we have fewer than 32 profiles in total right now, so even if every profile had been forced into a new family, we could have made it work. But this still makes life hard on number allocation; we could easily get into a state where we need a new profile number below another but in family – but we’ve reached 0 in that family already.

# Conclusion

So, overall, I prefer the simple 32-bit mask approach; it’s simple, unstructured, and would have worked for AVC as we built it. Profile identifiers are just assigned in order from 1; their value is just the index into the bitmap, so we don’t need any ordering, gaps, etc.

A 5 bit profile IDC leaves 3 bits, which we can reserve to 0 and call ‘profile space’; the meaning of profile IDC and the compatibility bits is initially only defined for profile space 0.

We can make this tidy, with the following bit allocation:

- 3 bits, profile_space (reserved 0 initially);

- 5 bits profile_idc, meaning contextual on profile space;

- 12 bits, reserved indicator flags (required to be 0 in conforming bitstreams and ignored by the decoder);

- 12 bits, level_idc;

- 32 bits, profile compatibility, meaning contextual on profile space

# Acknowledgments

Gary Sullivan was instrumental in providing the data for the AVC diagrams, and in developing the final bit allocation.

# Patent Statement

The author is unaware of any patent rights that apply to this contribution.

Apple Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).