| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0500 |
| --- | --- |

| Title: | Definition of slice types |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karsten Suehring Fraunhofer HHI | Email: | Karsten.Suehring@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

A text/software mismatch has been reported in bug tracker issue #389. This contribution proposes to reorder the different values of slice_type based on the expected probability.

# slice_type definitions

The draft text defines the semantics of slice_type aligned with the definition in H.264/AVC:

| - slice_type | - Name of slice_type |
| --- | --- |
| - 0 | - P (P slice) |
| - 1 | - B (B slice) |
| - 2 | - I (I slice) |

In the HM software, the following values are used:

| - slice_type | - Name of slice_type |
| --- | --- |
| - 0 | - I (I slice) |
| - 1 | - P (P slice) |
| - 2 | - B (B slice) |

# Proposed ordering

The slice_type syntax element is coded using ue(v) variable length codes. Thus the value 0 has a coding advantage compared to the other values. During the development of H.264/AVC we decided that P slices are the most likely used type. In the HEVC development (e.g. common coding conditions), B slices now have replaced P slices in most use cases. Thus the author proposes to move the B slice type to the top of the list, i.e. assign the value zero for B slices.

| - slice_type | - Name of slice_type |
| --- | --- |
| - 0 | - B (B slice) |
| - 1 | - P (P slice) |
| - 2 | - I (I slice) |

# Patent rights declaration(s)

Fraunhofer HHI does not have any current or pending patent rights relating to the technology described in this contribution.