| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0501 m25033 |
| --- | --- |

| Title: | Crosscheck of Dependent Slices (JCTVC-I0229) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Gordon Clare (Orange Labs) Félix Henry (Orange Labs) | Tel: Email: | +33 2 99 12 45 58 gordon.clare@orange-ftgroup.com felix.henry@orange-ftgroup.com |
| Source: | Orange Labs |  |  |

_____________________________

# Abstract

Contribution JCTVC-I0229 proposes that a new mode of slice is introduced, called dependent slice. A dependent slice preserves a number of dependencies with previously encoded slices, including entropy coding dependencies such as context information (as opposed to entropy slice). The source code of the proposal was examined and appears to match the description. The rate distortion performance was measured and appears to match those of the proponent except for one single point (configuration error is suspected).

# Introduction

JCTVC-I0229 proposes that a new mode of slice, called dependent slice, is introduced. Regular slice break all dependencies at their boundaries (prediction used during decoding process such as decoded pixels and motion vectors, and contexts used during parsing). A dependent slice is similar to a slice except that these dependencies are re-established. It is asserted by the proponents that the concept of dependent slice combined with wavefront parallel processing (WPP) reduces encoder delay and facilitates network transmission.

# Compression performance verification

The source code provided by the proponents of JCTVC-I0229 has been compiled and run under JCTVC-H1100 test conditions [1] and the following test configurations indicated with the corresponding table:

One WPP substream per row with dependent slices – (1 slice, rest is dependent slices), table 1

One WPP substream per row with dependent slices – (2,3,4 slices, cases depending on resolution, rest is dependent slices), table 2

The software used for the first experiment produced the performance recorded in attached excel sheets. The rate-distortion performance matches the one measured by the proponents for the tested configuration, except for one point (Class F Random Access nSlice_PlusDepSlices luma point) where a difference of 0.1% was observed. A configuration error is suspected.

]

# Source code examination

The source code has been studied and matches the description of the algorithm provided in JCTVC-I0229 for the wavefront version of dependent slices for the 1-slice-per-LCU-row case, which was cross checked. The tile-version of dependent slices was not implemented in the software.

The description of the proposed technology does not cover cases more complicated than one slice per row.

# Bibliography

[1] F. Bossen, “Common test conditions”, JCTVC-H1100, San Jose, February 2012.