| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0502 |
| --- | --- |

| Title: | On Chroma considerations for Interlaced material coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jerome VIERON Jean-Marc THIESSE Pierre LARBIER ATEME 26 Burospace, Route de Gisy 91570 BIEVRES, FRANCE | Tel: Email: | +33 16 935 8998 j.vieron@ateme.com, jm.thiesse@ateme.com p.larbier@ateme.com |
| Source: | ATEME |  |  |

_____________________________

# Abstract

This contribution proposes a solution related to the chroma phase issue when encoding 4:2:0 Interlaced material in the current HEVC design. Specifically, the proposed method deals both with 1) the misalignment of chroma samples locations of top field with regard to bottom field and 2) the misalignment of chroma samples locations with regard to the so-called “nominal” samples locations of 4:2:0 content in progressive mode.

It is proposed to align both top and bottom fields chroma samples before encoding, to signal the chroma phase parameters and realign the chroma samples in a proper way at the decoding side.

This proposal reports further experiments based on the HM6.0. The evaluation has been made on different Interlaced 4:2:0 sequences. Using the proposed method, it is reported that average gains of -6.3% and -6.5% on respectively U and V chroma components can be observed.

# Introduction

While the current development of HEVC is very promising and generates high interests among the broadcasting community, its focus on progressive format could compromise its adoption by broadcasters and service providers. Indeed, legacy interlaced video content represent still a very large part of the current video traffic.

So far, the support of interlace format in HEVC has been achieved through the proposal and the adoption of the field indication SEI message at San Jose meeting [1].

However, when considering 4:2:0 interlaced material top and bottom fields chroma samples are misaligned leading to inconsistency between the current derivation process of motion vector information and the nature of interlace format. Moreover, in its current design, LM mode only considers 4:2:0 content with so-called “nominal” luma and chroma samples position. Hence, in case of Interlaced field content, respective luma samples misalignment with chroma samples leads to inconsistency during the luma-based chroma prediction process.

In the current contribution, in order to deal with the chroma misalignment problems when processing interlaced 4:2:0 content, we propose to align both top and bottom fields chroma samples before encoding. The related chroma displacements have then to be signaled in the bitstream, at the decoding side, to realign the chroma samples in a proper way.

# Chroma phase considerations

The chroma phase corresponds to the location of the chroma samples related to the luma samples. For each position, a couple of horizontal and vertical chroma phases (Cx,Cy), in unit of luma samples of a frame can be specified.

## “Nominal” sampling locations of 4:2:0 material

Figures 1 and 2 depict the so-called ‘nominal’ luma and chroma sampling locations of respectively 4:2:0 progressive and interlace materials.

Figure 1: Nominal locations of 4:2:0 luma and chroma samples in progressive format.

Figure 2: Nominal locations of 4:2:0 luma and chroma samples in top and bottom fields.

In case of 4:2:0 progressive content, the nominal vertical and horizontal relative locations of luma and chroma samples correspond to (Cx=0, Cy=0.5).

In case of 4:2:0 interlaced content, the nominal vertical and horizontal relative locations of luma and chroma samples correspond to (Cx=0, Cy=0.25).

## Chroma phase misalignment

In addition to the different vertical sampling position of chroma components, there is a pixel displacement between top and bottom fields in 4:2:0 format. This misalignment leads to inconsistency during the chroma motion vector derivation and the luma-based chroma prediction mode (LM mode).

Indeed regarding the chroma vector misalignment; when a luma motion vector in a top field picture points to somewhere in a bottom field picture, the corresponding chroma motion vector points to 0.25 pixel below the correct position. Similarly, when a luma motion vector of a bottom field points to a top field picture, the corresponding chroma motion vector points to 0.25 pixel above the correct position.

Moreover, in the current design, LM mode only considers 4:2:0 content with “nominal” luma and chroma samples position. In case of interlaced content, it leads to a misalignment of interpolated luma samples with regard to the chroma samples.

# Proposal

## Description

In the current contribution, in order to deal with the chroma misalignment problems when processing interlaced 4:2:0 content, we propose to align both top and bottom fields chroma samples before encoding. The related chroma displacements have then to be signaled in the bitstream, in order, at the decoding side, to realign the chroma samples in a proper way (as in the original source content for instance).

In order to validate our proposal, various alignment policies have been studied and evaluated. Quite simple Lanczos 3 filtering (6 taps), has been used to shift the chroma components. As expected, the best configuration consist in a phase shifting of ¼ pixels down on the top field and of ¼ pixels up on the bottom field.

With such an approach, the encoding input chroma samples have the same relative phase with regard to luma samples, as for progressive material. Moreover, such phasing is fully in line with the so-called “nominal” chroma samples locations in progressive mode knowing (Cx=0, Cy=0.5).

Thus, we recommend to perform the following phase re-alignment when encoding interlaced content with HEVC:

- phase shifting of ¼ pixels down on the top field

- phase shifting of ¼ pixels up on the bottom field.

Then, based on the phase information signalled in the bitstream, the decoder side can properly de-shift the chroma samples in a suitable way.

## Signalling method

In the present contribution, we propose to signal the chroma samples shifting informations using the SEI Field indication message expanded with complementary syntax elements.

## Syntax changes

It is proposed to insert syntax elements related to chroma samples location in the Field indication SEI message as described in Table 1.

Table 1.  Proposed Field Indication SEI Message

| field_indication( payloadSize ) { | Descriptor |
| --- | --- |
| sequence_type_flag | u(1) |
| progressive_source_flag | u(1) |
| bottom_field_flag | u(1) |
| top_field_first_flag | u(1) |
| duplicate_flag | u(1) |
| chroma_sample_loc_delta_present_flag | u(1) |
| reserved_zero_2bits /* equal to 0 */ | u(2) |
| if (chroma_sample_loc_delta_present_flag == 1 ) { |  |
| chroma_sample_loc_delta_x | u(3) |
| chroma_sample_loc_delta_y | u(3) |
| } |  |
| } |  |

Three new syntax elements (in red) are proposed chroma_sample_loc_delta_flag, chroma_sample_loc_delta_x and chroma_sample_loc_delta_y. The semantics of these elements are as follows.

- chroma_sample_loc_delta_flag equal to 1 indicates that chroma_sample_loc_delta_x and chroma_sample_loc_delta_y are present in the field indication SEI message. chroma_loc_info_present_flag equal to 0 indicates that chroma_sample_loc_delta_x and chroma_sample_loc_delta_y are not present in the field indication SEI message.

- chroma_sample_loc_delta_x, specifies the horizontal chroma samples phase shift, in units of fourth luma samples of a frame, for potential use in chroma samples inverse shifting process of the output decoded frame to recover the original source chroma samples location. The value of chroma_sample_loc_delta_x shall be in the range of -3 to +3, inclusive.

- chroma_sample_loc_delta_y, specifies the vertical chroma samples phase shift, in units of fourth luma samples of a frame, for potential use in chroma samples inverse shifting process of the output decoded frame to recover the original source chroma samples location. The value of chroma_sample_loc_delta_y shall be in the range of -3 to +3, inclusive.

- lès:

NOTE – source chroma sample location is indicated by the addition of the location specified by the VUI syntax elements chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field, as described in Section E.2.1 and illustrated in Error: Reference source not found, and the displacement signalled by chroma_sample_loc_delta_x and chroma_sample_loc_delta_y described in this section.

## Application scenario

In the sequel, we assume the application scenario described in section 3.1 considering a 4:2:0 source content with the “nominal” chroma samples locations (i.e. chroma_sample_loc_type_top_field=0 and chroma_sample_loc_type_bottom_field=0).

Thus, in order to signal a phase shifting of ¼ pixels down on the top field and of ¼ pixels up on the bottom field the proposed syntax elements are set as follows.

Top field:

chroma_sample_loc_delta_present_flag		set to 1

chroma_sample_loc_delta_x				set to 0

chroma_sample_loc_delta_y				set to -1

Bottom field:

chroma_sample_loc_delta_present_flag		set to 1

chroma_sample_loc_delta_x				set to 0

chroma_sample_loc_delta_y				set to +1

# Test conditions

Experiments are conducted on HM-6.0 software. One field picture in interlaced sequences is considered as one frame and therefore interlaced sequences are encoded as sequences with half height and doubled frame rate.

Four SD (720x480/60i) and four HD (1920x1080/60i) sequences specified in ITU-R BT.1120[2] have been used: #15 "Flower garden SD", #29 "Table tennis SD", #30 "Mobile and calendar SD" and #38 "Football SD"; #8 "Walk through the square HD", #13 "Church HD ", #16 "Whale show HD " and, #22 "Marching in HD ". Those sequences are encoded under the common condition described in F900[3] considering "Random Access High Efficiency" configurations.

Before encoding, top and bottom fields of each input sequences are separated. Then, phase shifting of + ¼ pixels on the top field and - ¼ pixels on the bottom field are performed using Lanczos-3 filtering (6 taps). Then, after encoding and decoding with HM 6.0, inverse shifting is performed.

In order to assess the performance of the proposal, for each sequences three cases have been considered:

- REF_ENCODED: corresponds to the sequence made up of separate fields coded and decoded.

- SHIFTED_ENCODED1: corresponds to the shifted sequence coded and decoded followed by an inverse shifting process.

- SHIFTED_ENCODED2: corresponds to the shifted sequence coded and decoded without inverse shifting process.

# Experiments

## Results

Table 1 summarizes the results (BD-rate variations in %) obtained with the proposed chroma re-alignment. Proposed solution (i.e. SHIFTED_ENCODED1) is compared to reference solution (i.e. REF_ENCODED) without additional chroma processing, both encoded with HM 6.0.

Table 1 Results of BD-Rate comparing Proposal (SHIFTED_ENCODED1) to HM6.0.

|  | BD-rate (RA-HE) |  |  |
| --- | --- | --- | --- |
| Video Sequences | Y | U | V |
| Flower_garden_SD | -0.15% | -10.5% | -6.2% |
| Football_SD | -0.16% | -6.4% | -4.7% |
| Mobile_and_calendar_SD | -0.11% | -8.7% | -6.1% |
| Table_tennis_SD | -0.05% | -3.6% | -9.4% |
| Walk_through_the_Square_HD | -0.37% | -8.0% | -9.6% |
| Church_HD | 0.13% | -1.9% | -0.57% |
| Whale_Show_HD | -0.02% | 3.5% | 0.02% |
| Marching_in_HD | -0.14% | -14.7% | -15.2% |
| Overall | -0.11% | -6.3% | -6.5% |

An average coding gain of -6.3% and -6.5% on respectively chroma U and V components is reported for Random Access High Efficiency combined with a gain of -0.11% for luma component. Proposed solution is particularly efficient for panning sequences such as  “Walk_through_the_Square_HD” and “Marching_in_HD” sequences with respectively (-8.0%; -9.6%) and (-14.7%; -15.2%) for (U,V) and a significant gain in luma due to the improvement of chroma prediction.

In order to fully assess the current chroma re-alignment proposal, we investigated performances without SEI considerations at the decoder side. Table 2 reports the corresponding gain when no post-processing is performed at decoder level.

Table 2 Results of BD-Rate comparing Proposal (SHIFTED_ENCODED2) to HM6.0.

|  | BD-rate (RA-HE) |  |  |
| --- | --- | --- | --- |
| Video Sequences | Y | U | V |
| Flower_garden_SD | -0.15% | -0.83% | -2.3% |
| Football_SD | -0.16% | 2.2% | -1.2% |
| Mobile_and_calendar_SD | -0.11% | -2.6% | -1.9% |
| Table_tennis_SD | -0.05% | -2.1% | -1.5% |
| Walk_through_the_Square_HD | -0.37% | -1.1% | -2.6% |
| Church_HD | 0.13% | -0.69 | -0.27% |
| Whale_Show_HD | -0.02% | 2.9% | 1.3% |
| Marching_in_HD | -0.14% | -5.4% | -4.5% |
| Overall | -0.11% | -0.95% | -1.6% |

An average coding gain of -0.95% and -1.6% respectively for chroma U and V is reported with same results for luma component as ones reports in Table 1. The efficiency of this solution is indeed smaller but there is still an improvement due to the bitrate decrease at all tested QPs while PSNR-U and PSNR-V are worse. Results confirm the need for SEI consideration (and therefore chroma inverse shifting) at decoder side when using this proposal (respectively -5.3% and -4.9% in average between both solutions).

## Further comments

In the reported experiments, simple Lanczos 3 filter has been used in order to shift the chroma samples.With a more accurate filter design, one can expect more significant performances gain closed to 15% in average on chroma as presented in [4].

# Conclusion

In this contribution, we propose a method to deal with the misalignment of chroma samples locations of top field with regard to bottom field, that does not require any modification in the HEVC encoding process. Thus, we propose to signal that both top and bottom fields chroma samples have been aligned prior to the encoding process in order to allow a proper realignment after decoding.

It has been shown that the proposed method leads to significant gains of performance when encoding 4:2:0 interlaced material. Therefore, we suggest the JCTVC group to consider the current signalling proposal for adoption.

# Reference(s)

- Benjamin Bross, Woo-Jin Han, Jens-Rainer Ohm, Gary J. Sullivan and Thomas Wiegand, “High efficiency video coding (HEVC) text specification draft 6”, JCT-VC Document JCTVC-H1003-v22, February 2012, San Jose, USA.

- Rec. ITU-R BT.1210-3, "Test materials to be used in subjective assessment"

- "Common test conditions and software reference configurations", JCTVC-F900, July 2011, Torino IT

- Jumpei Koyama, Akihiro Yamori, Kimihiko Kazui, Satoshi Shimada and Akira Nakagawa, “Modification of derivation process of motion vector information for interlace format”, JCTVC-G196, Nov, 2011, Geneva, CH.

# Patent rights declaration(s)

ATEME may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).