| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0504 |
| --- | --- |

| Title: | Non-CE3: Cross-check of JCTVC-I0269 on High Throughput Level Coding with Context Reduction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Vivienne Sze | Email: | sze@ti.com |
| Source: | Texas Instruments |  |  |

_____________________________

# Abstract

This contribution reports cross-check results from Texas Instruments for I0269.  The BD-rate results under common conditions match results provided by RIM.

# Summary based on Analysis of Source Code

For coeff_abs_level_greater1_flag, I0269 proposes using bypass coding or coding it as part of coeff_abs_level_remaining rather than context index 0.

After the first coeff_abs_level_greater1_flag=1, bypass coding is used for the next bin coeff_abs_level_greater1_flag and then the remaining coefficients are coded with coeff_abs_level_remaining.

In the code, coeff_abs_level_greater1_flag is either context coded, bypass coded or coded as part of coeff_abs_level_remaining.  The location of the 2nd bin after coeff_abs_level_greater1_flag=1 needs to be known for coding coeff_abs_level_remaining.

Two schemes are in the code:

- switching to bypass always occurs after first coeff_abs_level_greater1_flag=1

- only apply bypass switching when ctxSet is odd, which indicates that previous sub-blocks/coefficient groups has many trailing ones.

Some additional changes were made in encoder for rate calculation.

rateIncUp[ uiBlkPos ] = xGetICRate( 1, uiOneCtx, uiAbsCtx, uiGoRiceParam, c1Idx, c2Idx );

is used rather than

rateIncUp[ uiBlkPos ] = m_pcEstBitsSbac->m_greaterOneBits[ uiOneCtx ][ 0 ];

# Experiment Results

The HM-6.0 software is used for the evaluation, the simulation platforms is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz  64 bits Linux machines, the common test conditions and reference configurations

Scheme 1

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 104% |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.3% | -0.2% | -0.1% | 0.2% | -0.2% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| Class D | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | 0.2% |
| Class E |  |  |  |  |  |  |
| Overall | 0.1% | -0.1% | 0.0% | 0.1% | 0.0% | 0.0% |
|  | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 97% |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | -0.2% | 0.1% | 0.1% | 0.1% | 0.0% |
| Class C | 0.1% | 0.2% | 0.3% | 0.0% | 0.1% | -0.1% |
| Class D | 0.1% | 0.3% | -0.3% | 0.1% | -0.2% | -0.3% |
| Class E | 0.1% | -0.9% | 0.1% | 0.0% | -0.4% | 0.2% |
| Overall | 0.1% | -0.1% | 0.0% | 0.1% | -0.1% | 0.0% |
|  | 0.1% | -0.1% | 0.0% | 0.1% | 0.0% | 0.1% |
| Class F | -0.1% | 0.0% | -0.4% | 0.1% | 0.7% | 0.7% |
| Enc Time[%] | 95% |  |  | 95% |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

Scheme 2

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| Class B | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% |
| Class E | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% |
| Overall | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% |
|  | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class F | -0.1% | 0.0% | 0.0% | -0.1% | -0.1% | -0.2% |
| Enc Time[%] | 95% |  |  | 97% |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.2% | -0.1% | -0.2% | 0.2% | -0.2% | -0.4% |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% | -0.1% |
| Class C | -0.1% | 0.1% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class D | -0.1% | 0.1% | 0.0% | 0.0% | 0.1% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.2% |
|  | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% | -0.2% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 102% |  |  | 99% |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.2% | 0.1% | 0.0% | -0.3% | -0.1% |
| Class C | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class D | 0.0% | -0.1% | 0.2% | 0.0% | -0.2% | -0.1% |
| Class E | 0.1% | -1.1% | -0.5% | -0.1% | -0.7% | 0.5% |
| Overall | 0.0% | -0.3% | 0.0% | 0.0% | -0.3% | 0.1% |
|  | 0.0% | -0.2% | 0.0% | 0.0% | -0.2% | 0.0% |
| Class F | -0.2% | -0.1% | -0.4% | 0.0% | 0.5% | 0.4% |
| Enc Time[%] | 96% |  |  | 92% |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |