| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0505 |
| --- | --- |

| Title: | Non-CE3: Cross-check of JCTVC-I0102 on diagonal scan for quantization matrix coefficients |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Vivienne Sze | Email: | sze@ti.com |
| Source: | Texas Instruments |  |  |

_____________________________

# Abstract

This contribution reports cross-check results from Texas Instruments for I0102.  The BD-rate results under common conditions match results provided by ETRI.  The quantization matrices scaling_list_symmetry1_r.txt is used.

# Summary based on Analysis of Source Code

Diagonal scan is used for scanning quantization matrices coefficients rather than zig-zag scan.

UInt* scan  = g_auiFrameScanXY [ (sizeId == 0)? 1 : 2 ];

is replaced by

UInt* scan  = g_auiSigLastScan [ SCAN_DIAG ] [ (sizeId == 0)? 1 : 2 ];

# Experiment Results

The HM-6.0 software is used for the evaluation, the simulation platforms is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz  64 bits Linux machines, the common test conditions and reference configurations

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0000% | 0.0000% | 0.0000% | 0.0000% | 0.0000% | 0.0000% |
| Class B | 0.0001% | 0.0000% | 0.0000% | 0.0001% | 0.0000% | 0.0000% |
| Class C | 0.0001% | 0.0001% | 0.0001% | 0.0001% | 0.0001% | 0.0001% |
| Class D | 0.0003% | 0.0003% | 0.0003% | 0.0003% | 0.0003% | 0.0003% |
| Class E | 0.0001% | 0.0001% | 0.0001% | 0.0001% | 0.0001% | 0.0001% |
| Overall | 0.0001% | 0.0001% | 0.0001% | 0.0001% | 0.0001% | 0.0001% |
|  | 0.0001% | 0.0001% | 0.0000% | 0.0001% | 0.0001% | 0.0001% |
| Class F | 0.0001% | 0.0001% | 0.0001% | 0.0001% | 0.0001% | 0.0001% |
| Enc Time[%] | 101.0794% |  |  | 95.6179% |  |  |
| Dec Time[%] | 100.6166% |  |  | 93.8199% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0004% | 0.0003% | 0.0003% | 0.0004% | 0.0003% | 0.0003% |
| Class B | 0.0004% | 0.0004% | 0.0004% | 0.0004% | 0.0004% | 0.0004% |
| Class C | 0.0008% | 0.0008% | 0.0008% | 0.0008% | 0.0007% | 0.0007% |
| Class D | 0.0025% | 0.0023% | 0.0023% | 0.0026% | 0.0023% | 0.0024% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0010% | 0.0009% | 0.0009% | 0.0010% | 0.0009% | 0.0009% |
|  | 0.0010% | 0.0009% | 0.0012% | 0.0010% | 0.0009% | 0.0012% |
| Class F | 0.0008% | 0.0008% | 0.0008% | 0.0008% | 0.0008% | 0.0008% |
| Enc Time[%] | 97.7316% |  |  | 94.7001% |  |  |
| Dec Time[%] | 96.5784% |  |  | 92.3816% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0005% | 0.0004% | 0.0004% | 0.0005% | 0.0004% | 0.0004% |
| Class C | 0.0008% | 0.0007% | 0.0007% | 0.0008% | 0.0007% | 0.0007% |
| Class D | 0.0024% | 0.0022% | 0.0022% | 0.0025% | 0.0022% | 0.0022% |
| Class E | 0.0029% | 0.0026% | 0.0026% | 0.0030% | 0.0026% | 0.0026% |
| Overall | 0.0015% | 0.0013% | 0.0014% | 0.0015% | 0.0013% | 0.0013% |
|  | 0.0015% | 0.0014% | 0.0014% | 0.0015% | 0.0014% | 0.0014% |
| Class F | 0.0018% | 0.0016% | 0.0016% | 0.0018% | 0.0016% | 0.0017% |
| Enc Time[%] | 100.0935% |  |  | 99.9121% |  |  |
| Dec Time[%] | 100.8546% |  |  | 99.4238% |  |  |