| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0507 |
| --- | --- |

| Title: | Non-CE1: Coding of SAO merge left and merge up |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Koohyar Minoo and David Baylon 6450 Sequence Drive San Diego, CA 92121 USA I. S. Chong, M. Karczewicz Qualcomm 5775 Morehouse Dr, San Diego, CA 92121, USA | Tel: Email: | +1 (858) 404-2521 kminoo@motorola.com david.baylon@motorola.com Marta Karczewicz +1-858-658-5673 {ichong, martak}@qualcomm.com |
| Source: | Motorola Mobility and Qualcomm Inc. |  |  |

_____________________________

# Abstract

In the HM6.0 process, the coding of the SAO merge left and merge up is performed for each color component.  This contribution proposes to use the merge left flag to signal whether all color components are merge left, and to use the merge up flag to signal whether all color components are merge up.  Otherwise, SAO offset parameters are signaled for each color component.

# Introduction

In LCU-based SAO in HM6.0, the SAO parameters for an LCU can be inferred from a left neighbor (“merge left”), an above neighbor (“merge up”), or explicitly signaled (no merge).  In the existing HM6.0 version, the merge decision is first done on a color component basis.  For a given color component it is first signaled whether the SAO parameters for the LCU will be obtained from merge left (assuming the left neighbor exists).  If merge left is not chosen, merge up is checked (assuming the above neighbor exists).  If merge up is not chosen, then the parameters are explicitly signaled. These parameters include the SAO type, which can be one of three states of Skip, Edge or Band. If the SAO type is Edge or Band, then a corresponding number of offset values would be transmitted.

# Coding of merge left and merge up across color components

Instead of signaling merge left for each component separately and independently, it is proposed that the merge left flag is first signaled to indicate whether all color components are inferred from SAO parameter from left (merge left).  If so, all color components will use merge left mode.  If not, then merge up mode is checked for all color components (as explained above for merge left).  If merge up mode is not chosen, or if it does not exist then explicit SAO parameters are signaled for each color component.

# Proposed text changes

### 7.3.4  Slice data syntax

| slice_data( ) { | Descriptor |
| --- | --- |
| CtbAddrRS = SliceCtbAddrRS |  |
| CtbAddrTS = CtbAddrRStoTS[ CtbAddrRS ] |  |
| moreDataFlag = 1 |  |
| if( adaptive_loop_filter_flag && alf_cu_control_flag ) |  |
| AlfCuFlagIdx = −1 |  |
| do { |  |
| xCtb = InverseRasterScan( CtbAddrRS, CtbSize, CtbSize, pic_width_in_luma_samples, 0 ) |  |
| yCtb = InverseRasterScan( CtbAddrRS, CtbSize, CtbSize, pic_width_in_luma_samples, 1 ) |  |
| NumPCMBlock = 0 |  |
| CtbAddrInSlice = CtbAddrRS − ( slice_address >> SliceGranularity ) |  |
| AddrUp= CtbAddrRS – PicWidthInCtbs |  |
| if( slice_sao_interleaving_flag ) { |  |
| if(slice_sample_adaptive_offset_flag) |  |
| sao_unit_cabac( xCtb, yCtb, 0 ) |  |
| if(sao_cb_enable_flag) |  |
| sao_unit_cabac( xCtb, yCtb, 1 ) |  |
| if(sao_cr_enable_flag) |  |
| sao_unit_cabac( xCtb, yCtb, 2 ) |  |
| } |  |
| moreDataFlag = coding_tree( xCtb, yCtb, Log2CtbSize, 0 ) |  |
| CtbAddrTS++ |  |
| if( moreDataFlag && ( ( tiles_or_entropy_coding_sync_idc = = 1 && TileId[ CtbAddrTS ] != TileId[ CtbAddrTS − 1 ] ) ¦ ¦ ( tiles_or_entropy_coding_sync_idc = = 2 && num_substream_minus1 > 0 && CtbAddrTS / PicWidthInCtbs <= num_substream_minus1 && CtbAddrTS % PicWidthInCtbs = = 0 ) ) ) { |  |
| rbsp_trailing_bits( ) |  |
| if( nextbits(24) = = 0x000002 ) { |  |
| entry_point_marker_two_3bytes | f(24) |
| tile_idx_minus_1 | u(v) |
| } |  |
| } |  |
| } while( moreDataFlag ) |  |
| } |  |

#### 7.3.4.1  Sample adaptive offset unit CABAC syntax

| sao_unit_cabac( rx, ry, cIdx ){ | Descriptor |
| --- | --- |
| if( rx > 0 & & CtbAddrInSlice != 0 ) |  |
| sao_merge_left_flag | ae(v) |
| if( !sao_merge_left_flag ) { |  |
| if( ry > 0 & & ( AddrUp > 0 ¦ ¦ slice_loop_filter_across_slices_enabled_flag ) ) |  |
| sao_merge_up_flag | ae(v) |
| if( !sao_merge_up_flag ) { |  |
| sao_offset_cabac( rx, ry, cIdx ) |  |
| sao_offset_cabac( rx, ry, 0 ) |  |
| if(sao_cb_enable_flag) |  |
| sao_offset_cabac( rx, ry, 1 ) |  |
| if(sao_cr_enable_flag) |  |
| sao_offset_cabac( rx, ry, 2 ) |  |
| } |  |
| } |  |
| } |  |

# Simulation results

Tables 1-3 show the results of the proposed method compared to HM6.0 with SAO off for LCU sizes of 64, 32, and 16, respectively.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.6% | -1.4% | -1.5% | -0.3% | -0.6% | -1.9% |
| Class B | -0.4% | -1.5% | -2.6% | -0.2% | -0.5% | -1.1% |
| Class C | -0.5% | -2.2% | -3.2% | -0.4% | -0.8% | -1.3% |
| Class D | -0.3% | -1.3% | -1.8% | -0.3% | -0.3% | -0.6% |
| Class E | -0.4% | -2.7% | -2.9% | -0.3% | -1.0% | -0.7% |
| Overall | -0.4% | -1.8% | -2.4% | -0.3% | -0.6% | -1.2% |
|  | -0.4% | -1.7% | -2.4% | -0.3% | -0.6% | -1.1% |
| Class F | -2.3% | -3.4% | -4.5% | -2.4% | -2.0% | -2.9% |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -2.2% | -4.3% | -3.4% | -0.6% | -2.0% | -2.3% |
| Class B | -1.9% | -3.6% | -3.7% | -0.8% | -1.4% | -2.0% |
| Class C | -0.9% | -3.2% | -4.0% | -0.7% | -1.5% | -2.1% |
| Class D | 0.1% | -1.6% | -1.6% | -0.3% | -1.1% | -1.2% |
| Class E |  |  |  |  |  |  |
| Overall | -1.3% | -3.2% | -3.2% | -0.6% | -1.5% | -1.9% |
|  | -1.2% | -3.2% | -3.1% | -0.6% | -1.5% | -1.9% |
| Class F | -4.1% | -4.7% | -6.7% | -4.0% | -2.3% | -4.4% |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -1.9% | -5.7% | -6.1% | -1.1% | -1.0% | -1.6% |
| Class C | -1.4% | -6.0% | -9.0% | -1.1% | -2.0% | -3.1% |
| Class D | -0.2% | -3.7% | -4.3% | -0.5% | -0.6% | -0.4% |
| Class E | -1.1% | -5.5% | -6.3% | -0.9% | 0.4% | 1.0% |
| Overall | -1.2% | -5.3% | -6.4% | -0.9% | -0.9% | -1.2% |
|  | -1.2% | -5.2% | -6.4% | -0.9% | -1.0% | -1.1% |
| Class F | -6.8% | -7.2% | -10.8% | -5.5% | -2.0% | -5.4% |
| Enc Time[%] | 111% |  |  | 112% |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -10.7% | -8.8% | -8.9% | -1.9% | -1.0% | -2.1% |
| Class C | -6.8% | -8.4% | -10.6% | -2.0% | -2.9% | -3.8% |
| Class D | -3.9% | -5.5% | -5.9% | -1.4% | -1.7% | -1.9% |
| Class E | -10.4% | -9.5% | -9.9% | -2.2% | -1.3% | -0.2% |
| Overall | -8.0% | -8.0% | -8.8% | -1.8% | -1.7% | -2.1% |
|  | -8.1% | -7.9% | -8.8% | -1.8% | -1.7% | -2.2% |
| Class F | -8.5% | -8.2% | -12.2% | -6.7% | -3.3% | -6.4% |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

Table 1.  Proposed method with LCU = 64.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | 0.1% | 0.1% | 0.1% | 0.0% | -0.8% |
| Class B | -0.1% | 0.1% | -0.1% | 0.1% | 0.2% | 0.1% |
| Class C | -0.2% | -0.4% | -0.9% | -0.1% | 0.1% | -0.1% |
| Class D | -0.1% | 0.1% | -0.1% | -0.1% | 0.2% | 0.1% |
| Class E | 0.0% | 0.1% | 0.2% | 0.1% | 0.2% | 0.3% |
| Overall | -0.1% | 0.0% | -0.2% | 0.0% | 0.1% | -0.1% |
|  | -0.1% | 0.0% | -0.2% | 0.0% | 0.1% | -0.1% |
| Class F | -2.0% | -0.9% | -1.5% | -1.9% | -0.1% | -0.5% |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.7% | -1.2% | -0.9% | 0.0% | -0.7% | -1.0% |
| Class B | -1.1% | -1.0% | -0.8% | -0.2% | 0.0% | 0.0% |
| Class C | -0.4% | -1.1% | -1.5% | -0.2% | -0.1% | -0.2% |
| Class D | 0.6% | 0.1% | 0.0% | 0.2% | 0.2% | 0.4% |
| Class E |  |  |  |  |  |  |
| Overall | -0.4% | -0.8% | -0.8% | -0.1% | -0.1% | -0.2% |
|  | -0.4% | -0.8% | -0.8% | -0.1% | -0.1% | -0.2% |
| Class F | -3.0% | -1.9% | -3.2% | -2.8% | -0.2% | -1.3% |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.8% | -0.9% | -0.8% | -0.2% | -0.1% | -0.1% |
| Class C | -0.6% | -1.5% | -3.0% | -0.3% | 0.0% | -0.1% |
| Class D | 0.3% | -0.4% | -1.0% | 0.3% | 0.3% | 0.4% |
| Class E | 0.3% | 0.5% | -0.2% | 0.9% | 0.9% | 1.3% |
| Overall | -0.3% | -0.7% | -1.3% | 0.1% | 0.2% | 0.3% |
|  | -0.3% | -0.6% | -1.3% | 0.1% | 0.3% | 0.2% |
| Class F | -4.4% | -2.3% | -3.8% | -3.3% | -1.8% | 0.7% |
| Enc Time[%] | 102% |  |  | 100% |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -7.7% | -2.4% | -1.9% | -0.5% | -0.2% | 0.1% |
| Class C | -5.2% | -3.0% | -4.2% | -1.0% | -0.5% | -1.2% |
| Class D | -2.8% | -1.2% | -1.4% | -0.7% | 0.5% | -0.2% |
| Class E | -6.7% | -0.6% | -0.9% | -0.1% | 0.4% | 0.3% |
| Overall | -5.7% | -1.9% | -2.2% | -0.6% | 0.0% | -0.3% |
|  | -5.7% | -2.0% | -2.2% | -0.6% | 0.0% | -0.2% |
| Class F | -6.0% | -2.1% | -4.8% | -4.2% | -0.1% | -1.3% |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

Table 2.  Proposed method with LCU = 32.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.2% | 0.2% | 0.1% | 0.1% | 0.0% |
| Class B | 0.2% | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% |
| Class C | 0.2% | 0.2% | 0.3% | 0.1% | 0.1% | 0.1% |
| Class D | 0.2% | 0.3% | 0.3% | 0.1% | 0.1% | 0.1% |
| Class E | 0.2% | 0.3% | 0.3% | 0.1% | 0.1% | 0.1% |
| Overall | 0.2% | 0.2% | 0.3% | 0.1% | 0.1% | 0.1% |
|  | 0.2% | 0.2% | 0.3% | 0.1% | 0.1% | 0.1% |
| Class F | -0.8% | 0.7% | 0.7% | -0.9% | 0.4% | 0.4% |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.4% | 0.5% | 0.4% | 0.4% | 0.4% | 0.4% |
| Class B | 0.4% | 0.8% | 0.8% | 0.5% | 0.5% | 0.5% |
| Class C | 0.4% | 0.5% | 0.6% | 0.3% | 0.3% | 0.3% |
| Class D | 1.0% | 1.1% | 1.0% | 0.6% | 0.5% | 0.5% |
| Class E |  |  |  |  |  |  |
| Overall | 0.5% | 0.7% | 0.7% | 0.5% | 0.4% | 0.4% |
|  | 0.6% | 0.7% | 0.7% | 0.5% | 0.4% | 0.5% |
| Class F | -0.8% | 1.1% | 1.0% | -0.6% | 0.9% | 0.8% |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.5% | 0.4% | -0.2% | 0.4% | 0.4% | 0.5% |
| Class C | 0.4% | 0.6% | 0.4% | 0.3% | 0.2% | 0.3% |
| Class D | 0.8% | 0.7% | 0.6% | 0.6% | 0.6% | 0.7% |
| Class E | 2.3% | 2.2% | 2.6% | 2.2% | 2.0% | 1.9% |
| Overall | 0.9% | 0.9% | 0.7% | 0.8% | 0.7% | 0.8% |
|  | 0.9% | 0.9% | 0.7% | 0.8% | 0.7% | 0.8% |
| Class F | -0.5% | 0.9% | -0.2% | 0.3% | 1.1% | 0.9% |
| Enc Time[%] | 110% |  |  | 110% |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -2.1% | 0.1% | 0.3% | 0.5% | 0.0% | -0.4% |
| Class C | -1.5% | 0.0% | 0.3% | 0.2% | 0.0% | 0.3% |
| Class D | -0.5% | 0.4% | 1.1% | 0.3% | 0.9% | 0.9% |
| Class E | 0.5% | 2.2% | 2.3% | 2.1% | 1.8% | 1.8% |
| Overall | -1.1% | 0.5% | 0.9% | 0.7% | 0.6% | 0.5% |
|  | -1.1% | 0.6% | 0.9% | 0.7% | 0.6% | 0.6% |
| Class F | -1.4% | 1.1% | 3.3% | -0.4% | 1.3% | 1.5% |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

Table 3.  Proposed method with LCU = 16.

# Patent rights declaration(s)

Motorola Mobility may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).