| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0509 |
| --- | --- |

| Title: | Cross-check of Updated design of the probability estimator and the coding engine of the M Coder in JCTVC-I0276 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Seung-Hwan Kim, Andrew Segall 5750 NW Pacific Rim Blvd Camas, WA 98607 | Tel: Email: | +1 360 817 8451 kimse@sharplabs.com asegall@sharplabs.com |
| Source: | Sharp |  |  |

_____________________________

# Abstract

This is a cross-check report for JCTVC-I0276 on updated design of the probability estimator and the coding engine of the M coder. The proposal document was studied and the changes in the source provided by the proponents were found to be consistent with the proposal document. The proponents provided corresponding software which was used to generate the results under the common test condition. It is confirmed that all the results are matched to the results in JCTVC-I0276.

# Introduction

This contribution proposes a set of modifications to the current design of the probability estimator and the entropy coding engine of HEVC. In JCTVC-I0276, the probability model initialization for LPS is updated to cover the range of 112 states instead of 63. Consequently the initialization values are adjusted accordingly.  The following shows probability update process between JCTVC-I0276 and HM6.0. There is no additional required computational complexity in this process. Only expected load will be the extended 112 stats instead of 63 stats.

<Probability update in JCTVC-I0276>

uiLPS   = TComCABACTables::sm_aucLPSTable[ rcCtxModel.getState() ][ ( m_uiRange >> 5 ) & 7 ];

<Probability update in HM 6.0>

UInt  uiLPS = TComCABACTables::sm_aucLPSTable[rcCtxModel.getState()][(m_uiRange >>6 ) & 3 ];

# Conclusion

The updated design of the probability estimator and the coding engine of the M coder proposed in JCTVC-I0276 was evaluated and found to be consistent with that claimed by the proponents. There is some quite changes (intial states, probability update) in software.

# References

[1]	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-G1100, San Jose, CA, USA, February 2012.

[2]	B. Bross, W.-J. Han,  J.-R. Ohm, G. J. Sullivan, T. Wiegand,  “High Efficiency Video Coding (HEVC) Test Model 6 (HM 6) Encoder Description,” JCT-VC Document, JCTVC-G1003, San Jose, CA, USA, February 2012.