| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0513 |
| --- | --- |

| Title: | new 4K test sequence for HEVC standard extension |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kazuo Sugimoto Akira Minezawa Shun-ichi Sekiguchi Information Technology R&D Center Mitsubishi Electric Corporation 5-1-1, Ofuna, Kamakura, JAPAN | Tel: Email: | +81-467-41-2463 Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

# Abstract

As requested at the last meeting in San Jose, Mitsubishi Electric Corporation made available a new test sequence, Traffic4K, for the development of HEVC standard extension.

# Proposed test sequences

The original sequence was in RGB 10bit format captured using a DALSA Origin II camera. After rounding to 8 bit, it was converted to YUV4:4:4 format by using the following color conversion equation.

Y =   0.2126R + 0.7152G + 0.0722B

U =  -0.1146R - 0.3854G + 0.5000B

V =   0.5000R - 0.4542G - 0.0458B

Then YUV4:4:4 to YUV4:2:0 down-sampling was applied using 14 tap filter as below.

filter[14] = {2,0,-4,-3,5,19,26,19,5,-3,-4,0,2,6} >> 6

Finally, luma and chroma samples were clipped to the range of [16, 235] and [16, 240] respectively.

The test sequence has been uploaded named as traffic_4096x2048_30p.zip to the hannover ftp site as follows:

directory: scalable/sources_under_consideration

resolution: 4096 x 2048

duration: 10 seconds (300 frames)

format: YUV 4:2:0 (BT.709)

bit depth: 8bit/sample

md5sum: 54750c315b743cd4f013af64a5f2424a

Note that the pixel values of Traffic sequence for Class A may not be identical to the pixel values in this Traffic4K sequence as the conversion procedure is not the same.

Sample still images from each of the sequences is shown below.

# Copyright information

The test sequence and all intellectual property rights therein remain the property of the owner below. This material can only be used for the purpose of developing HEVC standards. This material cannot be distributed with charge. The owner makes no warranties with respect to the material and expressly disclaims any warranties regarding its fitness for any purpose.

Owner of the sequence "Traffc"

Owner: Plannet inc.

Production: Plannet inc. and IMAGICA Corp.

# Conclusion

A new test sequence, Traffic4K, was made available for the development of HEVC standard extension. Specification of the test sequence and copyright information are also provided.