| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0514 WG11 Number: m25071 |
| --- | --- |

| Title: | Cross-check of JCTVC-I0080 on parallel NxN merge mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | J. Jung Orange Labs 38-40 rue du Général Leclerc 9279 ISSY LES MOULINEAUX | Tel: Email: | +33 1 45 29 53 66 joelb.jung@orange.com |
| Source: | Orange Labs |  |  |

_____________________________

# Abstract

The document reports cross-check results for the contribution JCTVC-I0080. It has been verified that common conditions are followed and that results provided by the other tester (MediaTek) are complete and match with the results reported in this document. The source code has also been checked, the software has been compiled and no issue is reported.

# Test 1

For the NxN merge mode, the spatial merging candidates located in other NxN PUs are included in the merging candidate list, which prevents the NxN merge mode from being processed in parallel (NxN merge is not allowed in the common test conditions, yet allowed by CD (and by the Main Profile)). In JCTVC-I0080, the spatial merging candidates located in the other NxN PUs are excluded to remove data dependency between NxN PUs.

Anchor: HM6 with common conditions.

Tested: HM6 + inter 4x4

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0,1% | 0,0% | 0,1% | 0,0% | 0,0% | 0,0% |
| Class B | 0,0% | 0,0% | 0,0% | 0,0% | -0,1% | -0,1% |
| Class C | -0,1% | -0,3% | -0,3% | -0,1% | -0,3% | -0,4% |
| Class D | -0,3% | -0,4% | -0,5% | -0,2% | -0,3% | -0,5% |
| Class E |  |  |  |  |  |  |
| Overall | -0,1% | -0,2% | -0,2% | -0,1% | -0,2% | -0,2% |
|  | -0,1% | -0,2% | -0,1% | -0,1% | -0,2% | -0,2% |
| Class F | -0,4% | -0,4% | -0,5% | -0,4% | -0,5% | -0,4% |
| Enc Time[%] | 114% |  |  | 112% |  |  |
| Dec Time[%] | 99% |  |  | 103% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0,0% | -0,3% | -0,2% | 0,0% | -0,2% | -0,2% |
| Class C | -0,2% | -0,3% | -0,3% | -0,2% | -0,1% | -0,4% |
| Class D | -0,3% | -1,0% | -0,1% | -0,3% | -1,1% | -1,0% |
| Class E | 0,1% | -0,4% | -0,1% | 0,0% | -0,6% | 0,1% |
| Overall | -0,1% | -0,5% | -0,2% | -0,1% | -0,5% | -0,4% |
|  | -0,1% | -0,5% | -0,2% | -0,1% | -0,4% | -0,4% |
| Class F | -0,7% | -0,9% | -0,3% | -0,6% | -0,1% | 0,2% |
| Enc Time[%] | 119% |  |  | 114% |  |  |
| Dec Time[%] | 97% |  |  | 100% |  |  |

The software has been verified and compiled. It does not raise any issue.

The cross check of the results have been done for all classes and Inter configurations. Results obtained by MediaTek and Orange Labs are reported in the following table. A perfect match is achieved.

# Test 2

Anchor: HM6 + inter 4x4

Tested: HM6 + inter 4x4 + proposed method

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0,0% | 0,0% | -0,1% | 0,0% | 0,0% | 0,0% |
| Class B | 0,0% | 0,0% | -0,1% | 0,0% | 0,0% | 0,0% |
| Class C | 0,1% | 0,1% | 0,1% | 0,0% | 0,1% | 0,1% |
| Class D | 0,1% | 0,0% | 0,2% | 0,0% | -0,1% | 0,0% |
| Class E |  |  |  |  |  |  |
| Overall | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
|  | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class F | 0,1% | 0,1% | 0,1% | 0,0% | 0,1% | 0,1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 98% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0,0% | 0,1% | 0,1% | 0,0% | 0,1% | 0,1% |
| Class C | 0,1% | 0,1% | 0,0% | 0,1% | 0,1% | 0,2% |
| Class D | 0,1% | 0,5% | -0,1% | 0,0% | 0,5% | 0,1% |
| Class E | 0,0% | 0,6% | 0,4% | 0,0% | 0,3% | -0,2% |
| Overall | 0,0% | 0,3% | 0,1% | 0,0% | 0,3% | 0,1% |
|  | 0,0% | 0,3% | 0,1% | 0,0% | 0,2% | 0,0% |
| Class F | 0,0% | -0,2% | -0,2% | 0,1% | 0,2% | -0,5% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 107% |  |  | 96% |  |  |

The software has been verified and compiled. It does not raise any issue.

The cross check of the results have been done for all classes and Inter configurations. Results obtained by MediaTek and Orange Labs are reported in the following table. A perfect match is achieved.

# Conclusion

This verification contribution confirms the results obtained by MediaTek.

It is observed that inter 4x4 only brings 0.1% BDR gain. On top of this, enabling full parallel processing brings very minor losses (below 0.1%).