| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-I0515 |
| --- | --- |

| Title: | Cross-check report of JCTVC-I0171 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Oguz Bici Kemal Ugur | Email: | ext-oguz.bici@nokia.com kemal.ugur@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution presents cross verification of Panasonic proposal JCTVC-I0171 on refinement for SAO and ALF syntax in the APS. It is verified that BD rate results perfectly match.

# Introduction

In JCTVC-I0171, it is proposed to change the name and the semantics of the four syntax elements in the APS, namely sao_num_lcu_in_width_minus1, sao_num_lcu_in_height_minus1, alf_num_lcu_in_width_minus1 and alf_num_lcu_in_height_minus1. According to the proposal, “number of SAO (or ALF) parameter units in picture height and width” are signaled in the APS instead of number of “LCUs in picture height and width”. In this way, it is claimed that, for the case of signaling in APS, grouping multiple LCUs into regions become possible, resulting in reduction of the signaling overhead.

# Cross verification and results

In the simulations, picture-based encoding algorithm is used where the SAO/ALF parameter set is signalled in the APS. The cross-check experiments presented in Table 1 correspond to the case where LCU size is 16x16.

Common test conditions are followed [1] using the software HM6.0 with the following modifications:

TICKET400 is used

Configuration is modified in the software as:

#if TEST_CONFIG

m_saoInterleavingFlag = 0;

m_bALFPicBasedEncode = 1;

m_iALFMaxNumberFilters = 16;

m_bALFParamInSlice = 0;

#if TEST_CONFIG == 2

m_uiMaxCUWidth = 16;

m_uiMaxCUHeight = 16;

m_uiMaxCUDepth = 2;

m_uiQuadtreeTULog2MaxSize = 3;

m_uiQuadtreeTULog2MinSize = 2;

m_uiQuadtreeTUMaxDepthIntra = 1;

m_uiQuadtreeTUMaxDepthInter = 2;

#endif

#endif

Table 1: Simulation results

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class B | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class C | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class D | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class E | -0.2% | -0.2% | -0.2% | -0.2% | -0.2% | -0.2% |
| Class F | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Overall | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
|  | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 99% |  |  | 97% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | -0.2% | -0.2% | -0.2% | -0.2% | -0.2% |
| Class B | -0.3% | -0.3% | -0.3% | -0.2% | -0.2% | -0.2% |
| Class C | -0.2% | -0.2% | -0.2% | -0.2% | -0.2% | -0.2% |
| Class D | -0.2% | -0.2% | -0.2% | -0.2% | -0.2% | -0.2% |
| Class E |  |  |  |  |  |  |
| Class F | -0.3% | -0.3% | -0.3% | -0.3% | -0.2% | -0.2% |
| Overall | -0.2% | -0.2% | -0.2% | -0.2% | -0.2% | -0.2% |
|  | -0.2% | -0.2% | -0.2% | -0.2% | -0.2% | -0.2% |
| Enc Time[%] | 100% |  |  | #NUM! |  |  |
| Dec Time[%] | 100% |  |  | 105% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.4% | -0.4% | -0.4% | -0.3% | -0.3% | -0.3% |
| Class C | -0.3% | -0.3% | -0.3% | -0.3% | -0.3% | -0.3% |
| Class D | -0.3% | -0.2% | -0.2% | -0.3% | -0.5% | -0.2% |
| Class E | -0.9% | -0.9% | -0.9% | -0.7% | -0.6% | -0.6% |
| Class F | -0.6% | -0.6% | -0.6% | -0.5% | -0.5% | -0.5% |
| Overall | -0.5% | -0.5% | -0.5% | -0.4% | -0.4% | -0.4% |
|  | -0.5% | -0.5% | -0.5% | -0.4% | -0.4% | -0.4% |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.4% | -0.4% | -0.4% | -0.3% | -0.3% | -0.3% |
| Class C | -0.3% | -0.3% | -0.3% | -0.2% | -0.2% | -0.2% |
| Class D | -0.3% | -0.2% | -0.2% | -0.2% | -0.3% | 0.1% |
| Class E | -1.0% | -0.9% | -0.9% | -0.7% | -0.7% | -0.7% |
| Class F | -0.6% | -0.6% | -0.6% | -0.5% | -0.5% | -0.5% |
| Overall | -0.5% | -0.4% | -0.4% | -0.4% | -0.4% | -0.3% |
|  | -0.5% | -0.4% | -0.4% | -0.4% | -0.4% | -0.3% |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

It has been verified that the BD-rate results shown in Table 1 perfectly matches with the ones reported in JCTVC-I0171.

# Conclusion

In this report, the experiment results presented in JCTVC-I0171 have been confirmed. The claimed benefit in JCTVC-I0171 is that it provides the possibility of grouping multiple LCUs into regions (for the case of signaling in APS) in order to reduce the signaling overhead. To justify the benefits, simulations for LCU size of 16x16 is conducted and coding gains of -0.1% for AI, -0.2% for RA, (-0.5)-(-0.4)% for LB and (-0.5)-(-0.4)% for LP on the average is reported.

# References

[1]	Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-H1100,  San Jose, USA, Feb 2012.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).