| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0520 |
| --- | --- |

| Title: | Parallel Scalability and Efficiency of WPP and Tiles. |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | C. C. Chi, M. Alvarez-Mesa, V. George B. Juurlink, T. Schierl Fraunhofer HHI Einsteinufer 37 10587, Berlin, Germany Technische Univesitat Berlin Einsteinufer 17, Berlin, Germany | Tel: Email: | +49 30 31002-227 thomas.schierl@hhi.frauhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document presents a parallel scalability and efficiency analysis of parallelization approaches being considered in HEVC, namely Tiles and Wavefront Parallel Processing (WPP). The two approaches have been implemented into HM4 and evaluated on an Intel Xeon/Westmere parallel machine with 12 cores running at 3.33 GHz. This document presents a comparison in terms of parallel scalability, processor usage efficiency and memory bandwidth.

# Introduction

In HEVC two main approaches are being considered for high-level parallel processing: Wavefront Parallel Processing (WPP) and Tiles. WPP allows creating picture partitions that preserve all the dependencies, including reconstruction and parsing. In order to respect all the dependencies and, at the same time, exploit parallelism, WPP follows a wavefront Coding Tree Blocks (CTBs) processing order. Tiles are rectangular groups of CTBs separated by vertical and horizontal boundaries. Tiles break all the dependencies, but filtering is applied across tiles boundaries.

The coding efficiency and syntax of these parallelization strategies have been analyzed previously but a performance comparison, however, has not yet been performed. In this document a parallel scalability analysis of WPP and Tiles is presented. For doing that, a parallel decoder that supports both approaches has been implemented in top of HM4 reference decoder. Performance results have been collected by running the multithreaded implementation on an Intel Xeon/Westmere multicore machine consisting of 12 cores (2 sockets with 6 cores each). Different experiments have been conducted for comparing both approaches including: speedup analysis, overhead of parallelization, efficiency of processor usage for different decoder stages and memory bandwidth requirements.

# Common decoder architecture

We have implemented both a parallel Tiles and WPP decoder using a pipelined decoder organization, illustrated in Figure 1. It consists of 3 pipeline stages: parse, issue, and output. Additionally there are N decoding threads that take work from a common queue. The issue thread waits until all work issued to the queue has been processed and the decoding threads have signaled completion. Thereafter, the issue thread informs the output thread of completion.

Figure 1 - Common parallel decoder architecture

## Tiles

For parallel Tiles decoding there are five parallel phases: (1) entropy decode and reconstruction, (2) vertical edge deblocking filter, (3) horizontal edge deblocking filter, (4) SAO, and (5) ALF. These phases are separated with barrier synchronizations implemented using the issue thread, which waits for the decoding threads to finish the phase before starting the next phase. For improved performance the filtering phases use a task size of 8 consecutive CTBs and use efficient atomic counters to divide the work among the decoding threads. Because the SAO and ALF filtered pixels cannot overwrite the unfiltered pixels a different picture buffer is used to store the filtered pixels illustrated in Figure 2.

In principle there should be also the possibility to filter the inner part of each tile in an in-loop step, while having an extra sequential Tile border filtering phase, after a single synchronization barrier following the parallel Tile decoding phase. Implementing such a step would require a much higher software code complexity and was not possible within the HM software structure. Therefore the implementation of such a step was not included in our implementation.

Figure 2 - Filtered pixels by SAO and ALF are written in a different picture buffer

## WPP

For WPP decoding all the operations are performed in one decoding step as illustrated in Figure 3, Figure 4, Figure 5, and Figure 6. To make sure the filters are applied in the right order compliant to HM 4.1, the deblocking filter is delayed by 0.5 CTB, the SAO is delayed 1 CTB horizontally and 4 pixels vertically, the ALF is delayed 1 CTB + 12 pixels horizontally and 8 pixels vertically. These delays are larger than absolutely necessary, but required much less changes in the software. Further investigation is necessary to determine the right offsets for each filter for both hardware and software implementations.

Figure 3 - Deblock vertical edges

Figure 4 - Deblock horizontal edges

Figure 5 - SAO

Figure 6 - ALF

In WPP the SAO and ALF intermediate picture buffers can be avoided because of the strict wavefront decoding order.  Figure 7 shows how the intermediate pictures can point to the same picture in WPP. Filtered pixels will be implicitly written to a location that is slightly offsetted. This is not possible for Tiles because there is no order constraint. The additional memory requirements are small, with only 3 extra pixel lines in height and 6 pixels columns in width. To keep the CTBs aligned to 16 bytes, however, a horizontal offset of 8 pixels is used per picture buffer.

Figure 7 - Intermediate buffers can point to a single slightly enlarged picture

# Results

## Decoder Capabilities

As the parallel code is based on HM.4.1 [1], it implements mostly the coding tools in HM 4.1. For the experiments the deblocking filter has been updated to HM 6.0, using the vertically filtered pixels for horizontal decisions. Additionally, in order to implement CTB based ALF, the ALF CTB on/off flags are limited to one per CTB, i.e, depth 0.

## Experimental setup

Multithreading has been performed using the C++ Boost libraries, which offer a convenient C++ wrapper around platform-dependent threading libraries such as POSIX threads (pthreads). The sequential codebase has been improved by reducing redundant memory buffers and operations. For the experiments, the sequences encoded with 1 slice per picture are used as the baseline against which speedup will be measured.

The system employed to measure performance is a cache-coherent shared memory system with two Intel Xeon Westmere X5680 processors, each with 6 cores. More details of the hardware/software environment are listed in Table 1.

Table 1 - Experimental setup

| System |  | Software |  |
| --- | --- | --- | --- |
| Processor | Intel X5680 | Boost C++ | 1.46.1 |
| architecture | Westmere | Compiler | gcc 4.6.1 |
| Sockets | 2 | Opt. level | -O3 |
| Cores/socket | 6 | Kernel | 3.0.0-16 |
| Frequency | 3.33 GHz | OS | Kubuntu 11.10 |
| L3-cache | 12MB/socket | Measurement | perf, PAPI |
| SMT | disabled | tools | taskset, time |
| TurboBoost | disabled | HM | 4.1-dev (r1527) |

In the experiments we pin decoding threads to cores to reduce the influence of the operating system scheduling policy and improve the reproducibility of the results. The experiments are performed for all sequences and using 1 to 12 decoding threads.

## Encoder configuration and test sequences

The simulations are performed according to [2] using the Random Access High Efficiency configuration. Because parallelization would be required mainly for High Definition we tested only Class A and Class B sequences. Additionally to the common test conditions, two 2160p sequences have been added [3].

## Picture Partitions

For WPP, a configuration with one picture partition per LCU is used. This approach results in 17, 25 and 34 picture partitions for 1080p, 1600p and 2160p respectively. For Tiles, configurations with square tiles (or the closest match) and with the same (or the closest match) level of parallelism than WPP are chosen. The selected Tiles configurations (in cols x rows) are: 6x3, 6x4 and 7x5 for 1080p, 1600p and 2160p respectively.

## Speedup

The speedup as a function of the number of decoding threads is depicted in Figure 8, Figure 9 and Figure 10 for 1080p, 1600p and 2160p resolutions respectively. The speedup results show that for each resolution Tiles achieves slightly higher speedup than WPP. The results also show that in general the speedup increases with the resolution. For both approaches the speedup is not linear, indicating that there are some parallelization inefficiencies. Those can be explained in more detail by examining the CPU time increase and the CPU usage factor.

Figure 8 - Speedup for Class A (1080p)

Figure 9 - Speedup for Class B (1600p)

Figure 10 - Speedup for Class 'S' (2160p)

## CPU usage and CPU time increase

The relation between the speedup, CPU time increase, and CPU usage factor is given in Equation 1,

(1)

where is the sequential execution time,  is the parallel execution time,  is the CPU usage factor, and  is the CPU inefficiency factor. The CPU time increase is the percentile form of the CPU inefficiency factor. We introduced the  and  metrics, because they represent the scalability and (in)efficiency of the parallel solution, and can be easily derived from the CPU time (user + system time) measured by the UNIX time command. The CPU usage factor is presented in Figure 11, Figure 12 and Figure 13 for 1080p, 1600p and 2160p resolutions respectively. The CPU time increase is shown in Figure 14, Figure 15 and Figure 16 for the same resolutions.

The CPU usage factor shows the average aggregated CPUs used during execution. This factor is derived by dividing the CPU time (the sum of the execution time of all threads) by the parallel execution time. Ideally this is equal to the number of used threads and shows the scalability of the parallel algorithm. The figures show that Tiles in all cases have the highest CPU usage factor, closely followed by WPP. With higher resolutions the scalability is higher in general as there is more parallelism available.

The CPU usage factor is, however, not ideal for any of the strategies. For WPP this is caused by dependency stalls and ramping inefficiencies (roll-on/roll-off delays). Dependency stalls originate from the variability in the CTB execution times. Wavefront execution must respect the dependencies to the top right CTB, which results in dependency stalls when the difference in CTB execution time between rows becomes too large. WPP has, in addition, the ramping stalls, which occur at the beginning and end of decoding a picture when not all decoding threads can be active. Tiles, while having completely independent parallel phases, also does not scale ideally. This originates from a load balancing problem, in which not every decoding thread gets the same amount of work to process. For instance in 1080p there are 18 tiles for the entropy and reconstruction phase. This does not divide evenly when there are, e.g., 4 decoding threads. Furthermore, similar to CTBs, Tiles vary in execution time which exacerbates the problem.

Figure 11 - CPU usage factor for Class B (1080p)

Figure 12 - CPU usage factor for Class A (1600p)

Figure 13 - CPU usage factor for Class 'S' (2160p)

Figure 14 - CPU time increase for Class B (1080p)

Figure 15 - CPU time increase for Class A (1600p)

Figure 16 - CPU time increase for Class 'S' (2160p)

The CPU time increase corresponds to the increase in CPU time compared to the baseline, using one decoding thread for the sequences encoded with 1 slice per frame. The CPU time is the aggregated active thread time. A thread is active only when a thread is scheduled by the operating system. Ideally this metric should stay constant when increasing the number of threads. An increase in CPU time represents inefficiencies in the parallel hardware, algorithm, and/or implementation. A higher CPU time indicates either increased number of executed instructions or fewer instructions per cycle (IPC). Extra instructions can be required, for example, due to polling synchronization or excessive blocking and waking up of threads. We have measured no significant increase in the number of executed instructions, and therefore the increase in CPU time is mainly caused by a decreasing IPC. With higher number of threads, contention on shared resources, such as shared caches and memory controllers, and cache coherence misses result in IPC decreases. The CPU time increase for Tiles and WPP is shown in,  and .

The CPU time results show that Tiles have in our implementation a much larger CPU time increase compared to WPP. This is caused by having to perform the in-loop filters in separate picture passes for Tiles. Having additional passes results in more requests to the shared L3 cache and the off-chip memory controller as the picture does not fit in the private L2 caches. Due to contention on these resources when increasing the number of threads, the individual requests take more time. For WPP all the kernels are performed in one pass, in which most of the pixels touched in each kernel are almost immediately reused in the next stage, and then they are highly likely to remain in the private caches.

## IPC (instructions per cycle) analysis

We have analyzed the effect of the parallelization strategies on each individual decoding stage. In the following, the IPC (Instructions per Cycle) of each kernel is plotted against the number of decoding threads.

For entropy decoding (ED) (Figure 17, Figure 18 and Figure 19) the overall IPC is low due to the sequential nature of the CABAC algorithm. When increasing the number of decoding threads the IPC for Tiles stay constant while for WPP it decreases slightly. This is because for Tiles entropy decoding is completely independent, while for WPP contexts are selected based on the entropy decoded syntax elements of the top neighboring row.

Figure 17 - IPC for entropy decoding (1080p)

Figure 18 - IPC for entropy decoding (1600p)

Figure 19 - IPC for entropy decoding (2160p )

For reconstruction (REC) (Figure 20, Figure 21 and Figure 22) the two approaches are performing similar. With higher number of decoding threads the IPC decreases because of contention to the shared cache and memory controller for reading the reference data. While the Tiles approach has better motion compensation bandwidth characteristics, this is not appreciable in multicore architectures with shared caches.

Figure 20 - IPC for reconstruction (1080p)

Figure 21 - IPC for reconstruction (1600p)

Figure 22 - IPC for reconstruction (2160p)

The IPC results for the deblocking filter (Figure 24, Figure 25) show that WPP has a constant IPC independent of the number of threads (and sockets). Tiles have a lower IPC, due to separated filter passes. For 1080p the entire input and output picture fits in the last-level cache and the difference between WPP and Tiles is smaller. For higher resolutions (1600p and 2160p), the LLC is not big enough to contain the pictures which results in lower IPC for Tiles. Tiles are also affected more by NUMA effects (inter-socket communication when more than 6 cores are used).

Figure 23 - IPC for deblocking filter (1080p)

Figure 24 - IPC for deblocking filter (1600p)

Figure 25 - IPC for deblocking filter (2160p)

Figure 26, Figure 27 and Figure 28 show the IPC of SAO for 1080p, 1600p and 2160p resolutions respectively. For 1080p and one socket WPP and Tiles have a similar IPC. This is due to the fact that SAO is not accessing the entire picture (the filter can be turn off for entire areas) and the data can fit in the cache. When the resolution increases Tiles IPC decreases while WPP remains constant. Like in the deblocking filter Tiles IPC decreases because of NUMA effects.

Figure 26 - IPC for SAO (1080p)

Figure 27 - IPC for SAO (1600p)

Figure 28 - IPC for SAO (2160p)

Finally, Figure 29, Figure 30 and Figure 31 show the IPC for ALF for 1080p, 1600p and 2160p resolutions respectively. Tiles have a lower IPC compared to WPP due to the separate filter step and the difference increases with the resolution and with the number of cores due to increased cache misses and NUMA effects respectively.

Figure 29 - IPC for ALF (1080p)

Figure 30 - IPC for ALF (1600p)

Figure 31 - IPC for ALF (2160p)

### Summary of IPC results of filter steps

IPC results of the three in-loop filters show clearly the advantage of performing the filters in the same CTB loop iteration for WPP. Because each filter kernel reuses the pixel data produced of the previous kernel, this data remains in the private caches for WPP.

For Tiles, however, each filter is implemented using a separate pass. This approach has low cache locality as the entire picture must be processed completely before the pixels can be reused. Because the data cannot remain in the private caches more shared cache and off-chip requests are required, leading to overall reduced performance.

### Effect on memory bandwidth

We have also estimated the required external memory bandwidth. Using hardware performance counters we measured the number of Last-level cache (LLC) misses per kilo instruction for the entire application, depicted in Figure 33 and Figure 34 for 1080p, 1600p and 2160p respectively. Tiles have 50%, 62%, and 85% higher LLC misses than WPP for 1080p, 1600p and 2160p respectively.

This is caused by the additional full picture filter passes for Tiles in our implementation. The number of LLC misses directly translates in to memory bandwidth and when cores are used in more than one socket it also includes the inter-socket coherence traffic.

By further interpreting the curves below the performance difference can be derived for the two presented implementations of WPP and Tiles. The memory bandwidth performance is mainly caused by the different filter implantations. Different implementations have been selected due to feasibility of integrating them into the HM software.

Figure 32 - Last Level Cache Misses (1080p)

Figure 33 - Last Level Cache Misses (1600p)

Figure 34 - Last Level Cache Misses (2160p)

# Absolute performance

Table 2 shows the performance, in terms of frames per second, of the two parallelization approaches for different QP values for 1, 6 and 13 cores. 1080p50 can be achieved with 6 threads for QPs higher than 27. For 1600p and 2160p additional performance improvements, apart from high-level parallelization, are required to achieve real-time decoding.  It should be noted that additional optimizations, like SIMD vectorization, can be applied to improve the single threaded performance.

Table 2 - Performance in frames per second for different QP values

| Resolution | Cores | QP22 |  |  | QP27 |  |  | QP32 |  |  | QP37 |  |  | AVG |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | WPP | Tiles |  | WPP | Tiles |  | WPP | Tiles |  | WPP | Tiles |  | WPP | Tiles |  |
| 1080p | 1 | 8.8 | 8.7 |  | 11.5 | 11.4 |  | 13.7 | 13.8 |  | 15.9 | 16.0 |  | 12.5 | 12.5 |  |
| 1080p | 6 | 40.2 | 42.9 |  | 50.8 | 55.3 |  | 60.3 | 65.7 |  | 68.2 | 75.1 |  | 54.9 | 59.7 |  |
| 1080p | 12 | 50.9 | 57.6 |  | 64.2 | 70.7 |  | 75.0 | 81.5 |  | 84.9 | 90.6 |  | 68.7 | 75.1 |  |
| 1600p | 1 | 4.0 | 4.0 |  | 4.9 | 4.8 |  | 5.8 | 5.7 |  | 7.0 | 6.9 |  | 5.4 | 5.3 |  |
| 1600p | 6 | 19.8 | 20.6 |  | 23.8 | 24.6 |  | 27.6 | 29.1 |  | 33.0 | 34.9 |  | 26.0 | 27.3 |  |
| 1600p | 12 | 29.9 | 31.6 |  | 35.6 | 37.2 |  | 41.7 | 43.1 |  | 48.7 | 50.4 |  | 39.0 | 40.6 |  |
| 2160p | 1 | 1.5 | 1.5 |  | 2.1 | 2.1 |  | 2.5 | 2.4 |  | 2.8 | 2.7 |  | 2.2 | 2.2 |  |
| 2160p | 6 | 8.1 | 8.5 |  | 11.2 | 11.3 |  | 13.1 | 13.1 |  | 14.6 | 14.6 |  | 11.8 | 11.9 |  |
| 2160p | 12 | 13.2 | 14.8 |  | 17.9 | 19.2 |  | 21.1 | 22.0 |  | 23.3 | 24.2 |  | 18.8 | 20.1 |  |

# Issues and Remarks

- In HM software a lot of memory is used to hold the transform coefficients and IPCM related buffers. These buffers are allocated for all the CTBs in all the reference pictures (36 kB per CTB). Only one set of buffers is needed per thread, however. For our experiments we have included this optimization to give more meaningful results as otherwise many additional memory accesses and cache misses would needlessly take place.

- Some remarks have been made that Tiles require less line buffers than WPP. We agree with this when comparing a single-threaded Tiles decoder to a single-threaded WPP decoder. However, multiple WPP decoders do not require any additional line buffers as the single line buffer set can be shared by any number of WPP decoders. Because of the wavefront order WPP decoders will always use a different part of the line buffer. For each Tiles decoder, however, separate line buffers need to be allocated.

- A direct side-by-side comparison of memory bandwidth was not possible from our results due to differences in implementations of the two decoder types. It should be remarked that the presented WPP implementation includes all decoding steps in a single decoder loop. While, due to a lack of feasibility, the implementation of Tiles in the HM software did not include all theoretically possible optimizations.

- Tiles can have many configurations, including complicated combinations with slices, and the amount of parallelism is not fixed, except some lower limit that can be enforced in a Level. The parallelism in WPP can be fixed to one substream per row and, therefore, scales naturally with resolution. This ensures that codec implementations have sufficient parallelism.

# Conclusion

We have shown implementations of a parallel WPP and Tiles decoder based on the HM software. Experiments conducted on a parallel system with 12 processors show that Tiles and WPP have a similar parallel scalability. Results of the implementation have been presented. Finally, remarks have been made dealing with the comparison of implementing the Tiles and WPP approaches.

# References

- HM 4.1 Software, http://hevc.kw.bbc.co.uk/trac/browser/tags/HM-4.1

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, Torino, December. 2011.

- SVT High Definition Multiformat Test Set. Sveriges Television, 2006. ftp://vqeg.its.bldrdoc.gov/HDTV/SVT_MultiFormat/SVT_MultiFormat_v10.pdf

- K. Misra, J. Zhao, and A. Segall, “Lightweight slicing for entropy coding,” JCTVC-D070, Jan. 2011

- F. Henry and S. Pateux, “Wavefront Parallel Processing,” JCTVC-E196, March 2011.

- A. Fuldseth, M. Horowitz, S. Xu, and M. Zhou, “Tiles,” JCTVC-E408, March 2011.

# Patent rights declaration(s)

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e. V. and Technische Universität Berlin may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).