| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-Ixxxx |
| --- | --- |

| Title: | AHG14: Color format extension for HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kazuo Sugimoto Akira Minezawa Shun-ichi Sekiguchi Information Technology R&D Center Mitsubishi Electric Corporation 5-1-1, Ofuna, Kamakura, JAPAN | Tel: Email: | +81-467-41-2463 Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

# Abstract

This contribution proposes support of color format extension for HEVC standard. The main point of this proposal is to limit modifications from the current specification as small as possible support 4:2:2 and 4:4:4 format. Modified version of HM6.0 software to support 4:2:2 and 4:4:4 format is provided, however, the implementation has not been completed.

# Introduction

The HEVC standard has been focusing on its application to new visual services in near-future [1]. Although such target should be important, it is still clear that 4:2:2 format contents would be used in broadcasting and contents production area. 4:4:4 format is expected to be important for future video codec, especially for high end or professional equipments.

In this contribution, we propose to start discussion on how to support 4:2:2 and 4:4:4 format in HEVC in a reasonable way. The proposed approach is to introduce modifications to block-based coding tools as small as possible.

# 4:2:2 support in HEVC

The proposed modifications to the current specification are listed:

- When chroma_format_idc is equal to 2 which indicates 4:2:2 format, the following modifications are applied.

  - TU height is always 2 times of TU for chroma signals, and other shape for TU chroma is prohibited.

  - No syntax change except the number of coefficients within chroma TU.

  - Only tools supported in main profile are considered.

  - Currently, deblocking filter and SAO are not supported.

  - Intra prediction is simply extended to support Nx2N size for chroma signals.

  - Coefficient scanning is the same way as in CD, but the length of scanning is 2 times longer in height.

  - For the vertical MC interpolation filter for chroma, 8 tap filter used for luma is applied.

  - for chroma coefficient coding in 4:2:2 format, context index is vertically repeated 2 times reusing context index for 4:2:0 format as shown in Figure 1 as an example.

Figure 1. context index of 4x4 and 8x8 significance_coeff_flag for chroma coefficient in 4:2:2 format

# 4:4:4 support in HEVC

The proposed modifications to the current specification are listed:

- When chroma_format_idc is equal to 3 which indicates 4:4:4 format, the following modifications are applied.

  - All processing, size and syntax except intra prediction mode for chroma signal are identical to those of luma signal.

# Current implementation status

Currently, the implementation work has not been completed. With the attached source code, Enc/Dec mismatch is still observed.

# Conclusion

Modification of the current specification to support 4:2:2 and 4:4:4 format in HEVC is presented. We propose to study further on chroma format extension for HEVC standard through the adhoc group activity by completing the implementation based on the provided software as a starting point.

# Patent rights declaration(s)

Mitsubishi Electric Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).