| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0524 |
| --- | --- |

| Title: | Hook for scalable extensions: video parameter set |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela, Dmytro Rusanovskyy Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 miska.hannuksela@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

INTERNATIONAL ORGANISATION FOR STANDARDISATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC1/SC29/WG11

CODING OF MOVING PICTURES AND AUDIO

ISO/IEC JTC1/SC29/WG11 MPEG2012/M24896

May 2012, Geneva, Switzerland

| Source | Nokia Corporation |
| --- | --- |
| Status | Input contribution |
| Title | 3DV-HTM high-level syntax: video and sequence parameter set design |
| Author | Miska M. Hannuksela, Dmytro Rusanovskyy |

_____________________________

- Abstract

This contribution has been submitted to both JCT-VC and MPEG 3DV, because it proposes generic scalable extension hooks to the HEVC specification and a 3DV extension of using these hooks to the 3DV-HTM.

The contribution uses two definitions: a component sequence and a component picture. A component sequence can be for example a texture view, a depth view, or an enhancement layer of spatial/quality scalability. Bitstreams compliant with the current non-scalable HEVC design have only one component sequence. An access unit consists of one or more component pictures. In the current HEVC specification a component picture is the coded picture of an access unit, and in the future scalable extensions it would be for example a view component, a depth map, or a layer representation.

The contribution proposes a video parameter set (VPS), which serves as the highest level parameter set on top of sequence parameter sets. In the current HEVC specification, the VPS is proposed to contain:

- An identifier vps_id

- Profile and level information for bitstream subsets defined for each component sequence.

- Level information for temporal subsets of component sequences.

No profile and level information is proposed to be included in the sequence parameter set.

The slice header of IDR and CRA pictures is proposed to include an identifier (ref_vps_id) of the active VPS of the coded video sequence.

Furthermore, in scalable extensions of HECVC, reserved_one_5bits in the nal_unit syntax structure is re-specified to be called cpd_id_plus1, indicating an identifier for a component sequence/picture.

In scalable extensions of HEVC, the VPS is proposed to be extended to contain:

- The dependencies between component sequences.

- The mapping of component sequence/picture identifiers to specific scalability properties (e.g. dependency_id, quality_id, view order index).

Moroever, the semantics and use of depth and texture view component types as component pictures are proposed into the 3DV extension of HEVC.

- Introduction

One lesson learnt from the H.264/AVC extensions is that sequence parameter sets turned out to be activated per layer or per view, while they still contain information documenting properties of the entire bitstream and the cross-layer or cross-view properties, such as view dependencies. While this somehow works, it might not be the most elegant way to present structured information and may also be inefficient as analyzed recently for the 3DV extension of H.264/AVC in MPEG M24892.

JCTVC-H0388 presented the concept of a video parameter set (VPS). Generally we agree on the principle of having a higher-level parameter set on top of sequence parameter sets. This document presents our thinking on the design of the VPS and implications on other syntax structures (namely sequence parameter set and slice header).

Section 2 contains the proposed changes (i.e. scalability hooks) to HEVC CD. Section 2 is proposed to be handled in JCT-VC, while Sections 3 and 4 are merely for information in JCT-VC.

Section 3 contains the proposed NAL unit header changes and extension of the VPS that are proposed for all scalable extensions of HEVC. Section 4 contains the proposed semantics of certain syntax elements of VPS to be used for 3DV. Sections 3 and 4 are proposed to be handled in MPEG 3DV, while Section 2 provides the necessary background but should rather be handled in JCT-VC.

- Proposed Changes to HEVC CD

  - General

A coded video sequence contains one or more component sequences.  A component sequence can be for example a texture view, a depth view, or an enhancement layer of spatial/quality scalability. Each component sequence may refer to a separate sequence parameter set, and several component sequences may refer to the same sequence parameter set.  Each component sequence is uniquely identified by variable CPId, which is derived from the 5 reserved bits in the second byte of the NAL unit header.  Temporal subsets of the coded video sequence are not considered to be component sequences; instead temporal_id is regarded as an orthogonal property. Bitstreams compliant with the current non-scalable HEVC design contain only one component sequence having CPId equal to 0.

An access unit consists of one or more component pictures. In the current HEVC specification a component picture is the coded picture of an access unit, and in the future scalable extensions it would be for example a view component, a depth map, or a layer representation.

  - Video Parameter Set

VPS serves as index of the scalability properties of the coded video sequence. It is proposed to contain

- An identifier vps_id

- Profile and level information for bitstream subsets defined for each component sequence.

- Level information for temporal subsets of component sequences (assuming that the profile information stays unchanged if temporal subsets are used – if that is not the case, also the profile could be provided for each temporal subset).

Zero or one VPS shall be active.  A VPS is activated by the first VCL NAL unit of a coded video sequence or bitstream and remains active until the next VPS is activated or until the end of the bitstream, whichever is earlier in decoding order. The VPS must be available before the NAL unit that activates the VPS.  Once activated, the VPS applies to the entire coded video sequence.  A referred VPS ID ref_vps_id is proposed to be included in coded slices of IDR and CRA pictures.

The proposed VPS syntax is the following:

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| max_component_sequences_minus1 | u(5) |
| max_temporal_layers_minus1 | u(3) |
| for( i = 0; i <= max_component_sequences_minus1; i++ ) { |  |
| profile_struct( ) |  |
| for( j = 0; j < max_temporal_layers_minus1; j++ ) |  |
| level_idc[ i ][ j ] | u(8) |
| } |  |
| vps_id | ue(v) |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

max_component_sequences_minus1 + 1 specifies the number of component sequences that may be present in the coded video sequence. max_component_sequences_minus1 equal to 31 is reserved for future use by ITU-T | ISO/IEC.

max_temporal_layers_minus1 specifies the maximum temporal_id value that may be present in the coded video sequence.

level_idc[ i ][ j ] specifies the level for which the bitstream subset resulting from sub-bitstream extraction process with input temporal_id equal to j conforms to.

vps_id identifies a video parameter set. The value of vps_id shall be in the range of 0 to X, inclusive.

vps_extension_flag equal to 0 specifies that no vps_extension_data_flag syntax elements are present in the video parameter set RBSP syntax structure. vps_extension_flag shall be equal to 0 in bitstreams conforming to this Recommendation | International Standard. The value of 1 for vps_extension_flag is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all data that follow the value 1 for vps_extension_flag in a video parameter set NAL unit.

vps_extension_data_flag may have any value. It shall not affect the conformance to profiles specified in this Recommendation | International Standard.

The profile_struct( ) syntax structure can have the following syntax (as outlined in the adopted contribution JCTVC-E429). Alternatively the conventional 3-byte profile-level signaling, as done HEVC CD can be used.

| profile_struct( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| profileCnt = 0 |  |
| do { |  |
| for( i = 0; i < 7; i++ ) |  |
| profile_compatibility_idc[ profileCnt++ ] | u(1) |
| last_byte_flag | u(1) |
| } while( !last_byte_flag ) |  |
| } |  |

  - Sequence parameter set

- The profile and level information is proposed to be removed from the SPS, as they are now proposed to be carried in VPS:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| reserved_zero_8bits /* equal to 0 */ | u(8) |
| level_idc | u(8) |
| … |  |

  - Slice header

A referred VPS ID ref_vps_id of the active VPS is proposed to be included in coded slices of IDR and CRA pictures as follows:

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( !entropy_slice_flag ) { |  |
| … |  |
| if( IdrPicFlag ¦ ¦ CraPicFlag ) |  |
| ref_vps_id | ue(v) |
| … |  |

- Proposed Generic Changes to HEVC Scalable Extensions

  - Video Parameter Set

The VPS is proposed to be extended to contain:

- The dependencies between component sequences. While sub-bitstream extraction based on CPId can be done, there might be more than one scalability dimension present in the bitstream, such as view order index and depth/texture in 3D video coding. Consequently, dependency relations between component sequences are included in the proposed VPS.

- The mapping of CPId to specific scalability properties (e.g. dependency_id, quality_id, view order index). As the types of scalability and the syntax elements used to represent them are not known at the moment and new types of scalability may be introduced in further extensions, the proposed syntax enables parsing of VPS even if the scalability types were unknown for the decoder. The decoder might be able to decode a subset of the bitstream containing those scalability types that it is aware of.

The extensions are enabled by setting the extension_flag equal to 1. The following syntax is therefore added at the end of the VPS syntax structure:

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| video_parameter_set_id | ue(v) |
| vps_extension1_flag | u(1) |
| if( vps_extension1_flag ) { |  |
| for( i = 1; i <= max_component_sequences_minus1; i++ ) { |  |
| num_ref_component_seq[ i ] | ue(v) |
| for( j = 0; j < num_ref_component_seq; j++ ) |  |
| ref_component_seq_id[ i ][ j ] | u(5) |
| } |  |
| num_component_seq_types | ue(v) |
| for( i = 1; i <= num_component_seq_types; i++ ) { |  |
| component_sequence_type[ i ] | ue(v) |
| component_sequence_property_len[ i ] | ue(v) |
| len[ i ] = component_sequence_property_len[ i ] |  |
| } |  |
| for( i = 1; i <= max_component_sequences_minus1; i++ ) { |  |
| component_sequence_type_idx[ i ] | ue(v) |
| tp = component_sequence_type_idx[ i ] |  |
| component_sequence_property[ i ] | u( len[ tp ] ) |
| } |  |
| } |  |
| vps_extension2_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

num_ref_component_seq[ i ] specifies the number of component sequences that the component sequence with CPId equal to i depends on.

ref_component_seq_id[ i ][ j ] specifies the vps_id values of the component sequences that the component sequence with CPId equal to i depends on.

component_sequence_type[ i ] specifies the type of the component sequence with type index equal to i. component_sequence_type[ 0 ] is inferred to indicate HEVC base component sequence.

component_sequence_property_len[ i ] specifies the size in bits of component_sequence_property[ ] syntax element which is preceded by component_sequence_type_idx[ ] syntax element having value equal to i.

component_sequence_type_idx[ i ] specifies the type index for the component sequence with CPId equal to i. The component sequence with CPId equal to i is of type component_sequence_type[ component_sequence_type_idx [ i ] ].

component_sequence_property[ i ] specifies the value or values characterizing the component sequence with CPId equal to i. The semantics of component_sequence_property[ i ] are specified according to component_sequence_type[ component_sequence_type_idx [ i ] ].

  - NAL Unit Header

reserved_one_5bits in the nal_unit syntax structure is re-specified to be called cpd_id_plus1. The value of CPId is specified to be equal to cpd_id_plus1 – 1.

Component pictures shall appear in ascending order of CPId within the access unit.

- Proposed Changes to HEVC 3D Extension with Depth

  - Component Sequence Types for Depth and Texture View Components

A specific value component_sequence_type[ ] is specified to indicate a texture view component. Another specific value of component_sequence_type[ ] is specified to indicate a depth view component.

component_sequence_property_len[ ] for depth and texture view components shall be the number of bits that covers the maximum number of views in a bitstream, e.g. 10 bits as in MVC.

component_sequence_property[ ] specifies the view order index of the depth view component for a component sequence of a depth view or respectively the view order index of the texture view component for a component sequence of a texture view.

# Patent rights declaration(s)

Nokia Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).