| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0528 |
| --- | --- |

| Title: | Cross-check of JCTVC-I0265 – Chroma QP extension and signalling enhancement |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dzung Hoang 3960 Freedom Circle Santa Clara, CA 95054 USA | Tel: Email: | +1-408-529-2007 dzung.hoang@zenverge.com |
| Source: | Zenverge, Inc. |  |  |

_____________________________

# Abstract

This contribution is an independent verification of the methods and results described in JCTVC-I0265 on extending the range of chroma QP and related means to signal the extension.

# Introduction

JCTVC-I0265 proposes to extend the range of chroma QP from [0, 39] to [0, 51]. Two methods are proposed, namely Method A and Method B. Method A removes the chroma QP mapping table g_aucChromaScale[52] from HM 6.0 and extends the chroma QP range to [0, 51]. Method B keeps the chroma QP mapping table but performs clipping to [0, 51] after adding chroma QP offsets transmitted in the PPS and slice header.

The authors of JCTVC-I0265 also propose an encoder-side strategy with Method A that attempts to emulate the HM 5.0 chroma QP mapping table by setting the chroma QP offsets appropriately. The motivation is to show that the RD behaviour of the codec is not changed with this method.

# Review of Methods and Source Code

After review of JCTVC-I0265, we find the proposal to be sound and justified. Examination of the source code provided by the proponents shows that the implementation matches the description provided in the text.

# Experimental Results

Results are provided for the following two test cases.

- Method A

- Method A with HM chroma QP mapping table emulation

JCTVC-I0265 also provides results for a third test case, which is Test 2 above with additional slice-level coding of chroma QP offset. We did not have time to run the simulations for this additional test. However, after careful inspection of the code we are confident that the only difference is the additional overhead bits to signal chroma QP offset in the slice header. Given that a slice-level chroma QP offset is always 0 in the common test conditions, there should be no effects on the decoded pixels.

Our results for Test 1 match with JCTVC-I0265. We are about half-way through Test 2 and the completed runs match the results in JCTVC-I0265. This document will be updated when Test 2 has completed.

# Recommendation

After studying JCTVC-I0265, our recommendation is to adopt Method A. We are neutral with regards to the addition of chroma QP offset in the slice header.

# Patent rights declaration(s)

Zenverge does not have any current or pending patent rights relating to the technology described in this contribution.