| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0529 |
| --- | --- |

| Title: | AHG13: Proposed bugfix for tickets 410 and 470 related to lossless coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gordon Clare (Orange Labs) Félix Henry (Orange Labs) | Tel: Email: | +33 2 99 12 45 58 gordon.clare@orange-ftgroup.com felix.henry@orange-ftgroup.com |
| Source: | Orange Labs |  |  |

_____________________________

# Abstract

This contribution proposes that lossless coding is modified in the following way: a slice level flag indicates whether lossless coding is enabled in the slice, and a CU level flag is used (only in slices were lossless coding is enabled) to indicate that the current CU is lossless. It is asserted that the proposed modification solves bug #470 and that it also provides a better fix to bug #410 compared to the current bugfix since QP derivation process is avoided.

# Current design

In the current design of HEVC, lossless coding of a CU is indicated by the combination of a sequence level flag qpprime_y_zero_transquant_bypass_flag and QP’Y. A CU is encoded losslessly when level flag qpprime_y_zero_transquant_bypass_flag is equal to one and QP’Y is equal to zero. When lossless coding is active, the scaling and transform process and the in-loop filter are bypassed [1].

A first bug was identified (bug #410) related both to lossless coding and Sign Bit Hiding [2]. Sign Bit Hiding consists in avoiding the explicit transmission of a sign bits of the residue. Instead, the sign bits are carried by the parity of their respective coefficient group. Sign Bit Hiding is inherently lossy, because it may require that quantization levels are changed in order to obtain the desired parity. Therefore, it is not possible to use both lossless coding and Sign Bit Hiding in the same CU. A bugfix has been initially proposed [3] were Sign Bit Hidind is simply avoid in a CU were flag qpprime_y_zero_transquant_bypass_flag is equal to one and QP’Y is equal to zero. However, this approach requires that QP derivation is performed during the parsing process, something that was not required until now in the HEVC decoder.

A second bug was identified (bug #470) related to lossless coding. Lossless coding requires that QP’Y is set to zero. This is achieved by modifying cu_qp_delta syntax element. However, cu_qp_delta is only transmitted for CUs that have a non-zero residue. Therefore, if a block has a non zero residue and the predicted QP is not zero, then it becomes impossible to modify the QP in order to achieve lossless coding. As a consequence, the block will be filtered and will not be lossless. A possible workaround is to force the encoder to use another coding mode that has a non zero residue, but this is not always possible, as some configurations may produce a zero residue for all modes (Intra only or encoder-based restrictions on the modes). Similarly, if a block has a predicted QP of zero (because previously encoded blocks are losslessly encoded) and a zero residue, it is not possible to specify that the encoder wants this block to be lossy (and hence, filtered).

# Proposed solution

It is proposed that a slice level flag indicates whether lossless coding is enabled in the slice or not. Then, if and only if this flag is on, a CU level flag indicates whether the block is losslessly encoded or not. In case the CU level flag is on, the scaling and transform process and the in-loop filter are bypassed.

# Advantages

The proposed modification has the following advantages:

- Bug #410 can easily be fixed by making Sign Bit Hiding dependent on the CU level flag, thus making QP derivation not required

- Bug #470 is fixed because lossless coding can be activated / deactivated regardless of the presence of a residue

Also, making lossless coding independent from QP has the advantage that QP level can be maintained throughout a lossless section of the image, so that a protected lossless area does not disrupt the background QP level.

# Bibliography

[1] JCTVC-H0530, AHG19: A lossless coding solution for HEVC ,W. Gao, M. Jiang, H. Yu (Huawei), M. Zhou (TI), San Jose, February 2012.

[2] JCTVC-H0481, Multiple Sign Bits Hiding, X. Yu, J. Wang, D. He, G. Martin-Cocher, S. Campbell

[3] JCTVC-I0464, AHG13: Suggested Bug Fix for Lossless Coding in HM6.1 and HEVC text specification (Ticket 410), W. Gao, M. Jiang, H. Yu (Huawei), Geneva, April 2012.

# Patent rights declaration(s)

Orange Labs may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix: Text proposal

#### Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| reserved_zero_8bits /* equal to 0 */ | u(8) |
| level_idc | u(8) |
| seq_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| max_temporal_layers_minus1 | u(3) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| pic_cropping_flag | u(1) |
| if( pic_cropping_flag ) { |  |
| pic_crop_left_offset | ue(v) |
| pic_crop_right_offset | ue(v) |
| pic_crop_top_offset | ue(v) |
| pic_crop_bottom_offset | ue(v) |
| } |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| pcm_enabled_flag | u(1) |
| if( pcm_enabled_flag ) { |  |
| pcm_bit_depth_luma_minus1 | u(4) |
| pcm_bit_depth_chroma_minus1 | u(4) |
| } |  |
| qpprime_y_zero_transquant_bypass_flag | u(1) |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| for( i = 0; i <= max_temporal_layers_minus1; i++ ) { |  |
| max_dec_pic_buffering[ i ] | ue(v) |
| num_reorder_pics[ i ] | ue(v) |
| max_latency_increase[ i ] | ue(v) |
| } |  |
| restricted_ref_pic_lists_flag | u(1) |
| if( restricted_ref_pic_lists_flag ) |  |
| lists_modification_present_flag | u(1) |
| log2_min_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| if( pcm_enabled_flag ) { |  |
| log2_min_pcm_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_pcm_coding_block_size | ue(v) |
| } |  |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| scaling_list_enable_flag |  |
| chroma_pred_from_luma_enabled_flag | u(1) |
| deblocking_filter_in_aps_enabled_flag | u(1) |
| seq_loop_filter_across_slices_enabled_flag | u(1) |
| asymmetric_motion_partitions_enabled_flag | u(1) |
| non_square_quadtree_enabled_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| adaptive_loop_filter_enabled_flag | u(1) |
| if( adaptive_loop_filter_enabled_flag ) |  |
| alf_coef_in_slice_flag | u(1) |
| if( pcm_enabled_flag ) |  |
| pcm_loop_filter_disable_flag | u(1) |
| temporal_id_nesting_flag | u(1) |
| if( log2_min_coding_block_size_minus3 = = 0 ) |  |
| inter_4x4_enabled_flag | u(1) |
| num_short_term_ref_pic_sets | ue(v) |
| for( i = 0; i < num_short_term_ref_pic_sets; i++) |  |
| short_term_ref_pic_set( i ) |  |
| long_term_ref_pics_present_flag | u(1) |
| tiles_or_entropy_coding_sync_idc | u(2) |
| if( tiles_or_entropy_coding_sync_idc = = 1 ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height[ i ] | ue(v) |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } |  |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

### Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| if( first_slice_in_pic_flag = = 0 ) |  |
| slice_address | u(v) |
| slice_type | ue(v) |
| entropy_slice_flag | u(1) |
| if( !entropy_slice_flag ) { |  |
| pic_parameter_set_id | ue(v) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| if( IdrPicFlag ) { |  |
| idr_pic_id | ue(v) |
| no_output_of_prior_pics_flag | u(1) |
| } else { |  |
| pic_order_cnt_lsb | u(v) |
| short_term_ref_pic_set_sps_flag | u(1) |
| if( !short_term_ref_pic_set_sps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else |  |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_pics; i++ ) { |  |
| delta_poc_lsb_lt[ i ] | ue(v) |
| delta_poc_msb_present_flag[ i ] | u(1) |
| if( delta_poc_msb_present_flag[ i ] ) |  |
| delta_poc_msb_cycle_lt_minus1[ i ] | ue(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| slice_sao_interleaving_flag | u(1) |
| slice_sample_adaptive_offset_flag | u(1) |
| if( slice_sao_interleaving_flag && slice_sample_adaptive_offset_flag ) { |  |
| sao_cb_enable_flag | u(1) |
| sao_cr_enable_flag | u(1) |
| } |  |
| } |  |
| if( scaling_list_enable_flag ¦ ¦ deblocking_filter_in_aps_enabled_flag ¦ ¦ ( sample_adaptive_offset_enabled_flag && !slice_sao_interleaving_flag ) ¦ ¦ adaptive_loop_filter_enabled_flag ) |  |
| aps_id | ue(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| if( lists_modification_present_flag ) { |  |
| ref_pic_list_modification( ) |  |
| ref_pic_list_combination( ) |  |
| } |  |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| } |  |
| if( cabac_init_present_flag && slice_type != I ) |  |
| cabac_init_flag | u(1) |
| if( !entropy_slice_flag ) { |  |
| slice_qp_delta | se(v) |
| if( deblocking_filter_control_present_flag ) { |  |
| if( deblocking_filter_in_aps_enabled_flag ) |  |
| inherit_dbl_params_from_aps_flag | u(1) |
| if( !inherit_dbl_params_from_aps_flag ) { |  |
| disable_deblocking_filter_flag | u(1) |
| if( !disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( slice_type != I && ((collocated_from_l0_flag && num_ref_idx_l0_active_minus1 > 0) ¦ ¦ (!collocated_from_l0_flag && num_ref_idx_l1_active_minus1 > 0) ) |  |
| collocated_ref_idx | ue(v) |
| if( ( weighted_pred_flag && slice_type = = P) ¦ ¦ ( weighted_bipred_idc = = 1 && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| } |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) |  |
| five_minus_max_num_merge_cand | ue(v) |
| if( adaptive_loop_filter_enabled_flag ) { |  |
| slice_adaptive_loop_filter_flag | u(1) |
| if( slice_adaptive_loop_filter_flag && alf_coef_in_slice_flag ) |  |
| alf_param( ) |  |
| if( slice_adaptive_loop_filter_flag && !alf_coef_in_slice_flag ) |  |
| alf_cu_control_param( ) |  |
| transquant_bypass_enable_flag | u(1) |
| } |  |

### Coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| CurrCbAddrTS = MinCbAddrZS[ x0 >> Log2MinCbSize ][ y0 >> Log2MinCbSize ] |  |
| if( transquant_bypass_enable_flag ) { |  |
| cu_transquant_bypass_flag | u(1) |
| } |  |
| if( slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] | ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| else if( slice_type != I ¦ ¦ log2CbSize = = Log2MinCbSize ) { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( PredMode != MODE_INTRA ¦ ¦ log2CbSize = = Log2MinCbSize ) |  |
| part_mode | ae(v) |
| x1 = x0 + ( ( 1 << log2CbSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CbSize ) >> 1 ) |  |
| x2 = x1 − ( ( 1 << log2CbSize ) >> 2 ) |  |
| y2 = y1 − ( ( 1 << log2CbSize ) >> 2 ) |  |
| x3 = x1 + ( ( 1 << log2CbSize ) >> 2 ) |  |
| y3 = y1 + ( ( 1 << log2CbSize ) >> 2 ) |  |
| if( PartMode = = PART_2Nx2N ) |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| else if( PartMode = = PART_2NxN ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x0, y1 , log2CbSize ) |  |
| } else if( PartMode = = PART_Nx2N ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x1, y0 , log2CbSize ) |  |
| } else if( PartMode = = PART_2NxnU ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x0, y2 , log2CbSize ) |  |
| } else if( PartMode = = PART_2NxnD ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x0, y3 , log2CbSize ) |  |
| } else if( PartMode = = PART_nLx2N ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x2, y0 , log2CbSize ) |  |
| } else if( PartMode = = PART_nRx2N ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x3, y0 , log2CbSize ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x1, y0 , log2CbSize ) |  |
| prediction_unit( x0, y1 , log2CbSize ) |  |
| prediction_unit( x1, y1 , log2CbSize ) |  |
| } |  |
| if( !pcm_flag ) |  |
| transform_tree( x0, y0, x0, y0, log2CbSize, log2CbSize, log2CbSize, 0, 0 ) |  |
| } |  |
| } |  |

### Residual coding syntax

| residual_coding( x0, y0, log2TrafoWidth, log2TrafoHeight, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| last_significant_coeff_x_prefix | ae(v) |
| last_significant_coeff_y_prefix | ae(v) |
| if( last_significant_coeff_x_prefix > 3 ) |  |
| last_significant_coeff_x_suffix | ae(v) |
| if( last_significant_coeff_y_prefix > 3 ) |  |
| last_significant_coeff_y_suffix | ae(v) |
| numCoeff = 0 |  |
| do { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ numCoeff ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ numCoeff ][ 1 ] |  |
| numCoeff++ |  |
| } while( ( xC != LastSignificantCoeffX ) ¦ ¦ ( yC != LastSignificantCoeffY ) ) |  |
| numLastSubset = (numCoeff − 1) >> 4 |  |
| for( i = numLastSubset; i >= 0; i− − ) { |  |
| offset = i << 4 |  |
| if( scanIdx = = 1 && log2TrafoWidth = = 3 && log2TrafoHeight = = 3 ) { |  |
| xCG = 0 |  |
| yCG = i |  |
| } else if( scanIdx = = 2 && log2TrafoWidth = = 3 && log2TrafoHeight = = 3 ) { |  |
| xCG = i |  |
| yCG = 0 |  |
| } else { |  |
| xCG = ScanOrder[ log2TrafoWidth − 2 ][ log2TrafoHeight − 2 ][ scanIdx ][ i ][ 0 ] |  |
| yCG = ScanOrder[ log2TrafoWidth − 2 ][ log2TrafoHeight − 2 ][ scanIdx ][ i ][ 1 ] |  |
| } |  |
| implicitNonZeroCoeff = 0 |  |
| if( (i < numLastSubset) && (i > 0) ) { |  |
| significant_coeff_group_flag[ xCG ][ yCG ] | ae(v) |
| implicitNonZeroCoeff = 1 |  |
| } |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( (n + offset) < (numCoeff − 1) && significant_coeff_group_flag[ xCG ][ yCG ] && ( n > 0 ¦ ¦ implicitNonZeroCoeff = = 0 ) ) { |  |
| significant_coeff_flag[ xC ][ yC ] | ae(v) |
| if( significant_coeff_flag[ xC ][ yC ] = = 1 ) |  |
| implicitNonZeroCoeff = 0 |  |
| } |  |
| } |  |
| firstNZPosInCG = 16 |  |
| lastNZPosInCG = −1 |  |
| numSigCoeff = 0 |  |
| firstGreater1CoeffIdx = −1 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| if( numSigCoeff < 8 ) { |  |
| coeff_abs_level_greater1_flag[ n ] | ae(v) |
| numSigCoeff++ |  |
| if( coeff_abs_level_greater1_flag[ n ] && firstGreater1CoeffIdx = = −1 ) |  |
| firstGreater1CoeffIdx = n |  |
| } |  |
| if( lastNZPosInCG = = −1) |  |
| lastNZPosInCG = n |  |
| firstNZPosInCG = n |  |
| } |  |
| } |  |
| signHidden = ( lastNZPosInCG – firstNZPosInCG >= sign_hiding_threshold && !cu_transquant_bypass_flag) ? 1 : 0 |  |
| if( firstGreater1CoeffIdx != −1 ) |  |
| coeff_abs_level_greater2_flag[ firstGreater1CoeffIdx] | ae(v) |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] && (!sign_data_hiding_flag ¦ ¦ !signHidden ¦ ¦ n != firstNZPosInCG) ) |  |
| coeff_sign_flag[ n ] | ae(v) |
| } |  |
| numSigCoeff = 0 |  |
| sumAbs = 0 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| baseLevel = 1 + coeff_abs_level_greater1_flag[ n ] + coeff_abs_level_greater2_flag[ n ] |  |
| if( baseLevel = = ( ( numSigCoeff < 8 ) ? ( (n = = firstGreater1CoeffIdx) ? 3 : 2 ) : 1 ) ) |  |
| coeff_abs_level_remaining[ n ] | ae(v) |
| transCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( coeff_abs_level_remaining[ n ] + baseLevel ) * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| if( sign_data_hiding_flag && signHidden ) { |  |
| sumAbs += ( coeff_abs_level_remaining[ n ] + baseLevel ) |  |
| if( n = = firstNZPosInCG && (sumAbs%2 = = 1) ) |  |
| transCoeffLevel[x0][y0][cIdx][xC][yC] = − transCoeffLevel[x0][y0][cIdx][xC][yC] |  |
| } |  |
| numSigCoeff++ |  |
| } else |  |
| transCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = 0 |  |
| } |  |
| } |  |
| } |  |

### Semantices

transquant_bypass_enable_flag specifies whether a cu_transquant_bypass_flag syntax element is coded or not. If transquant_bypass_enable_flag is equal to 1, cu_transquant_bypass_flag is coded, otherwise it is not coded.

cu_transquant_bypass_flag equal to 1 specifies that the scaling and transform process as specified in subclause Error: Reference source not found and the in-loop filter process as specified in subclause Error: Reference source not found are bypassed. When cu_transquant_bypass_flag is not present, it is inferred to be equal to 0.