| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0536 |
| --- | --- |

| Title: | High-level syntax for 3D and scalable extensions: Signaling of layer identifiers and inter-layer decoding dependencies |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Thomas Rusert Ericsson Research 164 80 Stockholm Sweden | Tel: Email: | +46 767602660 thomas.rusert@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution and a related contribution JCTVC-I0535 have been submitted to both JCT-VC and MPEG 3DV. In the JCT-VC, they provide information on a proposed high-level syntax design for HEVC 3D and scalable extensions. The proposed design does not require changes to the HEVC base specification. It is suggested that the JCT-VC considers these contributions in the discussion on hooks for 3D and scalable extensions.

The present contribution assumes the commonly considered option of defining an abstract layer identifier layer_id_plus1 in the NAL unit header of HEVC 3D and scalability extensions, utilizing the reserved_one_5bits field in the NAL unit header of the base specification. Based on that, a method for mapping between layer_id_plus1 and concrete layer identifiers is proposed. Furthermore, a method for layer dependency signaling is proposed, which builds on the SPS prediction approach presented in JCTVC-I0535. Both layer mapping and dependency signaling are provided in SPSs associated with non-base layers. This implies extending the definition of SPS in the extension specifications, but it does not require any changes to the base specification.

# Introduction

In the context of HEVC extensions towards multiview video + depth and scalability, layered coding methods for HEVC will be developed, and an extended hierarchically organized bitstream design will be introduced. The hierarchically organized bitstream design will allow for extraction of sub-bitstreams that include subsets of the information contained in an original multi-layer bitstream. Extracted sub-bitstreams can be decoded into subsets of the video data represented by the original bitstream, for instance a subset of video and depth views in case of multiview video + depth coding, or lower spatial resolution or pixel fidelity in case of spatial or SNR scalability, respectively. Support for combining different scalability types in a single multi-layer bitstream will be present from the start, and it is desirable to make sure that the design can be efficiently extended towards further scalability dimensions or combinations of new and/or existing scalability dimensions in the future.

For the abovementioned purposes the HEVC high-level syntax needs to be extended with sufficiently flexible mechanisms to provide means for the following aspects.

- Identification of bitstream subsets based on layer identifiers in the NAL unit header.

- Mapping of abstract layer identifiers to concrete layer identifiers.

- Signaling of inter-layer decoding dependencies.

This contribution addresses the design of mechanisms for the items above, in particular items 2 and 3. It builds on a contribution to the previous meeting [2] and it is an extension to a related proposal in a separate contribution to this meeting [3].

Item 1 above, identification of bitstream subsets, will be facilitated by the use of the reserved_one_5bits field in the NAL unit header of the HEVC base specification. In layered HEVC extensions, this field will be used to signal one or several layer identifiers, thus allowing for each NAL unit to indicate a bitstream subset that the NAL unit belongs to. In this contribution, it is assumed that the most generic use of the reserved_one_5bits fields is applied, i.e. the field is interpreted as layer_id_plus1, and that NAL units associated with video and depth views have different associated values of layer_id_plus1. The proposed mechanisms are similarly applicable if a different NAL unit header design is selected.

Item 2 above denotes the mapping of an abstract layer identifier such as layer_id_plus1 to concrete layer identifiers, such as view_id, depth_flag, dependency_id, quality_id

Item 3 denotes signaling of decoding dependencies, as being relevant both for (a) the bitstream extraction process, i.e. to provision information about layers to be extracted for a certain operating point in the bitstream to be decodable, and (b) the decoding process, i.e. to provision information needed for purposes such as inter-layer reference picture handling.

Among the proposals that have been made in MPEG and the JCT-VC to address items 2 and 3 above, two alternative principles can be identified:

- Signaling of layer identifier mapping and decoding dependencies in a single data structure (SPS or VPS) for all layers.

- Signaling of layer identifier mapping and decoding dependencies distributed over separate data structures for each layer (SPS).

The mechanism proposed herein is based on principle B. Details of the proposed mechanism are provided in Section 2. Section 3 provides a discussion of the pros and cons of the proposed mechanism compared to approaches based on principle A.

# Proposed mechanism

Proposed syntax and semantics are provided further below. Since there is no draft specification text for HEVC-based 3D video coding yet, the definitions are based on the latest HEVC specification draft [4].

In the proposed mechanism, each layer (i.e. each texture and depth view in a multiview texture + depth configuration) is associated with a separate SPS. During the decoding process, each layer can have up to one active SPS. Layers are identified by a value of layer_id_plus1 signaled in the NAL unit header. Both the active SPS in a given layer and all VCL NAL units associated with that layer have the same value of layer_id_plus1.

## Mapping of layer identifiers

It is proposed that the mapping between layer_id_plus1 and the concrete layer identifiers is provided in the SPS. In the context of the HEVC-based 3D video coding test model, a mapping between layer_id_plus1 and two identifiers, view_id and view_order_idx, is needed.

The presence of the mapping is conditioned on the value of profile_idc. The SPS associated with the base video view is defined according to the HEVC base specification and thus not modified, and the values for view_id and view_order are inferred to be equal to 0. The base video view is indicated by a profile_idc value according to one or several profiles that will be defined in the HEVC core specification. In the syntax table below it is assumed that profile_idc = = 77 identifies the base video view. In SPSs associated with layers other than the base layer, view_id and view_order_idx may be explicitly signaled. If not explicitly signaled, the values of view_id and view_order_idx are inferred by means of SPS prediction as described in the following subsection.

In different extensions to HEVC, new mappings may be introduced conditioned on other values of profile_idc. For instance, if profile_idc is equal to a value that corresponds to a scalable profile, mapping between layer_id_plus1 and identifiers such as dependency_id and quality_id can be present.

## Semantics

SPSs associated with layers other than the base layer may contain layer references, as proposed in a separate contribution to this meeting [3]. The presence of such layer references in the SPS is conditioned on the value of profile_idc, similar as the mapping of layer identifiers above. In the SPS associated with the base layer, no layer references are signaled. In SPSs associated with layers other than the base layer, layer references may be signaled. In this contribution, the layer references in the SPS are provided to serve two different mechanisms:

- Indication of inter-layer decoding dependencies

- SPS prediction as proposed in [3]

With mechanism 1, by signaling a layer reference in an SPS in a given layer, decoding dependencies are indicated both with respect to the referenced layer and the layers that the referenced layer depends on. The decoding dependency information is required in the decoding process, e.g. for inter-layer reference picture handling.

In the current HEVC-based 3D video coding test model, two different types of inter-layer decoding dependencies exist:

- Inter-view decoding dependency: A video view with view_id equal to vId may be dependent on one or more video views having view_id not equal to vId. Analogously, a depth view with view_id equal to vId may be dependent on one or more video views having view_id not equal to vId.

- Texture-depth decoding dependency: A depth view with view_id equal to vId may be dependent on a video view with view_id equal to vId.

Two different types of layer references are proposed according to the two different types of decoding dependencies in the HEVC-based 3D video coding test model, “view reference” and “texture reference”. Using “view reference”, reference to a layer associated with a certain video or depth view is made. Using “texture reference”, reference from a layer associated with a depth view with view_id equal to vId to a layer that is associated with a texture view with view_id equal to vId is made.

The presence of the different reference types in the SPS is conditioned on the value of profile_idc. In SPSs associated with video views, indicated in the syntax table by a value of profile_idc not equal to 77 and not equal to “MVD”, only “view reference” may exist. In SPSs associated with depth views, indicated by profile_idc equal to “MVD” below, both “view reference” and “texture reference” may exist.

Besides indicating decoding dependencies, the layer references are utilized for inter-layer SPS prediction as introduced in [3], see mechanism 2 above. SPS prediction provides bit savings when several SPSs are used. It is thus particularly applicable when each layer is associated with a separate SPS as proposed herein. When “view reference” is used, all syntax elements except for profile_idc, level_idc, and seq_parameter_set_id are inferred from the active SPS in the referenced layer. When “texture reference” is used, SPS prediction is performed such that the syntax elements “view_id” and “view_order_idx” are inferred from the active SPS in the reference layer. Those operations result in bit savings in SPSs associated with layers other than the base layer.

Figure 1 below illustrates the use of layer references for the case of a 2-view video + depth representation.

Figure 1: Example of decoding dependencies and layer references for 2-view video + depth configuration. (a) Illustration of video and depth views and syntax element layer_id_plus1 associated with each view. The arrows indicate decoding dependencies due to inter-view prediction (horizontal arrows) and texture-depth prediction (vertical arrows). (b) Syntax elements in SPSs associated with the video and depth views in the example. Syntax elements in parentheses indicate that the values are inferred instead of being explicitly signaled. Arrows indicate prediction directions for SPS prediction according to layer reference types “view reference” and “texture reference”.

## Syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| reserved_zero_8bits /* equal to 0 */ | u(8) |
| level_idc | u(8) |
| seq_parameter_set_id | ue(v) |
| if( profile_idc ! = 77 ) { */ not base layer */ |  |
| view_ref_layer_present_flag | u(1) |
| if( view_ref_layer_present_flag = = 1 ) |  |
| delta_view_ref_layer_minus1 */ view reference */ | ue(v) |
| if( profile_idc = = MVD ) { */ depth */ |  |
| texture_ref_layer_present_flag | u(1) |
| if( texture_ref_layer_present_flag = = 1 ) |  |
| delta_texture_ref_layer_minus1 */ texture reference */ | ue(v) |
| } else { |  |
| view_id | ue(v) |
| view_order_idx | ue(v) |
| } |  |
| } |  |
| if( view_ref_layer_present_flag = = 0 ) { |  |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| max_temporal_layers_minus1 | u(3) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| pic_cropping_flag | u(1) |
| if( pic_cropping_flag ) { |  |
| pic_crop_left_offset | ue(v) |
| pic_crop_right_offset | ue(v) |
| pic_crop_top_offset | ue(v) |
| pic_crop_bottom_offset | ue(v) |
| } |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| pcm_enabled_flag | u(1) |
| if( pcm_enabled_flag ) { |  |
| pcm_bit_depth_luma_minus1 | u(4) |
| pcm_bit_depth_chroma_minus1 | u(4) |
| } |  |
| qpprime_y_zero_transquant_bypass_flag | u(1) |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| for( i = 0; i <= max_temporal_layers_minus1; i++ ) { |  |
| max_dec_pic_buffering[ i ] | ue(v) |
| num_reorder_pics[ i ] | ue(v) |
| max_latency_increase[ i ] | ue(v) |
| } |  |
| restricted_ref_pic_lists_flag | u(1) |
| if( restricted_ref_pic_lists_flag ) |  |
| lists_modification_present_flag | u(1) |
| log2_min_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| if( pcm_enabled_flag ) { |  |
| log2_min_pcm_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_pcm_coding_block_size | ue(v) |
| } |  |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| scaling_list_enable_flag |  |
| chroma_pred_from_luma_enabled_flag | u(1) |
| deblocking_filter_in_aps_enabled_flag | u(1) |
| seq_loop_filter_across_slices_enabled_flag | u(1) |
| asymmetric_motion_partitions_enabled_flag | u(1) |
| non_square_quadtree_enabled_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| adaptive_loop_filter_enabled_flag | u(1) |
| if( adaptive_loop_filter_enabled_flag ) |  |
| alf_coef_in_slice_flag | u(1) |
| if( pcm_enabled_flag ) |  |
| pcm_loop_filter_disable_flag | u(1) |
| temporal_id_nesting_flag | u(1) |
| if( log2_min_coding_block_size_minus3 = = 0 ) |  |
| inter_4x4_enabled_flag | u(1) |
| num_short_term_ref_pic_sets | ue(v) |
| for( i = 0; i < num_short_term_ref_pic_sets; i++) |  |
| short_term_ref_pic_set( i ) |  |
| long_term_ref_pics_present_flag | u(1) |
| tiles_or_entropy_coding_sync_idc | u(2) |
| if( tiles_or_entropy_coding_sync_idc = = 1 ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height[ i ] | ue(v) |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } |  |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |
| } |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

## Semantics

view_ref_layer_present_flag equal to 1 indicates that a view reference is present for sequence parameter set prediction. If view_ref_layer_present_flag is not present, it is inferred to be equal to 0.

delta_view_ref_layer_minus1 + 1 indicates the difference of the value of layer_id_plus1 and the value of layer_id_plus1 associated with the reference layer used for sequence parameter set prediction. The value of delta_view_ref_layer_minus1 shall be in the range of 0 to layer_id_plus1 - 2, inclusive.

texture_ref_layer_present_flag equal to 1 indicates that a texture reference is present for sequence parameter set prediction. If texture_ref_layer_present_flag is not present, it is inferred to be equal to 0.

delta_texture_ref_layer_minus1 + 1 indicates the difference of the value of layer_id_plus1 and the value of layer_id_plus1 associated with the reference layer used for sequence parameter set prediction. The value of delta_texture_ref_layer_minus1 shall be in the range of 0 to layer_id_plus1 - 2, inclusive.

view_id specifies a view identifier. If view_id is not present, it is inferred to be equal to 0.

view_order_index specifies the view order index. If view_order_index is not present, it is inferred to be equal to 0.

When a sequence parameter set RBSP is activated and delta_view_ref_layer_minus1 is present, the values of all syntax elements present in the active sequence parameter set RBSP in the layer with layer_id_plus1 equal to layer_id_plus1 - delta_view_ref_layer_minus1 - 1 and not present in the sequence parameter set RBSP are inferred to be active for the layer equal to layer_id_plus1. When a sequence parameter set RBSP is activated and delta_texture_ref_layer_minus1 is present, the values of view_id and view_order_idx in the active sequence parameter set RBSP in the layer with layer_id_plus1 equal to layer_id_plus1 - delta_texture_ref_layer_minus1 - 1 are inferred to be active for the layer equal to layer_id_plus1.

# Discussion

The benefits of the proposed mechanism are as follows:

- Embedded signaling: Layer identifier mapping and decoding dependency signaling for each layer is embedded into an SPS syntax structure associated with that layer. Thus when a sub-bitstream is extracted, all necessary high-level signaling is included, and no unnecessary information (such as high-level signaling related to already discarded layers) is included. This simplifies bitstream extraction operations in the network, in particular when several subsequent extractions are performed. Additionally, unnecessary bit overhead in extracted bitstreams is avoided.

- Extensibility and legacy compatibility: Due to the embedded signaling approach, new signaling required in future extensions to HEVC will be included in SPSs associated with layers that are specific to those new extensions. Any signaling in lower layers that do not include mechanisms from the new extensions will remain unchanged, and thus be compatible with decoders that do not support the new extensions.

- Bitrate efficiency: Due to the inter-layer SPS prediction, signaling overhead is reduced.

As compared to approaches that signal layer identifier mapping and decoding dependencies in a single data structure (SPS or VPS) for all layers, the proposed mechanism has the following potential drawbacks:

- Number of SPS NAL units: In the proposed approach, since layer mapping and dependency signaling is provided in the SPS, it is not possible to re-use SPS across layers. Thus a separate SPS NAL unit needs to be available for each layer, which potentially leads to an increased number of NAL units. It is remarked that in typical scenarios with few layers (such as the example given herein), the number of additional NAL units is very low. As of the SPS prediction, the bit overhead associated with the additional SPS NAL units is reduced to a minimum.

- Availability of scalability information: In order to identify all layer mapping and dependency information in a multi-layer bitstream, the proposed approach requires that SPSs for all available layers are parsed and activated. In a typical case where coded slices for all layers are included in the first access unit of the sequence, the scalability information will be available after parsing the slice headers of that first access unit. In a hypothetical case where there are no coded slices associated with some higher layers in the first access unit, further delay would occur. It is remarked that this fact does not have any negative impact on the normative decoding process. This potential drawback affects only non-normative operations such as performed in network elements that could benefit from having all scalability information available upfront. It is further remarked that other parameter set based mechanisms (based on SPS or VPS) require at least some form of parameter set activation (at least the base layer parameter sets need to be activated) before any scalability information is available. For applications that require upfront-availability of all scalability information for non-normative purposes, alternative signaling mechanisms may be more suitable.

# References

- Video Group, “Test Model under Consideration for HEVC based 3D video coding,” ISO/IEC JTC1/SC29/WG11 N12559, San Jose, CA, USA, February 2012.

- Thomas Rusert, “3D-HEVC-CE8 results on component extraction and bitstream extraction software,” ISO/IEC JTC1/SC29/WG11 M23769, San Jose, CA, USA, February 2012.

- Thomas Rusert, “3D-HEVC HLS: Inter-layer SPS prediction,” ISO/IEC JTC1/SC29/WG11 M24817, Geneva, CH, April 2012.

- JCT-VC, “High-efficiency video coding (HEVC) text specification draft 6,” JCTVC-H1003, San Jose, CA, USA, February 2012.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).