| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, CH, 27th April – 7th May, 2012 | Document: JCTVC-I0538 WG11 Number: m25125 |
| --- | --- |

| Title: | AHG15: Crosscheck - On Modification on Picture Marking Process (JCTVC-I0135) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Sachin Deshpande, Jie Zhao 5750 NW Pacific Rim Blvd Camas, WA 98607 | Tel: Email | +1-360-817-8486 sdeshpande@sharplabs.com |
| Source: | SHARP |  |  |

_____________________________

# Abstract

In this document, crosscheck results for JCTVC-I0135 “On Modification on Picture Marking Process” [1] are reported. Software and results were provided by Panasonic. The verification task has been done successfully. We confirmed that the code has decoding error when used without 2 of the specified macros as described in JCTVC-I0135, and no error when all the specified macros are enabled. We also verified the RD result match with the results in the excel sheet by the proponents.

# Cross-check Configurations

JCTVC-I0135 reports a mismatch between CD text and HM6.1 software regarding picture marking process when a picture is assigned to both short term and long term pictures. It also proposes a modified picture marking process.

Panasonic provided code, with ability to run the code using the proposed technology and without the proposed technology. The code base used was HM6.1-dev-ahg15, and four macros are added. Tests were performed using Class D sequences with Test cases 2.6 encoder_randomaccess_2_6_10s_30s and encoder_randomaccess_2_6_20s_60s specified in JCTVC-H0275 [2].

- BasketballPass, 416x240, 50fps, 4000frames(10s-30s-10s-30), 8000frames(20s-60s-20s-60s)

- BQSquare,       416x240, 60fps, 4800frames(10s-30s-10s-30), 9600frames(20s-60s-20s-60s)

- BlowingBubbles, 416x240, 50fps, 4000frames(10s-30s-10s-30), 8000frames(20s-60s-20s-60s)

- Racehorses,     416x240, 30fps, 2400frames(10s-30s-10s-30), 4800frames(20s-60s-20s-60s)

# Test without proposal

In the first test, the macros are defined as follows. This is without JCTVC-I0135 proposal but has modification to the HM6.1-dev-ahg15 source code to remove the restriction on disallowing to assign a picture to both short term and long term reference (function rpsContainsShortTermReference is commented out).

- #define REMOVE_DEPENDENCY_FOR_LISTENTRY   0

- #define MODIFY_MARKING_PROCESS            0

- #define LONGTERM_BUGFIX                   1

- #define RPS_CHECK_OFF                     1

In this setting, it results in decoding error when coded with the configurations listed in section 1. A sample decoder error message of picture lost is shown below:

Short-term reference picture with POC = 296 is lost or not correctly decoded! inserting lost poc : 296 copying picture 288 to 296 (292) DeltaPOC = { -2* 2* 6* } …..

# Test with proposal

In the second test, all four macros are defined as 1. This is with the proposed technology.

- #define REMOVE_DEPENDENCY_FOR_LISTENTRY   1

- #define MODIFY_MARKING_PROCESS            1

- #define LONGTERM_BUGFIX                   1

- #define RPS_CHECK_OFF                     1

In above setting, we confirmed that no error happened in both encoding and decoding when coded with the test configurations listed in section 1.

# RD Results

We obtained RD result of JCTVC- I0135 code using test configuration listed in section 1. Class D sequences are coded with 2400 to 9600 frames. Test case 2.6 config files encoder_randomaccess_2_6_10s_30s and encoder_randomaccess_2_6_20s_60s specified in JCTVC- JCTVC-H0275 are used. We obtained identical RD the result as provided in proponent’s excel sheet (Attached).

# Conclusion

Crosscheck for JCTVC-I0135 “On Modification on Picture Marking Process” is done successfully. We verified the decoding error occurred without proponent’s technology, and no decoding error with proponent’s technology. We also got identical RD result as provided by proponent.

# References

- T. Sugio, T. Nishi, S. M. T. Naing, C. S. Lim, "AHG15: On Modification on Picture Marking Process," 9th Meeting: Geneva, CH, 27th April – 7th May, 2012, JCTVC-I0135

- Y-K. Wang, M. M. Hannuksela, T. K. Tan, R. Sjöberg, “Common conditions for reference picture marking and list construction proposals,” JCT-VC document JCTVC-G1036, Geneva, Nov. 2011.

- Y-K. Wang, M. M. Hannuksela, T. K. Tan, R. Sjöberg, and Yan Ye, “Common conditions for reference picture marking and list construction proposals,” JCT-VC document JCTVC-H0725, San Jose, February 2012.