| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0542_r1 WG11 Number: m25130 |
| --- | --- |

| Title: | Deblocking filter length adjustment |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Andrey Norkin Rickard Sjöberg Färögatan 6 164 80 Stockholm Sweden | Tel: Email: | +46 10 714 84 02 andrey.norkin@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

The document proposes to introduce a slice/APS level parameter that controls the "length" of deblocking filters in HEVC. The proposed parameter affects two things: how many pixels are modified by deblocking filter and how often the strong filter is chosen. This enables to increasing the "span" of deblocking filter from the block boundary without increasing the number of block boundaries where the deblocking filtering is applied.

# Introduction

The number of pixels from the block boundary that are modified by deblocking filtering (in case the deblocking filtering is applied) is determined by the following equations.

The variable d is derived as follows:

dp0 = | p2,0 − 2*p1,0 + p0,0 | ;  dq0 = | q2,0 − 2*q1,0 + q0,0 |

dp3 =| p2,3 − 2*p1,3 + p0,3 |;   dq0 = | q2,3 − 2*q1,3 + q0,3 |

dpq0 = dp0 + dq0; dpq3 = dp3 + dq3; dp = dp0 + dp3; dq = dq0 + dq3

d = dpq0 + dpq3							(1)

The deblocking filtering is performed on the 4-sample part of block boundary if the following condition is satisfied:

d<β( QP)							(2)

If the condition in (2) is fulfilled, filtering performed for the block boundary. Then one of two types of filtering (weak or strong filtering) is performed. The choice between the strong and the weak filtering is done separately for each line depending on the following conditions. For the block boundary, strong filtering is performed if all the conditions below are satisfied (for lines i = 0 and i=3), otherwise, weak (normal) filtering is performed i.

2*dpqi < (β>>2)									(3)

and (|p3i - p0i| + |q0i – q3i|) < (β>>3)  	   					(4)

and |p0i – q0i| < ((5*tC + 1)>>1),						(5)

Weak filtering

In case weak filtering is chosen, it is possible that no pixels from the block boundary are filtered (if it is likely to be a natural edge), one pixel or two pixels are filtered. The decisions between filtering one or two pixels from the block boundary are done base on the following conditions.

Two pixels from side p are filtered in case the condition below is true.

dp  < ( β + ( β >> 1 ) ) >> 3							(6)

Otherwise, only one pixel from the block boundary is modified. The equation below has the same effect for pixels at "q" side.

dq  < ( β + ( β >> 1 ) ) >> 3							(7)

Once can see that the decisions between filtering one or two pixels from the block boundary, or two or three pixels from the block boundary depend on the same parameter β. However, the thresholds use in these equations are β/8, 3β/16, and β/4. Therefore, in order to adjust the deblocking filter to filter more pisels from the block boundary, parameters β has to be changed, which would also increase the frequency, with which the block boundaries are chosen for deblocking filterin. It is therefore suggested to introduce a parameter deblocking filter length offset, which enables modifying thresholds in (3), (4), (5), (6) independently of parameter β in (2).

# Proposed modifications

## Modification of thresholds calculation

It is proposed to introduce a threshold parameter for the deblocking length adjustment that would change the value of thresholds (β>>2), (β>>3), ( β + ( β >> 1 ) ) >> 3) in equations (4), (5), (7), (8). In order to achieve that, it is suggested to use instead of β the following parameter BetaLength.

BetaLength = β * (4 +  LoopFilterLengthOffset) 			(9)

and additionally divide the value of thresholds by 4. In this way, the value of BetaLength is approximately equal to the value of Beta when the LengthOffset is equal to 0. Then the operation required to obtain the value of BetaLength is basically multiplying β with 4 and then dividing it by 4 when calculating the threshold values in (3), (4), (6), (7).

A multiplication of beta with (4 +  LoopFilterLengthOffset) is used in order to have finer granularity threshold modifications signaling integer values of LengthOffset in the bitstream. The dependency of the  thresholds on the signaled offset is shown in Figure 1 and Figure 2).

The value of BetaLength can be used instead of β for calculation of all the mentioned thresholds (β>>2), (β>>3), ( β + ( β >> 1 ) ) >> 3) .

Figure 1 Dependency of the thresholds in equations (3) (4) (6) on QP values (solid red, green and blue lines respectively). The dashed lines correspond to the values of DeblockingFilterLenghOffset = 1.

Figure 2 Dependency of the thresholds in equations (4) (7) (5) on QP values (solid red, green and blue lines respectively). The dashed lines correspond to the values of DeblockingFilterLenghOffset = 2.

## Signaling the parameter

It is suggested to send a deblocking length offset (DeblcokingFilterLengthOffset) in the slice header or alternatively in APS. The proposed signaling is shown in the tables below.

Table 1 Signaling deblocking_filter_length_offset in slice header

| if( deblocking_filter_control_present_flag ) { |  |
| --- | --- |
| if( deblocking_filter_in_aps_enabled_flag ) |  |
| inherit_dbl_params_from_aps_flag | u(1) |
| if( !inherit_dbl_params_from_aps_flag ) { |  |
| disable_deblocking_filter_flag | u(1) |
| if( !disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| deblocking_filter_length_offset | se(v) |
| } |  |
| } |  |

Table 2 Signaling deblocking_filter_length_offset in APS

| aps_deblocking_filter_flag | u(1) |
| --- | --- |
| if(aps_deblocking_filter_flag) { |  |
| disable_deblocking_filter_flag | u(1) |
| if( !disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| deblcoking_filter_length_offset | se(v) |
| } |  |

The following operations are used in the source code.

Int iBeta = betatable_8x8[iIndexB]*iBitdepthScale;

Int iBetaLength = iBeta * (4+m_dfLengthOffset);   			  		(10)

Int iSideThreshold = (iBetaLength+(iBetaLength>>1))>>5;

Equations (3) and (4) become equations (11) and (12) respectively.

2*dpqi < (BetaLength >>4)								(11)

(|p3i - p0i| + |q0i – q3i|) < (BetaLength>>5)  	   					(12)

# Results

The following results have been obtained on version HM6.0.

When the LengthOffset is equal to 0, the results identical with HM6.0 is obtained (see)

Table 3 Comparision HM6.0 with LengthOffset set to 0. The results are identical

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 110% |  |  | 108% |  |  |
| Dec Time[%] | 109% |  |  | 107% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  | 106% |  |  |
| Dec Time[%] | 117% |  |  | 112% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 105% |  |  | 107% |  |  |
| Dec Time[%] | 113% |  |  | 118% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 104% |  |  | 104% |  |  |
| Dec Time[%] | 110% |  |  | 109% |  |  |

Table 4  Comparison of HM6.0 with LengthOffset set to 8.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.3% | 0.0% | 0.0% | 0.4% | 0.0% | 0.0% |
| Class B | 0.2% | 0.0% | 0.0% | 0.3% | 0.0% | 0.0% |
| Class C | 0.2% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% |
| Class D | 0.2% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% |
| Class E | 0.2% | 0.0% | 0.0% | 0.3% | 0.0% | 0.0% |
| Class F | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| Overall | 0.2% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% |
|  | 0.2% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% |
| Enc Time[%] | 92% |  |  | 94% |  |  |
| Dec Time[%] | 89% |  |  | 93% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.4% | -0.1% | 0.1% | 0.8% | 0.2% | 0.0% |
| Class B | 0.5% | 0.0% | 0.1% | 0.7% | 0.0% | 0.1% |
| Class C | 0.4% | 0.0% | -0.1% | 0.6% | 0.0% | 0.0% |
| Class D | 0.4% | 0.1% | 0.2% | 0.5% | 0.2% | 0.2% |
| Class E |  |  |  |  |  |  |
| Class F | 0.2% | 0.0% | 0.0% | 0.4% | 0.1% | 0.2% |
| Overall | 0.4% | 0.0% | 0.1% | 0.6% | 0.1% | 0.1% |
|  | 0.4% | 0.0% | 0.1% | 0.6% | 0.1% | 0.1% |
| Enc Time[%] | 92% |  |  | 96% |  |  |
| Dec Time[%] | 94% |  |  | 93% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.9% | 0.1% | 0.2% | 1.3% | 0.4% | 0.3% |
| Class C | 0.8% | 0.3% | 0.0% | 1.1% | 0.3% | 0.2% |
| Class D | 0.7% | -0.2% | 0.7% | 0.8% | 0.4% | 0.1% |
| Class E | 2.6% | 0.8% | 0.9% | 4.0% | 1.4% | 1.4% |
| Class F | 0.5% | 0.1% | 0.0% | 0.7% | -0.8% | 0.5% |
| Overall | 1.0% | 0.2% | 0.3% | 1.5% | 0.3% | 0.5% |
|  | 1.0% | 0.2% | 0.4% | 1.5% | 0.3% | 0.4% |
| Enc Time[%] | 94% |  |  | 97% |  |  |
| Dec Time[%] | 91% |  |  | 98% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.2% | -0.1% | -0.3% | 0.9% | 0.3% | -0.1% |
| Class C | 0.4% | -0.3% | -0.1% | 0.8% | 0.1% | 0.2% |
| Class D | 0.4% | 0.2% | 0.0% | 0.5% | 0.4% | -0.4% |
| Class E | 1.3% | 0.1% | 0.3% | 2.8% | 1.2% | 0.4% |
| Class F | 0.3% | 0.5% | -0.1% | 0.5% | 1.1% | -0.9% |
| Overall | 0.5% | 0.1% | -0.1% | 1.0% | 0.6% | -0.2% |
|  | 0.5% | 0.0% | -0.1% | 1.0% | 0.5% | -0.1% |
| Enc Time[%] | 95% |  |  | 95% |  |  |
| Dec Time[%] | 92% |  |  | 91% |  |  |

One can see the BD-rate increase when the Loop Filter Length Offset is set equal to 8. One can see that the decoded video sequences look on average slightly smoother when the LoopFilterLengthOffset is set to equal to 8 compared to the anchor.

# Conclusions

It is proposed to adopt the proposed modification for signaling the deblocking parameters to the next version of HM and the standard draft.

# Standard draft text modifications

In addition to the changes to slice header and APS syntax shown in Table 1 and Table 2, the following changes should be done to the standard draft text.

##### Decision process for luma block edge

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top left luma sample of the current coding unit,

–	a variable verticalEdgeFlag,

–	a variable bS specifying the boundary filtering strength,

Output of this process is:

–	variables dE, dEp, dEq containing decisions,

This process is invoked only when bS is not equal to 0.

Let s’ represent the luma sample array recPictureL of the current picture.

A variable qPL is derived as follows:

qPL = ( ( QPY + QPP + 1 ) >> 1 )

where QPP is specified by the QPY value of the coding unit containing the sample p0,0.	(8-1)

A variables β is specified as Error: Reference source not found with luma quantization parameter Clip3( 0, 51, qPL + ( beta_offset_div2 << 1 ) ) as input.

A variable βL is derived as follows:

βL = β * (4+dfLengthOffset)

A variable tC is specified as Error: Reference source not found with luma quantization parameter Clip3( 0, 55, qPL + 2*( bS − 1) + ( tc_offset_div2 << 1) ) as input.

Depending on verticalEdgeFlag, the following applies:

–	If verticalEdgeFlag is equal to 1, the following ordered steps apply:

- The sample values pi,k and qi,k with i = 0..3 and k = 0, 3 are derived as follows:

qi,k = s’[ xC + xB +i, yC + yB + k ]		(8-2)

pi,k = s’[ xC + xB – i − 1, yC + yB + k ]		(8-3)

- The variables dpq0, dpq3, dp, dq are derived as follows:

dp0 = | p2,0 − 2*p1,0 + p0,0 |

dp3 =| p2,3 − 2*p1,3 + p0,3 | 		(8-4)

dq0 = | q2,0 − 2*q1,0 + q0,0 | 		(8-5)

dq0 = | q2,3 − 2*q1,3 + q0,3 |		(8-6)

dpq0 = dp0 + dq0		(8-7)

dpq3 = dp3 + dq3		(8-8)

dp = dp0 + dp3		(8-9)

dq = dq0 + dq3		(8-10)

- The variable d is derived as follows:

d = dpq0 + dpq3		(8-11)

- The variables dE, dEp and dEq are set equal to 0.

- if d  is less than β, the following ordered steps apply:

- For the sample location ( xC + xB, yC + yB ), the decision process for a luma sample specified in subclause 0.1.1.1.2 is invoked with sample values pi,0, qi,0, the variables 2*dpq0, βL and tC as inputs and a decision dSam0 as output.

- For the sample location ( xC + xB, yC + yB + 3), the decision process for a luma sample specified in subclause 0.1.1.1.2 is invoked with sample values pi,3, qi,3, the variables 2*dpq3, βL and tC as inputs and a decision dSam3 as output.

- The variable dE is set equal to 1.

- If dSam0 is equal to 1 and dSam3 is equal to 1, the variable dE is set equal to 2.

- If dp is less than ( βL + ( βL >> 1 ) ) >> 5, the variable dEp is set equal to 1.

- If dq is less than ( βL + ( βL >> 1 ) ) >> 5, the variable dEq is set equal to 1.

–	Otherwise (verticalEdgeFlag is equal to 0), the following ordered steps apply:

- The sample values pi,k and qi,k with i = 0..3 and k = 0, 3 are derived as follows:

qi,k = s’[ xC + xB +k, yC + yB + i ]		(8-12)

pi,k = s’[ xC + xB +k, yC + yB – i − 1 ]		(8-13)

- The variables dpq0, dpq3, dp, dq are derived as follows:

dp0 = | p2,0 − 2*p1,0 + p0,0 | 		(8-14)

dp3 =| p2,3 − 2*p1,3 + p0,3 | 		(8-15)

dq0 = | q2,0 − 2*q1,0 + q0,0 | 		(8-16)

dq0 = | q2,3 − 2*q1,3 + q0,3 |		(8-17)

dpq0 = dp0 + dq0		(8-18)

dpq3 = dp3 + dq3		(8-19)

dp = dp0 + dp3		(8-20)

dq = dq0 + dq3		(8-21)

- The variables d is derived as follows:

d = dpq0 + dpq3		(8-22)

- The variables dE, dEp and dEq are set equal to 0.

- if d is less than β, the following ordered steps apply:

- For the sample location ( xC + xB, yC + yB ), the decision process for a luma sample specified in subclause 0.1.1.1.2 is invoked with sample values pi,0, qi,0 the variables 2*dpq0, βL and tC as inputs and a decision dSam0 as output.

- For the sample location ( xC + Xb + 3, yC + yB ), the decision process for a luma sample specified in subclause 0.1.1.1.2 is invoked with sample values pi,3, qi,3, the variables 2*dpq3, βL and tC as inputs and a decision dSam3 as output.

- The variable dE is set equal to 1.

- If dSam0 is equal to 1 and dSam3 is equal to 1, the variable dE is set equal to 2.

- If dp is less than ( βL + ( βL >> 1 ) ) >> 5, the variable dEp is set equal to 1.

- If dq is less than ( βL + ( βL >> 1 ) ) >> 5, the variable dEq is set equal to 1.

##### Decision process for a luma sample

Inputs of this process are:

–	sample values, pi and qi with i = 0..3,

–	variables d, βL and tC.

Output of this process is:

–	a variable dSam containing a decision

The variable dSam is specified as follows:

–	If d is less than ( βL >> 4 ), | p3 – p0 | + | q0 – q3 | is less than ( βL >> 5 ) and | p0 – q0 | is less than ( 5*tC + 1 ) >> 1, dSam is set equal to 1.

–	Otherwise, dSam is set equal to 0.

# Patent rights declaration(s)

Ericsson may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).