| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0545 |
| --- | --- |

| Title: | AHG5: Cross-verification for Method 1 of Unified context selection for significance map coding of 4x4 and 8x8 TUs (JCTVC-I0155) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Gergely Korodi, Dake He 295 Phillip St. Waterloo, ON, Canada N2L 3W8 | Tel: Email: | +1 519 888 7465 {gkorodi,dhe}@rim.com |
| Source: | Research in Motion Limited |  |  |

_____________________________

# Abstract

This contribution reports cross-verification results on Method 1 of JCTVC-I0155, “Unified context selection for significance map coding of 4x4 and 8x8 TUs.”  The results match those provided by the proponent for all mandatory test sequences.

# Introduction and Discussions

In HM-6.0, 4x4 and 8x8 significance map coding uses different context assignments based on TU size and type (luma or chroma).  Contribution [1] proposes to use the 4x4 luma context assignment for 4x4 chroma, and after downsampling the 8x8 indices, for 8x8 luma and chroma.

The proponent has provided the source code to [1] based on HM-6.0.  A study of the code concluded that it matched the algorithmic descriptions in [1].  Instead of the three context assignment tables used in HM-6.0 for 4x4 and 8x8 significance maps, [1] uses a single table of 16 constant unsigned characters (the usage of 4-bit values would be sufficient).  Compared to HM-6.0, which uses 9 contexts for 4x4 luma, 6 for 4x4 chroma, 11 for 8x8 luma and 11 for 8x8 chroma, the unified context assignment in [1] uses 9 contexts for each of the aforementioned TU sizes and types.  Unlike in HM-6.0, no additional DC context is used for 8x8 TUs.  Overall, Method 1 of [1] uses 1 less context than HM-6.0.  The specification of 4x4 and 8x8 significance map coding is simplified.  Apart from that, this contribution seems to have no other effects on HM-6.0.

The initial values for the contexts used for 4x4 and 8x8 significance map coding are derived from a straightforward remapping of HM-6.0 init values.  Unaffected contexts retain their values from HM-6.0.

# Results

The source code provided for [1] has been compiled and tested under 64-bit GNU/Linux environments, using GCC 4.4.6.  Cross-verification has been done on all the configurations not flagged as optional in [2], including low QP configurations.  The results agree with those presented in [1] for Method 1.  The complete spreadsheets for the cross-verified results are attached.

# References

- C.-W. Hsu, T.-D. Chuang, Y.-W. Huang and S. Lei, “AHG5: Unified context selection for significance map coding of 4x4 and 8x8 TUs”, JCTVC-I0155, JCT-VC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 9th Meeting, Geneva, April-May 2012.

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-H1100, JCT-VC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 8th Meeting, San Jose, February 2012.