| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0550 |
| --- | --- |

| Title: | Sum-128 Luma/Chroma Interpolation Filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongbo Zhu China | Tel: Email: | hurricanezhb@hotmail.com |
| Source: | None |  |  |

_____________________________

# Abstract

The luma/chroma interpolation filter whose sum of all coefficients are 128 is proposed in the document. The simulation is conducted under the common test condition [1] using the reference software HM5.0 r1682. The results is -0.1%Y, 0.1%U, 0.1%V for he_ra, 0.1%Y, 0.1%U, 0.2%V for lc_ra, 0.0%Y, 0.1%U, 0.2%V for lb_he, 0.5%Y, -0.2%U, -0.4%V for lb_lc, -0.2%Y, 0.1%U, -0.2%V for lp_he and -1.8%Y, -1.4%U, -1.6%V for lp_lc.

# The Proposed Interpolation Filter

The proposed luminance interpolation filter is shown in table 1. The proposed chrominance interpolation filter is shown in table 2. The sum of all coefficients is always 128. When implemented in the HM, the intermediate result is scaled to 15 bits signed number when the interpolation needs two steps, that is same as the current design.

| Sub-Pel position | Phase Shift | Filter |
| --- | --- | --- |
| 1/4 | 1/4 | -3, 9, -20, 116, 34, -11, 3, 0 |
| 2/4 | 2/4 | -3, 9, -21, 79, 79, -21, 9, -3 |
| 3/4 | 3/4 | 0, 3, -11, 34, 116, -20, 9, -3 |

Table 1: the proposed luminance interpolation filter

| Sub-Pel position | Phase Shift | Filter |
| --- | --- | --- |
| 1/8 | 1/8 | -5,117, 21, -5 |
| 2/8 | 2/8 | -7,105, 35, -5 |
| 3/8 | 3/8 | -7, 87, 55, -7 |
| 4/8 | 4/8 | -9, 73, 73, -9 |
| 5/8 | 5/8 | -7, 55, 87, -7 |
| 6/8 | 6/8 | -5, 35,105, -7 |
| 7/8 | 7/8 | -5, 21,117, -5 |

Table 2: the proposed chrominance interpolation filter

# The Results

The HM5.0 r1682 is used as the platform to test the proposed filter. The simulation experiments were conducted following the testing conditions in [1]. The encoding/decoding times were estimated using two PC listed in table 3. The results are listed in table 4.

| CPU | AMD Phenom(tm) II x6 1075T@3.0GHz (6 cores) | AMD bulldozer FX-8150 @3.6GHz (8 cores) |
| --- | --- | --- |
| DRAM | 6G 1333MHz | 8G 1333MHz |
| BUS | 1333MHz | 4.8GT/s |
| HardDisk | WD 1TB | WD 512GB |
| OS | Win7x64 | Win7x64 |
| EXE | X64 version (VC2008) | X64 version (VC2008) |

Table 1: the computers to generate the results

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.1% | 0.1% | 0.0% | 0.1% | -0.1% | 0.0% |
| Class B | 0.1% | 0.2% | 0.2% | 0.3% | 0.3% | 0.2% |
| Class C | -0.2% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | -0.3% | 0.0% | 0.1% | 0.0% | 0.2% | 0.3% |
| Class E |  |  |  |  |  |  |
| Overall | -0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.2% |
|  | -0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.3% | 0.4% | 0.4% | 0.9% | -0.1% | -0.4% |
| Class C | -0.2% | -0.3% | 0.0% | 0.2% | -0.8% | -0.7% |
| Class D | -0.4% | 0.1% | 0.3% | 0.3% | 0.8% | 0.1% |
| Class E | 0.4% | 0.1% | -0.2% | 0.9% | -1.0% | -0.7% |
| Overall | 0.0% | 0.1% | 0.2% | 0.5% | -0.2% | -0.4% |
|  | 0.0% | 0.1% | 0.1% | 0.5% | -0.2% | -0.5% |

|  | Low delay P HE |  |  | Low delay P LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.4% | 0.0% | -1.9% | -0.8% | -1.6% |
| Class C | -0.3% | -0.1% | -0.4% | -1.5% | -1.1% | -1.4% |
| Class D | -0.5% | 0.3% | -0.1% | -1.3% | -0.9% | -0.5% |
| Class E | 0.1% | -0.4% | -0.2% | -2.9% | -3.6% | -3.4% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
|  | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

Table 4: Rate-distortion results of the proposed interpolation filter

# Conclusion

The sum-128 luma/chroma interpolation filter is proposed in the document that is a little better than the default interpolation filter.

# References

- F. Bossen, “Common test conditions and software reference configurations,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-G1200, Nov. 2011.

# Patent rights declaration(s)

Hongbo Zhu may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).