| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0552 |
| --- | --- |

| Title: | AHG13: Suggested Bug-fixes for Ticket 472 and Ticket 476 Related to Lossless Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wen Gao Minqiang Jiang Haoping Yu Huawei Technologies (USA) 2330 Central Expressway Santa Clara, CA 95050 | Tel: Email: | +1 408 330 4813 wen.gao@huawei.com Minqiang.Jiang@huawei.com haoping.yu@huawei.com |
| Source: | Huawei Technologies (USA) |  |  |

_____________________________

# Abstract

In this contribution, suggested bug-fixes for ticket 472 and ticket 476 related to lossless coding are provided.

# Problem Statement for Ticket 472

JCT-VC HM Ticket #472 states:

"In HM6.1, the deblocking filter will disable filtering for the left and top boundaries of a losslessly coded block, while still filtering the bottom and right edges.

It is suggested to replace the following code in HM functions xEdgeFilterLuma and xEdgeFilterChroma:

#if LOSSLESS_CODING

// check if each of PUs is lossless coded

bPartPNoFilter = bPartPNoFilter || (pcCU->isLosslessCoded(uiAbsZorderIdx) );

bPartQNoFilter = bPartQNoFilter || (pcCU->isLosslessCoded(uiAbsZorderIdx) );

#endif

with the following code

#if LOSSLESS_CODING

// check if each of PUs is lossless coded

bPartPNoFilter = bPartPNoFilter || (pcCUP->isLosslessCoded(uiPartPIdx) );

bPartQNoFilter = bPartQNoFilter || (pcCUQ->isLosslessCoded(uiPartQIdx) );

#endif

"

# Suggested Solution to Ticket 472

The software bug has been confirmed. The bug doesn't exist in the software associated with JCTVC-H0530 [1]  but was introduced during the software integration.

Suggest adopting the fix into HM software

# Problem Statement for Ticket 476

JCT-VC HM Ticket #476 states:

"In Section 8.7.1.4.5 and Section 8.7.1.4.6, the text regarding bypassing deblock filter process in lossless coding is missing in the current CD. However, the related text was submitted in the JCTVC-H0530 contribution document, H0530-WD-r3.

In the attached document, the missing text is inserted back in Section 8.7.1.4.5 and Section 8.7.1.4.6."

# Suggested Solution to Ticket 476

The attachment mentioned in Ticket 476 is included in the Appendix.

Suggest adopting the text shown in the Appendix into HEVC Committee Draft .

# References

[1] W. Gao, M. Jiang, H. Yu, M. Zhou, “AHG19: A lossless coding solution for HEVC”, JCT-VC Document, JCTVC-H0530, San Jose, USA.  Feb. 2012.

[2]  B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand  “High Efficiency Video Coding (HEVC) text specification draft 6” JCT-VC Document, JCTVC-H1103, San Jose, USA.  Feb. 2012.

# Patent rights declaration(s)

Huawei Technologies (USA) may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation |ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix

The text is in the following pages with modified portion highlighted in yellow color.

##### Filtering process for a luma sample

Inputs of this process are:

–	sample values, pi and qi with i = 0..3,

–	variables dEp1 and dEq1 containing decisions to filter pixels p1 and q1 respectively,

–	a variable tC.

Output of this process is:

–	number of filtered samples nDp and nDq,

–	filtered sample values, pi’ and qj’ with i = 0..nDp − 1, j = 0..nDq − 1

Depending on dE, the following applies:

–	When the variable dE is equal to 2, the following strong filtering applies while nDp and nDq are set equal to 3:

p0’ = Clip3( p0−2*tc, p0+2*tc, ( p2 + 2*p1 + 2*p0 + 2*q0 + q1 + 4 ) >> 3 )	(8-1)

p1’ = Clip3( p1−2*tc, p1+2*tc, ( p2 + p1 + p0 + q0 + 2 ) >> 2 )	(8-2)

p2’ = Clip3( p2−2*tc, p2+2*tc, ( 2*p3 + 3*p2 + p1 + p0 + q0 + 4 ) >> 3 )	(8-3)

q0’ = Clip3( q0−2*tc, q0+2*tc, ( p1 + 2*p0 + 2*q0 + 2*q1 + q2 + 4 ) >> 3 )	(8-4)

q1’ = Clip3( q1−2*tc, q1+2*tc, ( p0 + q0 + q1 + q2 + 2 ) >> 2 )	(8-5)

q2’ = Clip3( q2−2*tc, q2+2*tc, ( p0 + q0 + q1 + 3*q2 + 2*q3 + 4 ) >> 3 )	(8-6)

–	Otherwise, the following weak filtering applies while nDp and nDq are set equal to 0:

 = ( 9 * ( q0 –  p0 ) − 3 * ( q1 – p1 ) + 8 ) >> 4		(8-7)

  - When abs() is less than tc*10, the following ordered steps apply:

    - The filtered sample values p0’ and q0’ are specified as follows:

 = Clip3( -tc, tc,  )		(8-8)

p0’ = Clip1Y( p0 +  )		(8-9)

q0’ = Clip1Y( q0 -  )		(8-10)

    - If dEp1 is equal to 1, the filtered sample value pi’ is specified as follows:

p = Clip3( -(tc >> 1), tc >> 1, ( ( ( p2 + p0 + 1 ) >> 1 ) – p1 +  ) >>1 )	(8-11)

pi’ = Clip1Y( p1 + p )		(8-12)

    - If dEq1 is equal to 1, the filtered sample value qi’ is specified as follows:

q = Clip3( -(tc >> 1), tc >> 1, ( ( ( q2 + q0 + 1 ) >> 1 ) – q1 –  ) >>1 )	(8-13)

qi’ = Clip1Y( q1 + q )		(8-14)

    - nDp is set equal to dEp1+1 and nDq is set equal to dEq1+1.

Each of the filtered sample values, pi’ with i = 0..nDp−1, is substituted by the corresponding input sample value pi if any of the following conditions are true.

–	pcm_flag of the coding unit covering the sample pi is equal to 1 and pcm_loop_filter_disable_flag is equal to 1.

–   QP’Y of the coding unit covering the sample pi is equal to 0 and qpprime_y_zero_transquant_bypass_flag is equal to 1.

Similarly, each of the filtered sample values, qj’ with j = 0..nDq−1, is substituted by the corresponding input sample value qj if any of the following conditions are true.

–	pcm_flag of the coding unit covering the sample qj is equal to 1 and pcm_loop_filter_disable_flag is equal to 1.

–	QP’Y of the coding unit covering the sample qj is equal to 0 and qpprime_y_zero_transquant_bypass_flag is equal to 1.

Table 8-1 – Derivation of threshold variables β and tC from input Q

| Q | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| β | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 6 | 7 | 8 |
| tC | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| Q | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 |
| β | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 20 | 22 | 24 | 26 | 28 | 30 | 32 | 34 | 36 |
| tC | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 | 4 | 4 | 4 |
| Q | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 |  |
| β | 38 | 40 | 42 | 44 | 46 | 48 | 50 | 52 | 54 | 56 | 58 | 60 | 62 | 64 | 64 | 64 | 64 | 64 |  |
| tC | 5 | 5 | 6 | 6 | 7 | 8 | 9 | 9 | 10 | 10 | 11 | 11 | 12 | 12 | 13 | 13 | 14 | 14 |  |

##### Filtering process for a chroma sample

Inputs of this process are:

–	sample values, pi and qi with i = 0..1,

–	a variable tC.

Output of this process is:

–	The filtered sample values, p0’ and q0’.

The filtered sample values p0’ and q0’ are derived by

 = Clip3( -tC, tC, ( ( ( ( q0 – p0 ) << 2 ) + p1 – q1 + 4 ) >> 3 ) )	(8-15)

p0’ = Clip1C( p0 +  )		(8-16)

q0’ = Clip1C( q0 -  )		(8-17)

The filtered sample value, p0’ is substituted by the corresponding input sample value p0 if any of the following conditions are true.

–	pcm_flag of the coding unit covering the sample p0 is equal to 1 and pcm_loop_filter_disable_flag is equal to 1.

–   QP’Y of the coding unit covering the sample p0 is equal to 0 and qpprime_y_zero_transquant_bypass_flag is equal to 1.

Similarly, the filtered sample value, q0’ is substituted by the corresponding input sample value q0 if any of the following conditions are true.

–	pcm_flag of the coding unit covering the sample q0 is equal to 1 and pcm_loop_filter_disable_flag is equal to 1.

–	QP’Y of the coding unit covering the sample q0 is equal to 0 and qpprime_y_zero_transquant_bypass_flag is equal to 1.