| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0562 |
| --- | --- |

| Title: | Cross-check of JCTVC-I0281 on adaptive Thresholds for Greater-than-1 and Greater-than-2 Flags |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Wei-Jung Chien 5775 Morehouse Dr. San Diego, CA 92130 USA | Tel: Email: | +1-858-845-1795 wchien@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution is a cross-check report for JCTVC-I0281 based on adaptive thresholds for Greater-than-1 and Greater-than-2 Flags. Two different tests are performed. First test aims to achieve more bypass-coded bins according to the information of the neighboring coefficient groups. The second test aims to achieve more bypass-coded bins according to the information of the previously coded coefficient groups. The cross check results match the results reported by proponents.

# Introduction

This contribution is a report of verification results of JCTVC-I0281[1], which is a contribution on reducing number of regular-coded Greater-than-1 and Greater-than-2 Flags. Two schemes were proposed. The first scheme uses the numbers of the significant coefficients in the neighboring coefficient group (bottom and right) to determine the threshold which controls the number of regular-coded Greater-than-1 and Greater-than-2 Flags in the current coefficient group. Therefore, some memory storages are needed for the scheme 1. The second schemes uses the previously coded coefficient groups as information to determine the threshold. No extra storage is needed for the scheme 2.

# Simulation results

Our encoding environment is using 64bit Linux cluster and decoding was done on single 64 bit windows PC. There was no mismatch between encoder and decoder. There is no mismatch of overall BD-rate between the verification results and the proponent’s results. The statistics of bypass-coded bins are listed in the

Table 1. JCTVC- I0281 simulation results in HM-6.0 for scheme A.

|  | Normal QP | Low QP |
| --- | --- | --- |
| Intra_Main | 0.01%, 0.00%, 0.02% | 0.00%, 0.02%, 0.02% |
| RA_Main | 0.09%, 0.14%, 0.09% | 0.14%, 0.33%, 0.23% |
| LD_Main | 0.03%, -0.02%, 0.32% | 0.17%, 0.32%, 0.22% |
| Intra_HE10 | 0.02%, 0.00%, 0.03% | 0.00%, 0.06%, 0.06% |
| RA_HE10 | 0.10%, 0.06%, 0.05% | 0.22%, 0.46%, 0.32% |
| LD_HE10 | 0.01%, -0.28%, 0.06% | 0.28%, 0.55%, 0.44% |

Table 2. JCTVC-I0281 simulation results in HM-6.0 for scheme B.

|  | Normal QP | Low QP |
| --- | --- | --- |
| Intra_Main | 0.02%, 0.04%, 0.04% | -0.01%, 0.01%, 0.01% |
| RA_Main | 0.06%, 0.09%, 0.05% | 0.05%, 0.22%, 0.17% |
| LD_Main | 0.02%, 0.04%, -0.08% | 0.11%, 0.23%, 0.18% |
| Intra_HE10 | 0.01%, 0.02%, 0.03% | -0.01%, 0.04%, 0.04% |
| RA_HE10 | 0.06%, 0.07%, 0.04% | 0.08%, 0.25%, 0.22% |
| LD_HE10 | -0.01%, -0.01%, 0.32% | 0.15%, 0.39%, 0.35% |

# Reference

- N. Nguyen, T. Ji, D. He, G. Martin-Cocher (RIM), “Adaptive Thresholds for Greater-than-1 and Greater-than-2 Flags”, JCTVC-I0281, Gevena, CH, April, 2012.

- F. Bossen, “Common HM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-H1100, February 2012.