| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, CH, 27-07 APRIL , 2012 | Document: JCTVC-I0563 |
| --- | --- |

| Title: | Non-CE1, Test2.1: Reduced number of band offsets per LCU |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Wen Huang MediaTek In Suk Chong Qualcomm Woo-Shik Kim Texas Instruments Inc., USA Elena Alshina, Samsung Koohyar Minoo Motorola Mobility | Tel: Email: | +886-3-5670766 ext. 25555 yuwen.huang@mediatek.com +1-858-658-5673 ichong@qualcomm.com +1-214-480-1562 wskim@ti.com +82-10-3026-1305 elena_a.alshina@samsung.com +1 (858) 404-2521 kminoo@motorola.com |
| Source: | MediaTek Inc., Qualcomm Inc., TI, Samsung Electronics Co. Ltd., Motorola Mobility |  |  |

_____________________________

# Abstract

This document describes SAO RDO search process modification which were agreed to be useful during CE1 phase 2 [1]. Results are included before to CE1 report only.

# Introduction

During CE1 and AhG6 activity several problems during SAO parameter estimation on encoder side were discovered:

- For SAO Interleaving mode CABAC is used for SAO parameters encoding but VLC is used during bits estimation.

- Band and Edge offsets are encoded differently: EO is coded as unsigned value, BO is signed. But both there assumed as signed during bits estimation.

These problems were fixed by RDO process modification only and tested as so-called S5 in CE1 phase 2 [1]. S/w was released to CE1 members (~90 members).

# Simulation Results

Some of suggested changes were studied on top of HM6.0, others are not.

All together they were implemented during 2nd phase of CE1. At this phase all conditions including Main-AI/RA/LDB/LDP and HE10-AI/RA/LDB/LDP were tested with three different LCU sizes (i.e., 64x64, 32x32, and 16x16). Both APS and Interleaving SAO modes were investigated. Since APS mode was agreed not to use anymore in this standard only interleaving mode performance is reported.

Previous step modifying Interleaving mode was S3. RDO fixes were included into testing under name S5. Performance impact of S5 compare to previous step is reported in Table 1. Table 2 reports more details about S5 performance vs S3.

Table 1. SAO RDO fixes performance (S5) vs SAO off (compare to previous step S3) in SAO interleaving mode.

| Anchor: SAO-off | S3 vs SAO off (interleaving) |  |  | S5 vs SAO off (interleaving) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y (%) | Cb (%) | Cr (%) | Y (%) | Cb (%) | Cr (%) |
| 64x64, all conditions, all classes | -2.3 | -3.1 | -4.0 | -2.3 | -3.5 | -4.4 |
| 32x32, all conditions, all classes | -1.3 | -0.5 | -0.8 | -1.4 | -0.8 | -1.2 |
| 16x16, all conditions, all classes | 0.2 | 0.7 | 0.7 | 0.1 | 0.7 | 0.6 |

Table 2. SAO RDO fixes performance (S5) vs S3 (previous step in CE1 phase 2) in SAO interleaving mode.

| LCU size | Intra |  | RA |  | LDB |  | LDP |  | ALL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | main | HE10 | main | HE10 | main | HE10 | main | HE10 |  |
| 64 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% |
| 32 | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | -0.6% | -0.1% | -0.1% |
| 16 | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | 0.0% | -0.7% | -0.1% | -0.1% |

S/w was released to CE1 members (~90 members) approximately 10 days prior the meeting start. Test results were released to CE1 members at April, 24.

After discussion between CE1 members following was agreed::

- These are reasonable things to do, even though the coding efficiency improvement is small.

# Specification text changes

No changes of specification text are needed

# Conclusion

MediaTek Inc., Qualcomm Inc., TI, Samsung Electronics Co. Ltd., Motorola Mobility suggest adopting SAO RDO process modification tested inCE1 phase 2 to next release of HM s/w.

# Patent rights declarations

Samsung Electronics Co. Ltd. has no current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. has no current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc. has no current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Texas Instruments Inc., has no current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Motorola Mobility has no current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Yu-Wen Huang, Elena Alshina, In Suk Chong, Wade Wan, Minhua Zhou, “CE1: Summary report of Core Experiment on sample adaptive offset filtering,” Document of Joint Collaborative Team on Video Coding, JCTVC-I0021, April 2012.