| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0564_r1 |
| --- | --- |

| Title: | Complexity reduction for intra transform skipping proposal JCTVC-I0408 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marta Mrak Andrea Gabriellini David Flynn Matteo Naccari | Email: | marta.mrak@bbc.co.uk |
| Source: | BBC Research & Development |  |  |

_____________________________

# Abstract

This contribution presents a method to reduce the complexity when the 2D transform is skipped in intra coded blocks. The transform skipping in intra coded blocks is proposed in document JCTVC-I0408 and involves a scaling operation for all the pixels belonging to the block whereby the transform is being skipped. The scaling in JCTVC-I0408 aims at bringing the pixels to same level as if transform would be performed. In this contribution the aforementioned scaling is avoided by modifying (compensating) thebit- shifting operation performed during quantization. Reported results demonstrate that this proposal does not introduce negative effects on BD-rate performance.

# Description of the simplification

In JCTVC-I0408, 2D transform skipping is proposed in the coding order as shown in Table 1. The same table indicates proposed changes (highlighted) and related complexities.

Table 1 Comparison of intra transform skipping methods

| Step | JCTVC-I0408 | Complexity of I0408 compared to HM | Proposed | Complexity of the proposal compared to HM |
| --- | --- | --- | --- | --- |
| (a) Prediction | No change. | Same | Same as in JCTVC-I0408 | Same |
| (b) Transform | Skipped. Instead, for transform skipping TUs, a simple scaling process is used. Since a 4x4 inverse transform in the current design scale down the coefficients by 32, to let transform skipping TUs have similar magnitudes as other TUs, a scaling-down process by 32 is performed on transform skipping TUs. | - Transform skipped (reduction of complexity) - Scaling of N*N coefficients needed | Skipped. | Transform skipped (reduction of complexity) |
| (c) De-quantization and scaling | No change | Same | No change except modified quantization shift once on TU level. | Same |
| (d) Entropy coding | A flag for each 4x4 intra TU is sent to indicate if transform is bypassed or not. Two contexts are added to code the flag for Y, U and V TUs. | Same | Same as in JCTVC-I0408 | Same |
| (e) Deblocking, SAO and ALF | No change | Same | Same as in JCTVC-I0408 | Same |
| (f) A flag in the SPS to indicate whether transform skipping is enabled or not. |  |  |  |  |
| (g) Change the default transform matrix to a flat matrix for 4x4 intra TUs, when transform skipping is enabled. |  |  |  |  |

According to JCTVC-I0408 the scaling of coefficients in step (b) from Table 1 is performed as follows (for 4 × 4 blocks):

If a transform is skipped and both nW and nH are equal to 4, residual sample value rij with i=0..(nW)−1, j=0..(nH)−1 is derived as follows,

rij = ( dij + (1 << ( shift − 1) ) ) >> shift

The variable shift is derived as follows:

–	If cIdx is equal to 0,

shiftTS = 13 - BitDepthY

–	Otherwise,

shiftTS = 13 – BitDepthC

Following the same logic as in JCTVC-I0408, for larger square blocks shiftTS can be defined as:

shiftTS = 15 - log2TrSize  - BitDepth                                                  (1)

for both luma and chroma.

In the HEVC Draft the variable shift (shiftQ) in quantization is derived as follows:

–	If cIdx is equal to 0,

shiftQ = BitDepthY + log2TrSize − 9

–	Otherwise,

shiftQ = BitDepthC + log2TrSize − 9

This variable is used in the derivation of scaled transform coefficient dij with i = 0..nW − 1, j = 0..nH − 1 during dequantization as in the following generalized equation:

dij = Clip3( −32768, 32767,( ( yij * m + o) >> (shiftQ+s ) ) )                           (2)

where m is a multiplier, o is an offset and s is added to shiftQ when scaling lists are not used. In that case s equals 4 – QP/6.

Since both shiftQ and shiftTS depend on the same parameters, in the case of transform skip excessive shifting can be simplified by removing scaling under (b) in Table 1 and by adapting the quantization bit-shift (in step (3) from Table 1) by replacing shiftQ in (2) by shiftQ' = 6 that is derived as:

shiftQ' = shiftQ + shiftTS = BitDepth + log2TrSize − 9 + 15 - log2TrSize  - BitDepth = 6

for both luma and chroma.

# Experiments

The proposed method is implemented on top of the intra transform skipping software from JCTVC-I0408. Intra transform skipping is implemented on top of HM-6.0.

While JCTVC-I0408 includes exhaustive tests of different combinations of block sizes on which transform skipping is applied and faster versions of the encoder, in this contribution only one test is performed. The test corresponds to the intra transform skipping with a fast encoding (Table 2 from JCTVC-I0408).

Results summarized in Table 2 are comparison of the proposed method with HM-6.0 as the anchor. The results are the same as reported in JCTVC-I0408. The results from Table 3, when JCTVC-I0408 is used as the anchor and the tested method remain the same as in Table 2, reveal nearly identical behavior of both methods, in terms of the BD-rate and run times.

Table 2 Proposed method compared to HM-6.0

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class C | -0.1% | 0.3% | 0.3% | -0.1% | 0.2% | 0.3% |
| Class D | 0.0% | 0.3% | 0.3% | 0.0% | 0.3% | 0.3% |
| Class E | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Overall | 0.0% | 0.2% | 0.2% | 0.0% | 0.2% | 0.2% |
|  | 0.0% | 0.2% | 0.2% | 0.0% | 0.2% | 0.2% |
| Class F | -7.3% | -5.5% | -5.4% | -7.3% | -5.7% | -5.5% |
| Enc Time[%] | 114% |  |  | 113% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | -0.1% | 0.0% | 0.0% | 0.2% | -0.2% |
| Class B | 0.0% | 0.0% | 0.3% | 0.0% | 0.0% | 0.2% |
| Class C | -0.1% | 0.2% | 0.2% | -0.1% | 0.2% | 0.2% |
| Class D | 0.0% | 0.0% | 0.3% | 0.0% | 0.4% | 0.3% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.2% | 0.0% | 0.2% | 0.1% |
|  | 0.0% | 0.1% | 0.2% | 0.0% | 0.2% | 0.2% |
| Class F | -5.2% | -3.9% | -3.7% | -5.0% | -4.2% | -4.1% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.2% | -0.2% | 0.0% | -0.2% | 0.3% |
| Class C | 0.0% | 0.2% | 0.0% | 0.0% | 0.3% | 0.2% |
| Class D | 0.1% | 0.2% | 0.4% | 0.1% | 0.0% | 0.4% |
| Class E | 0.0% | -0.5% | 0.4% | 0.0% | 0.3% | 0.3% |
| Overall | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.3% |
|  | 0.0% | -0.1% | 0.2% | 0.0% | 0.1% | 0.2% |
| Class F | -3.0% | -2.9% | -2.9% | -2.8% | -1.3% | -2.9% |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

Table 3 Proposed method for complexity reduction 
compared to fast intra transform skip from JCTVC-I0408

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

# Conclusions

A simplification of intra transform skipping is proposed. Processing of pixels before quantization and after dequantization is avoided by adjustment of the bit-shift used in quantization. Results confirm that such simplification does not influence coding performance.

# Encoder description

The following modifications are based on JCTVC-G1102_d1, to describe how an encoder can be if intra transform skipping is used. The changes as marked with track changes tool. For 6.9.1 Intra prediction mode and parameters, same changes are suggested as proposed in JCTVC-I0408

## 5.5.2 Scaling and quantization

The quantized transform coefficients qij (i, j=0..nS-1) are derived from the transform coeficients dij (i, j=0..nS-1) as

qij = ( dij * f[QP%6] + offset  ) >> ( shift ), with i,j = 0,...,nS-1

where

f[x] = {26214,23302,20560,18396,16384,14564}, x=0,…,5

- if the transform is used

shift = 29 + QP/6 – nS – BitDepth

- otherwise

shift =14 + QP/6

2shift - 1  < offset < 2shift

# Proposed text modifications

The following proposed modifications are based on JCTVC-H1003_dK and JCTVC-I0408. All changes proposed in JCTVC-I0408 are preserved, except suggested changes in 8.6.4 on Transformation process for scaled transform coefficients. Additional proposed changes as marked with the track changes tool.

8.6.3 Scaling process for transform coefficients

Inputs of this process are:

–	a variable nW specifying the width of the current transform unit,

–	a variable nH specifying the height of the current transform unit,

–	a (nW)x(nH) array c of transform coefficients with elements cij,

–	a variable cIdx specifying the chroma component of the current block,

–	a variable qP specifying the quantization parameter.

Output of this process is scaled transform coefficients as a (nW)x(nH) array of d with elements dij.

The variable log2TrSize is derived as follows:

log2TrSize = ( Log2( NW ) + Log2( NH ) ) >> 1		(8-1)

The variable shift is derived as follows:

–	If ts_flag[x0][y0][cIdx] is equal to 1 and both nW and nH are equal to 4,

shift = 6

–	Otherwise if cIdx is equal to 0,

shift = BitDepthY + log2TrSize − 9

–	Otherwise,

shift = BitDepthC + log2TrSize − 9

The variable levelLimit  is derived as follows:

–	If cIdx is equal to 0,

levelLimit = 1 << Min( 15, 12 + BitDepthY + log2TrSize – QP / 6 )

–	Otherwise,

levelLimit = 1 << Min( 15, 12 + BitDepthC + log2TrSize – QP / 6 )

The scaling array levelScale[·] is specified as levelScale[k] = { 40, 45, 51, 57, 64, 72 } with k=0..5.

The variable shiftScale is set equal to shift + 4 – QP/6.

The elements of array M[ i ][ j ] with i = 0..nW − 1, j = 0..nH − 1 are set equal to ScalingFactor[ SizeID ][ RefMatrixID ][ trafoType ][ i*nW+j ], where SizeID and RefMatrixID are specified in Error: Reference source not found and Equation Error: Reference source not found, respectively, and trafoType is derived by

trafoType = ( ( nW = = nH ) ? 0 : ( ( nW > nH ) ? 1 : 2 ) )	(8-2)

The scaled transform coefficient dij with i = 0..nW − 1, j = 0..nH − 1 is derived as follows.

–	If scaling_list_present_flag is equal to 0,

yij = Clip3( −32768, 32767, cij )		(8-3)

dij = Clip3( −32768, 32767,( ( yij * levelScale[ qP%6 ] << ( qP/6 ) ) + ( 1 << ( shift − 1 ) ) ) >> shift )	(8-4)

–	Otherwise, if shiftScale is greater than 0,

yij = Clip3( −32768, 32767, cij )		(8-5)

dij = Clip3( −32768, 32767, ( yij * M[ i ][ j ]*levelScale[ qP%6 ] + 					( 1 << ( shiftScale − 1 ) ) ) >> shiftScale )	(8-6)

–	Otherwise,

yij = Clip3( -LevelLimit, LevelLimit − 1, cij )		(8-7)

dij = Clip3( −32768, 32767, ( yij * M[ i ][ j ]*levelScale[ qP%6 ] ) << ( -shiftScale ) )	(8-8)

# Patent rights declaration(s)

The British Broadcasting Corporation (BBC) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, are prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).