| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0569 WG11 Number: m25193 |
| --- | --- |

| Title: | Cross-check of deblocking in I-PCM regions (JCTVC-I0035) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Cross-verification |  |  |
| Author(s) or Contact(s): | Andrey Norkin Färögatan 6 164 80 Stockholm Sweden | Tel: Email: | +46 10 714 84 02 andrey.norkin@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

The contribution is a cross-verification of proposal I0035 on I_PCM deblocking scheme. The cross-verifier confirms perfect match in BD-rate for the additional results (on the Sandstorm sequence) and similar run-times.

# Cross-check results

The cross-checker performed verification for the additional results, provided for the synthesized sequence. The results on the synthesized Sandstorms sequences were obtained for Anchor, Scheme 1 with pcm_loop_filter_disable = 1, Scheme 1 with pcm_loop_filter_disable = 0, Scheme 2 with QP=0 signaling and Scheme 2 with QP=0 signaling at slice level. The cross-checker reports perfect match for BDR results

The encoding and decoding time are approximately in the same range as those of the proponent with some small variation which are probably cause by the simulation environment. The obtained results are shown in Table 1.

Table 1 Comparison of  JCTVC-I0035 vs HM6.0 with IPCM bug fix as anchor

|  | Random access main (QP=0, 6, 10, 14) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Rate kbps | Y dB | U dB | V dB | Enc Time % | Dec Time % |
| Scheme 1 | -0.05 | 0.00 | -4.10 | -4.10 | 102 | 103 |
| Scheme 2 | 14.70 | 0.00 | -4.10 | -4.10 | 100 | 96 |
| Scheme 1 with pcm_loop_filter_disable=1 | 0.00 | 0.00 | 0.00 | 0.00 | 100 | 98 |
| Scheme 2 with QP=0 signaling | 19.47 | 0.00 | 0.00 | 0.00 | 100 | 103 |

.

# References

- K. Chono, G. Van der Auwera, M. Karczewicz, “AHG6: Report on unified QP derivation process in deblocking of I_PCM regions”, JCTVC-I0035, Geneva, April 2012.

- Gisle Bjontegaard, “Calculation of average PSNR differences between RD curves,” ITU-T SC16/Q6, VCEG-M33, Austin, USA, Apr 2001.