| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, CH, 27th April – 7th May, 2012 | Document: JCTVC-I0572 WG11 Number: m25198 |
| --- | --- |

| Title: | Crosscheck for APS Error Resilience and Partial Updating (JCTVC-I0069) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Sachin Deshpande 5750 NW Pacific Rim Blvd Camas, WA 98607 | Tel: Email | +1-360-817-8486 sdeshpande@sharplabs.com |
| Source: | SHARP |  |  |

_____________________________

# Abstract

In this document crosscheck results for JCTVC-I0069 “APS Error Resilience and Partial Updating” [1] are reported. Nokia provided software. The verification task has been done successfully and the encoder decoder outputs match as was reported by the proponents.

# Cross-check

JCTVC-I0069 discusses shortcomings of HEVC CD with regards to APS error resiliency and partial updating.

- It is asserted that HEVC CD provides no means to detect a loss of an adaptation parameter set (APS) NAL unit, if the aps_id value of the lost APS NAL unit has been used previously by another APS NAL unit. The contribution proposes constraints on aps_id assignment that reportedly enable decoders to detect losses of APS NAL units.

- The contribution also proposes a partial updating mechanism for adaptation parameter sets. The proposed syntax includes a possibility to copy any group of syntax elements (QM, deblocking filter, SAO, ALF) from either the same reference APS or from different APSes.

Nokia provided source code, with ability to run the code using the proposed technology related to partial updating mechanism mentioned above.

The code base used was HM 6 .0.

We visually inspected source code differences between original HM 6.0 source code and the source code from proponents.

The main changes include:

- Changes to the encoder and decoder to support new proposed syntax in the contribution section 3 [1] related to adaptation parameter set.

- The main chunk of code in TDecCAVLc.cpp checks common_reference_aps_flag flag in APS and if that flag is set identifies the common_reference_aps_id to use for copying of APS parameters. Then various individual sections check flag in it to decide to copy parameters from corresponding APS. For example scaling list section checks aps_scaling_list_data_referenced_flag to determine if parameters for that section should be copied. Then the actual copy operation is conducted by other code section.

# Results

We compiled the code and ran it for the WQVGA Basketball Pass sequence, with RA main configuration with resolution 416x240, at 30 fps. The number of frames encoded was 32. We found that the encoder and decoder results match for this.

# Conclusion

Crosscheck results for JCTVC-I0069 “APS Error Resilience and Partial Updating” are reported. We visually inspected the software against the proposed syntax. We also ran the encoder and decoder. We found the results to match.

# References

- M.M. Hannuksela, S. M. Gopalkrishna, "APS Error Resilience and Partial Updating," 9th Meeting: Geneva, CH, 27th April – 7th May, 2012, JCTVC-I0069