| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0177 |
| --- | --- |

| Title: | Unified context derivation with CABAC context reduction for last position coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Feng Zou, Oscar C. Au, Jingjing Dai, Xingyu Zhang Chao Pang The Hong Kong University of Science and Technology, Clear Water Bay, Hong Kong | Tel: Email: | +852-51328827 fengzou@ust.hk eeau@ust.hk jjdai@ust.hk eexyzhang@ust.hk pcece@ust.hk |
| Source: | The Hong Kong University of Science and Technology |  |  |

_____________________________

# Abstract

- In this proposal, a modification of JCTVC-I0331 is proposed. JCTVC-I0331 uses two formulas to compute m and n respectively. In this proposal, m is kept unchanged and n is modified slightly. With the slight modification, the advantages of JCTVC-I0331 (unified luma/chroma operation, formula to compute m and n) are retained. We have an added advantage that the context number is reduced from 15 to 13 for each direction.. It is reported that the proposed approach provides negligible loss over the HM6.0 in all common test coding conditions.

# Introduction

In the JCT-VC San Jose meeting in Feb, 2012, there is one technique JCTVC-H0537 [1] adopted, which is last significant (none zero) coefficient position coding related, comprising horizontal position x and vertical position y. The last significant coefficient position is represented by a binary string which consists of two parts. The first part is truncated unary code where each bin is encoded with CABAC contexts, while the second part is fixed binary code with by-pass coding (no context).

For each bin in the truncated unary codeword, the CABAC context assignment is shown in Table 1.

Table 1. The context assignment for each bin in the truncated unary codeword for Luma.

In order to get the CABAC context for each bin for different TU sizes, a 1D table is used to store the CABAC context index in HM6.0 shown in Table 6. The relationship between the entries in the 1D table and the CABAC context index for each bin (with context) of different TU sizes is shown as follows (for horizontal position x/ vertical position y):

For last_significant_coeff_x_prefix, ctxIdxInc is derived as follows.

ctxIdxInc =  lastCtx[( ( log2TrafoWidth – 2 )*( log2TrafoWidth + 1 ) ) + binIdx ]

For last_significant_coeff_y_prefix, ctxIdxInc is derived as follows.

ctxIdxInc =  lastCtx[ ( ( log2TrafoHeight – 2 )*( log2TrafoHeight + 1 ) ) + binIdx ]

Table 2 . 1D table to store the CABAC context index for each bin of different TU sizes in HM6.0

- In JCTVC-I0331, an equation based CABAC context derivation is proposed to replace the LUT (look-up table) as follows:

For last_significant_coeff_x_prefix, ctxIdxInc is derived as follows.

ctxIdxInc =  m+ (binIdx>>n)

For last_significant_coeff_y_prefix, ctxIdxInc is derived as follows.

ctxIdxInc =  m+ (binIdx>>n)

where  m = 3* k + ((k+1)>>2)                                                  (1)

n = (k+3)>>2,

k= log2(TrafoWidth)-2 for last_significant_coeff_x_prefix, or

k= log2(TrafoHeight)-2 for last_significant_coeff_y_prefix.

The resulting CABAC assignment is shown in Table 3.

Table 3. CABAC context assignment in JCTVC-I0331

| Bin index | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TU 4x4 | 0 | 1 | 2 |  |  |  |  |  |  |
| TU 8x8 | 3 | 3 | 4 | 4 | 5 |  |  |  |  |
| TU 16x16 | 6 | 6 | 7 | 7 | 8 | 8 | 9 |  |  |
| TU 32x32 | 10 | 10 | 11 | 11 | 12 | 12 | 13 | 13 | 14 |

# Proposed Method

Instead of using a LUT(look-up table) to obtain the CABAC context, we employ the equation based CABAC context derivation, and we further reduce the CABAC contexts from 15 to 13 by allowing more than 2 bins for TU32x32 to share the same context.

| Table 4. Proposed CABAC context assignment Bin index | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TU 4x4 | 0 | 1 | 2 |  |  |  |  |  |  |
| TU 8x8 | 3 | 3 | 4 | 4 | 5 |  |  |  |  |
| TU 16x16 | 6 | 6 | 7 | 7 | 8 | 8 | 9 |  |  |
| TU 32x32 | 10 | 10 | 10 | 10 | 11 | 11 | 11 | 11 | 12 |

For last_significant_coeff_x_prefix, ctxIdxInc is derived as follows.

ctxIdxInc =  m+ (binIdx>>n)

For last_significant_coeff_y_prefix, ctxIdxInc is derived as follows.

ctxIdxInc =  m+ (binIdx>>n)

where  m = 3* k + ((k+1)>>2)                                                  (1)

n = (k+1)>>1,

k= log2(TrafoWidth)-2 for last_significant_coeff_x_prefix, or

k= log2(TrafoHeight)-2 for last_significant_coeff_y_prefix.

# Results

We implemented the technique into the HM6.0 [2] software and tested with the common test condition [3].

The following table shows the result of the proposed scheme.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.06% | 0.00% | 0.05% | 0.07% | 0.01% | 0.05% |
| Class B | 0.00% | 0.02% | 0.00% | 0.00% | 0.00% | 0.01% |
| Class C | 0.00% | -0.01% | 0.06% | 0.00% | 0.03% | 0.01% |
| Class D | 0.01% | 0.05% | 0.01% | 0.02% | 0.05% | 0.01% |
| Class E | 0.02% | 0.03% | 0.06% | 0.03% | 0.04% | 0.07% |
| Class F | 0.00% | 0.06% | 0.04% | 0.03% | 0.11% | 0.16% |
| Overall | 0.02% | 0.03% | 0.04% | 0.02% | 0.04% | 0.05% |
|  | 0.02% | 0.02% | 0.03% | 0.02% | 0.03% | 0.05% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.15% | 0.15% | 0.08% | 0.12% | 0.29% | -0.05% |
| Class B | 0.03% | -0.10% | 0.06% | 0.04% | -0.13% | 0.04% |
| Class C | -0.01% | -0.02% | -0.17% | 0.01% | -0.06% | -0.11% |
| Class D | -0.04% | -0.12% | -0.03% | 0.06% | 0.04% | 0.10% |
| Class E |  |  |  |  |  |  |
| Class F | -0.04% | -0.06% | 0.04% | 0.02% | 0.04% | 0.04% |
| Overall | 0.02% | -0.03% | 0.00% | 0.05% | 0.03% | 0.00% |
|  | 0.01% | -0.03% | -0.02% | 0.05% | 0.03% | 0.00% |
| Enc Time[%] | 102% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.06% | -0.09% | 0.25% | 0.08% | 0.06% | 0.33% |
| Class C | 0.05% | -0.07% | 0.05% | 0.01% | 0.00% | 0.26% |
| Class D | 0.08% | 0.07% | -0.34% | -0.01% | 0.42% | -0.06% |
| Class E | -0.02% | -0.73% | 0.86% | 0.06% | 0.89% | 0.46% |
| Class F | -0.10% | -0.11% | -0.52% | 0.16% | -0.72% | 0.12% |
| Overall | 0.02% | -0.15% | 0.03% | 0.06% | 0.09% | 0.21% |
|  | 0.02% | -0.08% | 0.03% | 0.06% | 0.11% | 0.18% |
| Enc Time[%] | 98% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

# Conclusion

The proposed technique uses the equation based context derivation. With little RD performance change and reduced complexity, it is recommended to be adopted in the standard.

# Patent rights declaration(s)

The Hong Kong University of Science and Technology may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1] Liwei Guo, Marta Karczewicz, Wei-Jung Chien, Xianglin Wang. “CABAC Contexts Reduction for Last Position Coding” JCT-VC Document, JCTVC-H0537, Feb. 2012

[2] HM 6.0 Software, http://hevc.kw.bbc.co.uk/trac/browser/tags/HM-6.0

[3] F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-H1100, Feb. 2012.

# WD Changes based on JCTVC-H1003 V22

Scheme1:

##### 9.2.3.1.2 Derivation process of ctxIdxInc for the syntax elements last_significant_coeff_x_prefix and last_significant_coeff_y_prefix

Inputs to this process are the binIdx, the colour component index cIdx, the transform block width log2TrafoWidth and the transform block height log2TrafoHeight.

Output of this process is ctxIdxInc.

Table 9-1 – Specifcation of lastCtx[ i ]

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| lastCtx[ i ] | 0 | 1 | 2 | 2 | 3 | 4 | 5 | 5 | 2 | 2 | 6 | 7 | 8 | 8 |
| i | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 |
| lastCtx[ i ] | 9 | 9 | 2 | 2 | 10 | 11 | 12 | 12 | 13 | 13 | 14 | 14 | 2 | 2 |

The vector lastCtx[ i ] is specified in Table 9-1 and ctxIdxInc is derived as follows.

When cIdx is equal to 0,

–	For last_significant_coeff_x_prefix, ctxIdxInc is derived as follows.

ctxIdxInc =  lastCtx[( ( log2TrafoWidth − 2 )*( log2TrafoWidth + 1 ) ) + binIdx ]	(9-1)

–	For last_significant_coeff_y_prefix, ctxIdxInc is derived as follows.

ctxIdxInc =  lastCtx[ ( ( log2TrafoHeight − 2 )*( log2TrafoHeight + 1 ) ) + binIdx ]	(9-2)

When cIdx is greater than 0, ctxIdxInc is modified as follows.

–	For last_significant_coeff_x_prefix, ctxIdxInc is derived as follows.

ctxIdxInc =  (binIdx >>( log2TrafoWidth − 2 ) )+ 15	(9-3)

–	For last_significant_coeff_y_prefix, ctxIdxInc is derived as follows.

ctxIdxInc =  (binIdx >>( log2TrafoWidth − 2 ) )+ 15	(9-4)

Output of this process is ctxIdxInc,

ctxIdxInc = m + (binIdx>>n)                                                     (9-8)

When cIdx is equal to 0, m and n are derived as follows

m = 3* log2TrafoSizeMinus2 + ((log2TrafoSizeMinus2+1)>>2)                          (9-9)

n = (log2TrafoSizeMinus2+1)>>1                                                  (9-10)

When cIdx is greater than 0, m and n are derived as follows.

m = 15                                                                        (9-11)

n = log2TrafoSizeMinus2                                                         (9-12)

where

–	for last_significant_coeff_x_prefix, log2TrafoSizeMinus2 is derived as follows

log2TrafoSizeMinus2 = log2TrafoWidth – 2

–	for last_significant_coeff_y_prefix, log2TrafoSizeMinus2 is derived as follows

log2TrafoSizeMinus2 = log2TrafoHeight - 2