| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0576 |
| --- | --- |

| Title: | BoG report on SAO |  |  |
| --- | --- | --- | --- |
| Status: | Input Document of JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yu-Wen Huang |  | yuwen.huang@mediatek.com |
| Source: | BoG on SAO |  |  |

_____________________________

# Recommendations

- Adopt the interleaving mode part of JCTVC-I0563 (RDO bug-fix)

- Adopt the interleaving mode part of JCTVC-I0066 (clean-up of syntax)

- Adopt JCTVC-I0168, also in JCTVC-I0162 and JCTVC-I0073 (bypass coding for BO sign, unify EO and BO magnitude coding)

- Adopt JCTVC-I0172 (avoid merging across slice or tile boundaries)

- Remove the interleaving flag in the slice header

- Six proposals, JCTVC-I0073, JCTVC-I0137, JCTVC-I0162, JCTVC-I0184, JCTVC-I0199, and JCTVC-I247, will be revisited later after visual testing.

  - Block artifacts: JCTVC-I0137, JCTVC-I247

    - These two proposals will be further studied for one more meeting cycle if the issue is confirmed to be serious.

  - BD-rate improvements: JCTVC-I0184, JCTVC-I0199

    - These two proposals will be further discussed if there is no visual quality drop.

  - Memory reduction: JCTVC-I0073, JCTVC-I0162

    - These two proposals will be further discussed if there is no visual quality drop.

# Reviewed proposals

## JCTVC-I0377, CE1 Phase1 Test5.1 [Motorola, Samsung]

- For interleaving mode, four edge offset classes are reduced to only one; offset index equal to zero corresponds to non-zero offset value; pixel processing is described as follows.

- C: current pixel before applying EO
L: one of the two neighbors
R: one of the two neighbors
O: offset
V: current pixel after applying EO

- If 2C>(L+R), I=(L+R)>>1
If 2C<(L+R), I=(L+R+1)>>1
If 2C=(L+R), I=C

- When C!=I,
if O applied to C brings the pixel value away from I, V=C+O
if O applied to C brings the pixel value towards I but does not pass I, V=C+O
if O applied to C brings the pixel value towards I and it passes I, V=I

- When C=I, V=C

- Purpose: Coding efficiency improvement

| Anchor: HM-6.0 SAO interleaving mode with CE1 changes | Overall BD-Rate w/o Class F |  |  | Class F BD-Rate |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y (%) | Cb (%) | Cr (%) | Y (%) | Cb (%) | Cr (%) |
| Main-LDB, LCU=64x64 | 0.4 | -0.5 | -0.7 | 0.4 | -0.5 | 0.0 |
| HE10-LDB, LCU=64x64 | -0.2 | -1.7 | -2.0 | -0.8 | -0.5 | -1.2 |
| Main-LDB, LCU=32x32 | 0.4 | -1.4 | -1.7 | 0.0 | -0.2 | -0.9 |
| HE10-LDB, LCU=32x32 | -0.3 | -0.8 | -0.8 | -0.9 | -1.1 | -2.4 |
| Main-LDB, LCU=16x16 | 0.4 | 0.6 | 0.8 | 0.0 | -2.2 | -1.6 |
| HE10-LDB, LCU=16x16 | -0.1 | 0.3 | 0.6 | -0.7 | -1.9 | 0.0 |

- The crosschecker showed concern about complexity

- Line buffer increase from 2 bits per pixel to 8/10 bits per pixel depending on Main/HE10

- No support from non-proponents

## JCTVC-I0378, CE1 Phase1 Test6.1 [Motorola, Samsung]

- For interleaving mode, send N offsets in BO (N=4 for luma and N=2 for chroma) where each offset applies to a set of pixels values (offset bands). The N offset bands are derived from the statistics of a subset of pixels inside the LCU, where the subset of pixels are not affected by deblocking.

- Purpose: Coding efficiency improvement

| Anchor: HM-6.0 SAO interleaving mode with CE1 changes | Overall BD-Rate w/o Class F |  |  | Class F BD-Rate |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y (%) | Cb (%) | Cr (%) | Y (%) | Cb (%) | Cr (%) |
| Main-LDB, LCU=64x64 | 0.2 | -1.1 | -1.9 | 2.8 | 0.4 | 3.6 |
| HE10-LDB, LCU=64x64 | 0.1 | -1.0 | -1.4 | 0.9 | 0.2 | 2.3 |
| Main-LDB, LCU=32x32 | 0.1 | -1.2 | -1.8 | 1.7 | -0.2 | 1.2 |
| HE10-LDB, LCU=32x32 | 0.0 | -0.4 | -0.6 | 0.4 | -1.5 | 0.2 |
| Main-LDB, LCU=16x16 | 0.0 | -0.1 | 0.1 | 0.4 | -0.5 | -0.1 |
| HE10-LDB, LCU=16x16 | 0.0 | 0.0 | 0.1 | 0.2 | -1.2 | -0.7 |

- No gain (some loss) based on problematic RDO

- No support from non-proponents

## JCTVC-I0563, Phase2 S5 [MediaTek, Motorola, Qualcomm, Samsung, TI]

- For APS mode, fix rate estimation of an edge offset in RDO, which was signed and should be modified to unsigned

- For interleaving mode, fix rate estimation in RDO, which was VLC and should be modified to CABAC.

- Purpose: Bug-fix

| Anchor: SAO-off | S2 (APS) |  |  | S5 (APS) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y (%) | Cb (%) | Cr (%) | Y (%) | Cb (%) | Cr (%) |
| 64x64, all conditions, all classes | -2.6 | -5.0 | -6.2 | -2.6 | -4.9 | -6.2 |
| 32x32, all conditions, all classes | -2.6 | -5.3 | -6.6 | -2.6 | -5.3 | -6.6 |
| 16x16, all conditions, all classes | -2.5 | -7.2 | -9.2 | -2.5 | -7.1 | -9.2 |

| Anchor: SAO-off | S3 (interleaving) |  |  | S5 (interleaving) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y (%) | Cb (%) | Cr (%) | Y (%) | Cb (%) | Cr (%) |
| 64x64, all conditions, all classes | -2.3 | -3.1 | -4.0 | -2.3 | -3.5 | -4.4 |
| 32x32, all conditions, all classes | -1.3 | -0.5 | -0.8 | -1.4 | -0.8 | -1.2 |
| 16x16, all conditions, all classes | 0.2 | 0.7 | 0.7 | 0.1 | 0.7 | 0.6 |

- These are reasonable things to do, even though the coding efficiency improvement is small.

- Recommendation: adopt the interleaving mode part

## JCTVC-I0066 [Mitsubishi]

- For APS mode, replace exponential Golomb codes by truncated unary codes for edge offsets.

- Short length results show no BD-rate differences.

- For interleaving mode, replace unary binarization by truncated unary binarization for edge offsets.

Short length results show no BD-rate differences.

- One expert supported this.

- This is a clean-up.

- Recommendation: adopt the interleaving mode part

## JCTVC-I0073 [Ericsson]

- This proposal reduces memory for offsets by 75% and requires no clipping in SAO filtering process.

- For BO, keep 5-bit band position, double band width, send only one offset for the band indicated by the band position, and the coded offset is always nonzero and signed with a power-of-two magnitude.

The offset is restricted to keep SAO filtered value within allowed range of pixel values.

Compared with CE1 anchor, luma BD-rate is 0%, and chroma BD-rate is -0.3%.

- For EO, restrict offsets to 0 or (1<<(bitDepth-8)) when edgeIdx is equal to 0 or 1, and restrict edge offsets to 0 or –(1<<(bitDepth-8)) when edgeIdx is equal to 3 or 4.

Due to restricted offsets filtering with EO do not overshoot or undershoot edges, e.g. valley cannot turn into a hill.

- When the EO modification is combined with the BO modification, compared with CE1 anchor, luma BD-rate is 0%, and chroma BD-rate is -1.2%.

- Crosschecker had concern about visual quality. The proponent answered it is similar to HM-6.0.

- Two experts wanted to see the visual quality of this proposal.

- There are several nontrivial modifications in this proposal, and one expert expressed that these changes need to be studied one by one more carefully.

- Action: Conduct visual testing for this proposal and then decide later

## JCTVC-I0130 [Samsung]

- For APS mode, removing one unit flag suffers 0-0.1% coding efficiency loss.

- For APS mode, removing repeat row flag suffers 0-0.5% coding efficiency loss.

- For APS mode, removing one unit flag and repeat row flag suffers 0.1-0.7% coding efficiency loss.

- This proposal is not relevant any more since one of the CE1 decisions is to remove the APS mode.

### JCTVC-I0137 [Panasonic]

- This proposal is highly related to CE1 Test3.x.

- For BO, add a weighted offset to reconstructed samples along with LCU and band boundaries.

- The proponent requested to skip subjective testing for CE1 Test3.2, CE1 Test3.3, and CE1 Test3.4 and to conduct subjective testing for CE1 Test3.1 and JCTVC-I0137 in the 9th JCT-VC meeting.

- The crosscheckers expressed that the reported issue is not serious and the complexity of the solution is too high.

- Extra line buffers are needed.

- Action: Conduct visual testing to evaluate how serious this issue is and how this proposal solves it.

## JCTVC-I0162 [Samsung, Qualcomm, MediaTek]

- For interleaving mode, on top of CE1 Test2.2.1 (i.e., send one explicit edge offset per component per LCU and derive the rest three edge offsets), send only one explicit band offset, where the coded band offset can only be nonzero.

- On average, compared with CE1 anchor, luma BD-rate is 0.0%, -0.3%, and -0.4% when the LCU size is 64x64, 32x32, and 16x16, respectively, without averaging class F.

- Class E and class F sometimes have large losses (up to 1.5%) when the LCU size is 64x64.

- Two experts showed concern about class E loss (0.9%) in Main-LDB and class B loss (0.2%) in Main-RA.

- Lambda value is changed in this proposal, but the same change to the anchor is additional tested (no big change in BD-rate results when the anchor is changed).

- One expert supported to test JCTVC-I0162 on top of JCTVC-I0199.

- One expert expressed that always deriving offset of category 2-3 by dividing offset of category 1 & 4 may not be helpful for all sequences

- Two experts said the ringing artifact becomes more obvious than HM-6.0.

- How much memory is saved? When 4K is considered, offset line buffer is 1536 bytes, and 75% of the line buffer can be saved by this proposal.

- Visual testing is needed before adopting this.

- Visual testing is needed first.

- Action: Conduct visual testing for this proposal to see if it is necessary to further test it on top of other proposal

## JCTVC-I0168 [MediaTek]

- In HM-6.0, after removing signs for edge offsets, CABAC initialization, binarization, and context formation processes for BO are different from those for EO.

- In CD, both BO and EO are described by a same syntax table and share the same CABAC processes.

- Changing CD according to HM-6.0 needs to change hundreds of lines in CD, and nobody fixed it.

- It is proposed to decouple the BO sign from sao_offset, group BO signs of one LCU, code BO signs with bypass coding, and unify BO magnitude with unsigned EO, which changes 8 lines in HM-6.0 and 23 lines in CD.

- Results show no BD-rate differences.

- If band offset magnitude and edge offset share the contexts, two contexts are reduced.

- Two experts supported this to fix the mismatch between CD and HM.

- JCTVC-I0162 also contains this change.

- Recommendation: adopt

## JCTVC-I0171 [Panasonic]

- This proposal is highly related to CE1 Phase1 Test4.2.

- For APS mode, change LCU-based signaling to region-based signaling, where the number of regions in picture width and the number of regions in picture height are signaled.

- The concept of this proposal is the same as CE1 Test4.2. Only syntax design is a bit different.

- This proposal is not relevant any more since one of the CE1 decisions is to remove the APS mode.

### JCTVC-I0172 [MediaTek]

- Disallow merging across slice or tile boundaries.

- Two experts supported this.

- Recommendation: adopt

### JCTVC-I0183 [Canon]

- Send Cr offsets and derive Cb offsets from Cr offsets

- Compared with CE1 anchor, coding gain is 0.2%, 0.9%, and 1.5% for Y, Cb, and Cr, respectively, when LCU size 64x64.

- The crosschecker confirmed that results are correct.

- Results show consistent gain in all cases, which is very good.

- Need subjective testing

- May not work for different color transforms

- If this is switchable, it will probably need multi-pass encoding, resulting in no complexity reduction.

- No support from non-proponents

### JCTVC-I0184 [Canon]

- Non-normative and encoder-only

- Turn on/off the SAO tool for the current picture based on a statistical criterion of the previous picture.

- Can reduce average complexity by skipping SAO encoding processes for some pictures

- Compared with CE1 anchor, the BD-rate is -0.5%/-0.4%/-0.3% for Y/Cb/Cr when the LCU size is 64x64; the BD-rate is -1.6%/-1.5%/-1.4% for Y/Cb/Cr when the LCU size is 16x16.

- Need to test RA and LDP

- One expert suggested putting this in common test condition if RA and LDP results are good and reconfirm the visual quality in CE (if this harms visual quality, it should be disabled).

- One expert requested to test this proposal visually in this meeting.

- One expert expressed that non-normative proposals should have less priority (it can be applied anytime).

- One expert showed concern that disabling the entire picture based on previous picture may harm visual quality.

- One expert expressed that we should revisit this proposal after all data are available.

- Action: Test JCTVC-I0184 objectively and visually, additionally test JCTVC-I0184+JCTVC-I0199 objectively; release software and results to all CE1 members and this BoG members

## JCTVC-I0185 [Canon]

- This proposal is highly related to CE1 Phase2 S6 (i.e., AHG6 Test2, JCTVC-I0261 [TI]).

- Use an index to represent four edge offsets.

- 16 sets of edge offsets are allowed, and 4-bit fixed length coding is used for the index.

- Results of JCTVC-I0185 are very close to those of JCTVC-I0261 and are as follows.

| Anchor: HM-6.0 SAO interleaving mode with CE1 changes | Overall BD-Rate w/o Class F |  |  | Class F BD-Rate |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y (%) | Cb (%) | Cr (%) | Y (%) | Cb (%) | Cr (%) |
| Main-LDB, LCU=64x64 | 0.1 | -1.3 | -1.5 | 0.1 | -0.9 | -0.5 |
| HE10-LDB, LCU=64x64 | -0.2 | -1.0 | -1.3 | -0.3 | -0.1 | -0.5 |
| Main-LDB, LCU=32x32 | 0.1 | -1.0 | -1.2 | -0.1 | -0.4 | -0.6 |
| HE10-LDB, LCU=32x32 | -0.3 | -0.4 | -0.4 | -0.4 | -1.0 | -1.5 |
| Main-LDB, LCU=16x16 | 0.3 | 0.6 | 0.5 | 0.0 | -1.4 | -1.1 |
| HE10-LDB, LCU=16x16 | -0.1 | 0.2 | 0.4 | -0.5 | -1.4 | 2.4 |

- This proposal has no memory reduction.

- Gains are not big enough to be adopted even if the visual quality is the same as HM-6.0.

## JCTVC-I0199 [MediaTek, TI, Qualcomm]

- This proposal is highly related to CE1 Phase2 S3.

- JCTVC-I0199 was proposed for improving S3. Instead of being first jointly coded with an index, each of the three SAO LCU on/off flags of three color components is coded separately with a neighbor-based context model design before corresponding merge flags are coded. The efficient merge flag coding (i.e., avoid sending merge-left or merge-up flag when the left or upper neighbor is off) in S3 is still applied.

- The index design of three color components’ on/off flags in S3 was trained using common test conditions and might not work well for all sequences in all conditions, while JCTVC-I0199 can let CABAC automatically fit statistics of each slice.

- Average results of Main-AI/RA/LDB/LDP and HE10-AI/RA/LDB/LDP for three different LCU sizes are summarized as follows. Anchor is SAO-off.

|  | S3 |  |  | JCTVC-I0199, 2 ctx models |  |  | JCTVC-I0199, 3 ctx models |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y (%) | Cb (%) | Cr (%) | Y (%) | Cb (%) | Cr (%) | Y (%) | Cb (%) | Cr (%) |
| 64x64 | -2.3 | -3.1 | -4.0 | -2.4 | -3.2 | -4.0 | -2.4 | -3.2 | -4.1 |
| 32x32 | -1.3 | -0.5 | -0.8 | -1.2 | -0.5 | -0.8 | -1.3 | -0.6 | -0.9 |
| 16x16 | 0.2 | 0.7 | 0.7 | 0.7 | 1.2 | 1.2 | 0.7 | 1.1 | 1.2 |

- One expert questioned the parsing dependency issue. As discussion, the issue does not seem to be serious.

- One expert showed concern for two additional contexts, but one expert said this complexity increase is worth the gain.

- One expert showed concern that putting the SAO on/off in the beginning would affect the effectiveness of other tools; however, this is generally true for every tools.

- Picture-based optimization might benefit from merge-first in another competing proposal JCTVC-I0507, but no data is shown.

- The crosschecker said this proposal is better than JCTVC-I0193.

- One expert expressed that this might change the visual quality, but one expert said this proposal does not visually quality.

- Action: Conduct subjective viewing for this proposal, make decision later until results of JCTVC-I0184, JCTVC-I0199, and JCTVC-I0184+JCTVC-I0199 are all obtained.

## JCTVC-I0200 [TI]

- Non-normative and encoder-only.

- For interleaving mode, using pre-deblocked pixels of the bottom rows and right columns in a current LCU during parameter estimation.

| Anchor: HM-6.0 SAO interleaving mode with CE1 changes | Overall BD-Rate w/o Class F |  |  | Class F BD-Rate |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y (%) | Cb (%) | Cr (%) | Y (%) | Cb (%) | Cr (%) |
| Main-LDB, LCU=64x64 | -0.1 | -0.9 | -1.1 | -0.4 | -1.5 | -1.7 |
| HE10-LDB, LCU=64x64 | -0.1 | -0.6 | -0.8 | -0.3 | -0.7 | -1.3 |
| Main-LDB, LCU=32x32 | -0.3 | -1.2 | -1.4 | -0.9 | -0.6 | -2.9 |
| HE10-LDB, LCU=32x32 | -0.3 | -0.4 | -0.3 | -0.7 | -1.6 | -3.2 |
| Main-LDB, LCU=16x16 | -0.3 | -0.1 | 0.0 | -1.4 | -2.2 | -1.4 |
| HE10-LDB, LCU=16x16 | -0.2 | -0.1 | 0.1 | -1.4 | -2.2 | -2.1 |

## JCTVC-I0246 [Sony, Motorola, Samsung]

- A 1-bit flag is used to signal the SAO type (i.e. BO or EO), which is the same as CE1 Phase2 S7.

- The SAO on/off switch is also coded using a 1-bit flag with a separate context.

- The rest of the SAO type is coded using fixed length codes and bypass mode in CABAC.

- Full results are not yet obtained.

- Preliminary results show very tiny gains (0.01-0.04%).

- Crosscheck is JCTVC-I0540, but the report was not available yet.

- One expert expressed that bypass coding for the rest SAO type has some benefit for CABAC

- CD texts were not yet provided.

## JCTVC-I0247 [Sony]

- This proposal is highly related to CE1 Phase1 Test3.x.

- Reduce offset differences at band boundaries.

- Reduce offset differences at LCU boundaries.

- The claimed visual quality improvement has to be verified by subjective testing.

- BD-rate results are as follows.

| Anchor: HM-6.0 SAO APS mode for RA and interleaving mode with CE1 changes for LDB | Overall BD-Rate w/o Class F |  |  | Class F BD-Rate |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y (%) | Cb (%) | Cr (%) | Y (%) | Cb (%) | Cr (%) |
| Main-RA, LCU=64x64 | -0.1 | -0.1 | -0.1 | -0.1 | 0.0 | 0.0 |
| HE10-RA, LCU=64x64 | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 | 0.2 |
| Main-LDB, LCU=64x64 | 0.0 | 0.1 | 0.3 | -0.2 | 0.2 | 0.3 |
| HE10-LDB, LCU=64x64 | 0.0 | -0.2 | -0.1 | -0.1 | 0.5 | -0.2 |
| Main-LDB, LCU=32x32 | 0.0 | 0.1 | -0.1 | -0.2 | 0.4 | 0.3 |
| HE10-LDB, LCU=32x32 | 0.0 | 0.2 | 0.0 | 0.0 | -0.3 | 1.0 |
| Main-LDB, LCU=16x16 | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | -0.4 |
| HE10-LDB, LCU=16x16 | 0.0 | -0.1 | 0.1 | 0.0 | 0.5 | 0.8 |

- How does the encoder perform RDO? The proponent answered that it is the same as HM-6.0. However, the crosscheker did not think so.

- This proposal needs additional buffers to perform reconstruction.

- CD texts were not provided.

- Action: Conduct visual testing to evaluate how serious this issue is and how this proposal solves it.

## JCTVC-I0292 [Sharp]

- For interleaving mode, move SAO data from the beginning of each LCU to the end of each LCU.

- Not compatible with fine granularity slices (i.e., non-LCU-aligned slices)

- Beneficial on the encoder side (no LCU storage before SAO parameters are sent)

- No benefit on the decoder side, might be harmful for CU-based pipelining on the decoder side

- For fine granularity slices, if one LCU has multiple slices, disable SAO entirely for the LCU and do not send any SAO parameter

- No support from non-proponents

## JCTVC-I0295 [Samsung]

- Propose to add signs back for edge offsets in order to allow sharpening.

- An additional offset restriction is applied to avoid salt and pepper noise.

- Almost no change in BD-rate

- Needs visual testing

- The crosschecker expressed that adding sign back may confuse people.

- No support from non-proponents

## JCTVC-I0380 [Motorola, Samsung]

- Non-CE1: On SAO syntax unification

- Withdrawn

## JCTVC-I0384 [MediaTek]

- Send SAO enable flag before interleaving flag in both APS and slice header

- This proposal is not relevant any more since one of the CE1 decisions is to remove the APS mode.

- Recommendation: The interleaving flag in the slice header should be removed in HM-7.0.

## JCTVC-I0507 [Motorola, Qualcomm]

- First check for joint merge left

- If no merge-left, check joint merge-up

- If no merge-up, send on/off and offsets

- Late proposal

- Not yet crosschecked

- According to the proponent, when JCTVC-I0199 is anchor, JCTVC-I0507 is 0.2% worse than JCTVC-I0199 when LCU size is 64x64, is 0.1% worse than JCTVC-I0199 when LCU size is 32x32, is 0.8% better than JCTVC-I0199. JCTVC-I0507 uses 3 fewer contexts than JCTVC-I0199 and 2 fewer contexts than HM-6.0.

- JCTVC-I0184 may make this proposal unnecessary (not yet proved)

- One expert expressed that JCTVC-I0199 should be considered first because it was released before the document upload deadline, while this one is pretty late.

- No support from non-proponents

# List of e-mail receivers

| Company | Name | Email |
| --- | --- | --- |
| Broadcom | Wade Wan Peisong Chen Tim Hellman Yong Yu | wwan@broadcom.com peisongc@broadcom.com timhellman@broadcom.com yongyu@broadcom.com |
| Canon | Edouard Francois Patrice Onno Guillaume Laroche Christophe Gisquet | edouard.francois@crf.canon.fr patrice.onno@crf.canon.fr guillaume.laroche@crf.canon.fr Christophe.Gisquet@crf.canon.fr |
| Cisco | Arild Fuldseth Thomas Davies | arilfuld@cisco.com thdavies@cisco.com |
| eBrisk Video | Faouzi Kossentini Michael Horowitz Hsan Guermazi | faouzi@ebriskvideo.com michael@ebriskvideo.com hsan@ebriskvideo.com |
| Ericsson | Andrey Norkin Kenneth Andersson | andrey.norkin@ericsson.com kenneth.r.andersson@ericsson.com |
| ETRI | Jinho Lee Hui Yong Kim Sung-Chang Lim | jinosoul@etri.re.kr hykim5@etri.re.kr sclim@etri.re.kr |
| Fraunhofer HHI | Benjamin Bross | benjamin.bross@hhi.fraunhofer.de |
| Huawei | Yongbing Lin Wen Gao Xiaozhen Zheng | linyongbing@huawei.com wen.gao@huawei.com zhengxiaozhen@huawei.com |
| Institute for Infocomm Research (I2R) | Chuohao Yeo Yih Han Tan | chyeo@i2r.a-star.edu.sg yhtan@i2r.a-star.edu.sg |
| Intel | Yi-Jen Chiu Lidong Xu Wenhao Zhang Han Yu | Yi-jen.chiu@intel.com Lidong.xu@intel.com Wenhao.zhang@intel.com yu.han@intel.com |
| LG | Seungwook Park Yong-Joon Jeon Joonyoung Park Jungsun Kim | Seungwook.park@lge.com yongjoon.jeon@lge.com jy.park@lge.com jungsun.kim@lge.com |
| MediaTek | Chia-Yang Tsai Chih-Ming Fu Ching-Yeh Chen Chih-Wei Hsu Shawmin Lei Xun Guo Yu-Wen Huang | chiayang.tsai@mediatek.com chihming.fu@mediatek.com chingyeh.chen@mediatek.com cw.hsu@mediatek.com shawmin.lei@mediatek.com xun.guo@mediatek.com yuwen.huang@mediatek.com |
| Mitsubishi | Shun-ichi Sekiguchi Kazuo Sugimoto Akira Minezawa | Sekiguchi.Shunichi@eb.MitsubishiElectric.co.jp Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp Minezawa.Akira@ds.MitsubishiElectric.co.jp |
| Motorola Mobility | David Baylon Koohyar Minoo Yue Yu | david.baylon@motorola.com kminoo@motorola.com yyu@motorola.com |
| NTT | Masaaki Matsumura | matsumura.masaaki@lab.ntt.co.jp |
| NTT DOCOMO | Frank Bossen TK Tan | bossen@docomoinnovations.com ttk@pacific.net.sg |
| Nokia | Kemal Ugur Oguz Bici Jani Lainema | Kemal.Ugur@nokia.com ext-oguz.bici@nokia.com jani.lainema@nokia.com |
| Panasonic | Matthias Narroschke Thomas Wedi Semih Esenlik Toru Matsunobu Toshiyasu Sugio Hisao Sasai Takahiro Nishi | Matthias.Narroschke@panasonic.de Thomas.Wedi@eu.panasonic.com Semih.Esenlik@eu.panasonic.com matsunobu.toru@jp.panasonic.com sugio.toshiyasu@jp.panasonic.com sasai.hisao@jp.panasonic.com nishi.takahiro@jp.panasonic.com |
| Peking University | Siwei Ma | swma@jdl.ac.cn |
| Qualcomm | Marta Karczewicz In Suk Chong Wei-Jung Chien Sanjeev Kumar | martak@qualcomm.com ichong@qualcomm.com wchien@qualcomm.com sanjeevk@qualcomm.com |
| Samsung | PoLin Lai Felix C. Fernandes Elena Alshina Elena Alshina Il-Koo Kim Ankur Saxena | wlai@sta.samsung.com ffernandes@sta.samsung.com elena_a.alshina@samsung.com elena.alshina@gmail.com ilkoo.kim@samsung.com asaxena@sta.samsung.com |
| Santa Clara University | Guichun Li Rafael Kaliski | g1li@scu.edu RKaliski@scu.edu |
| Sejong University | Ju Ock Lee | jolee@sju.ac.kr |
| Sharp | Tomohiro Ikai Tomoyuki Yamamoto Yukinobu Yasugi Andrew Segall Jane Zhao Kiran Misra | ikai.tomohiro@sharp.co.jp yamamoto.tomoyuki@sharp.co.jp yasugi.yukinobu@sharp.co.jp asegall@sharplabs.com jzhao@sharplabs.com misrak@sharplabs.com |
| SK Telecom | Joo-Hee Moon Jeongyoen Lim | jhmoon@sejong.ac.kr jeongyeon@sk.com |
| Sony | Ali Tabatabai Ehsan Maani Masaru Ikeda | Ali.Tabatabai@am.sony.com Ehsan.Maani@am.sony.com Masaru.Ikeda@jp.sony.com |
| Texas Instruments (TI) | Madhukar Budagavi Minhua Zhou Woo-Shik Kim Do-Kyoung Kwon | madhukar@ti.com zhou@ti.com wskim@ti.com d-kwon@ti.com |
| Toshiba | Takeshi Chujoh Tomoo Yamakage | takeshi.chujoh@toshiba.co.jp tomoo.yamakage@toshiba.co.jp |
| Zenverge | Dzung Hoang | Dzung.Hoang@Zenverge.com |