| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0580_r1 WG11 Number: m25214 |
| --- | --- |

| Title: | BoG report on review of non-CE ALF proposals |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Andrey Norkin | Email: | andrey.norkin@ericsson.com |
| Source: | BoG on review non-CE ALF proposals |  |  |

_____________________________

This contribution summarizes activities of the break out group (BoG) on review of ALF related non-CE proposals.

# Summary of Recommendations

- The BoG recommends adopting I0278

- The BoG recommends  adopting two part of  I0386, namely Proposal 1 (use EG(k) instead of G(k)) and Proposal 2 (deriving the center coefficient and not sending ΔC9)

- The BoG recommend adopting I0215 into the DIS text and HM provided that the BA ALF stays in the standard text.

# Review of proposals

The proposals are sorted into categories several categories.

1. Signaling/syntax

2. Shapes and coefficient expressions

3. RA/BA study

4. Simplification of filter application

5. Unification of in-loop filtering (signaling)

## Signaling/syntax

JCTVC-I0157 AHG6: Baseline options for ALF [Yu-wen Huang (MediaTek)]

Cross-check: JCTVC-I0493 Crosscheck of Option 2 (RA) and independent ALF chroma control of AHG6: Baseline options for ALF (JCTVC-I0157) [M. Budagavi (TI)] [late]

In this contribution, three options for ALF are proposed.

In option 1, all ALF parameters are coded at LCU level. Each LCU can choose a new filter, a previously coded filter, or off.

In option 2, one filter set is coded in APS, and on/off flags are coded at LCU level. The APS filter set can be always BA or RA and is trained by using a previous picture. Each LCU can choose the APS filter set or off.

In option 3, one filter set is coded in APS, and ALF parameters are also coded at LCU level, which is a combination of option 1 and option 2. Each LCU can choose the APS filter set, a new filter, a previously coded filter, or off.

These three options have already been discussed in CE2. Therefore, the authors did not request a presentation.

## Shapes and coefficient expressions

JCTVC-I0287 Non-CE2: Reduced coefficient precision for ALF complexity reduction [F. Kossentini, M. Horowitz, H. Guermazi, N. Mahdi (eBrisk Video)]

Cross-check: JCTVC-I0424 Non-CE2: Cross check of ALF filter modification (JCTVC-I0287, JCTVC-I0289) by Qualcomm  [I. S. Chong, M. Karczewicz (Qualcomm)]

This contribution presents a technique designed to reduce ALF complexity by constraining the precision of each of ALF coefficients to a specific range depending on its position.

Compared to HM 6.0, the reduced-precision ALF coefficients reportedly yield essentially the same coding efficiency levels while reducing the average bit-depth of the multipliers by approximately 25%.

Figure 1: Coefficient bit-depths in HM 6.0

Figure 2: Proposed coefficient bit-depths

Luma BD-Rate changes are (0.0, 0.0, 0.0, 0.0) for four configurations (AI-HE10, RA-HE10, LB-HE10 LP-HE10).

|  | All Intra HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.3% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.1% | 0.2% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.5% | 0.2% |
| Class F | 0.0% | 0.1% | 0.1% |
| Overall | 0.0% | 0.2% | 0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

|  | Random Access HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.2% | 0.1% |
| Class B | 0.0% | 0.2% | 0.1% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |
| Class F | 0.0% | 0.1% | 0.1% |
| Overall | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

|  | Low delay B HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | 0.2% | 0.2% |
| Class C | 0.0% | 0.3% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.4% | 0.1% |
| Class F | 0.0% | -0.2% | 0.8% |
| Overall | 0.0% | 0.1% | 0.2% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

|  | Low delay P HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | 0.2% | 0.1% |
| Class C | 0.0% | 0.1% | 0.1% |
| Class D | 0.0% | -0.1% | -0.2% |
| Class E | 0.2% | 1.2% | 0.6% |
| Class F | 0.0% | 0.8% | 0.3% |
| Overall | 0.0% | 0.4% | 0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

Two experts expressed support for the proposal.

It has also been noted that the proposal would have interaction with another proposal I0381 if  the latter proposal were adopted.

Recommendation: The break-out group recommends adopting this proposal.

JCTVC-I0289 Non-CE2: A cross 7x5 shape for ALF complexity reduction [F. Kossentini, M. Horowitz, H. Guermazi, N. Mahdi (eBrisk Video)]

Cross-check: JCTVC-I0424 Non-CE2: Cross check of ALF filter modification (JCTVC-I0287, JCTVC-I0289) by Qualcomm  [I. S. Chong, M. Karczewicz (Qualcomm)]

In this contribution, a cross 7x5 ALF shape is proposed instead of the cross 9x7 + center 3x3 square filter shape used in HM 6.0 in order to reduce the number of operations (multiplies and adds).

Using the cross 7x5 shape reportedly yields average BD-rates of 0.4%, -0.4% and -0.5% in Y, U and V (respectively) and decreasing the number of independent coefficients from 10 to 6, resulting in approximately 40% fewer multiplies and adds.

The proposed cross 7x5 shape reportedly has lower encoder/decoder implementation complexity than the HM6.0 shape.

Figure 1: ALF coefficients in HM 6.0

Figure 2: ALF coefficients in the proposed cross 7x5 shape

BD-Rates (AI-HE10, RA-HE10, LB-HE10, LD-HE10):

Y (0.3, 0.5, 03, 06)

U (-0.7, 0.4, -0.8, -0.4)

V (-0.8, 0.3, -0.7, -0.9)

The cross-checker confirmed encoding and decoding times.

One participant expressed a concern about BD-rate losses at some resolution classes.

It was proposed to consider this proposal together with I0386 that informs about the other options on the table.

No action was recommended.

JCTVC-I0386 Study of ALF shapes [M. Budagavi (TI)]

This contribution reports a study of different ALF filter shapes and is information proposal. In this proposal, different filter shapes were implemented by applying encoder side constraints. The software used in to obtain the results is AHG6 Option 2 software using the current picture for filter coefficients estimation.

The 10-tap symmetric 2D FIR filter used in HM-6.0 is shown below.

The proposal presents results for different shapes, such as crosses (and crosses with squares in the center, similar to the current HM6.0 filter) implemented on Option 2 software. For example, a 9x7 cross can be implemented in the encoder by constraining coefficients C2 and C4 to be zero, which reduces the encoder side complexity.

|  | Number mults | Y |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | AI-HE10 | RA-HE10 | LB-HE10 | LP-HE10 |
| HM-6.0 (9x7 cross + center 3x3 square) | 10 |  |  |  |  |
| 9x7 cross | 8 | 0.3% | 0.4% | 0.3% | 0.8% |
| 7x7 cross + center 3x3 square | 9 | 0.1% | 0.1% | 0.1% | 0.1% |
| 9x9 cross | 9 | 0.2% | 0.3% | 0.2% | 0.8% |
| 5x5 star | 9 | 0.2% | 0.3% | 0.3% | 0.1% |
| 7x7 cross + center 3x3 square (encoder constraint) | 10 | 0.1% | 0.1% | 0.1% | 0.1% |
| 9x7 cross (encoder constraint) | 10 | 0.3% | 0.4% | 0.3% | 0.9% |

|  | Number mults | U |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | AI-HE10 | RA-HE10 | LB-HE10 | LP-HE10 |
| HM-6.0 (9x7 cross + center 3x3 square) | 10 |  |  |  |  |
| 9x7 cross | 8 | 0.2% | 0.3% | 0.6% | 1.2% |
| 7x7 cross + center 3x3 square | 9 | 0.2% | 0.0% | 0.1% | 0.0% |
| 9x9 cross | 9 | 0.2% | 0.3% | 0.6% | 1.2% |
| 5x5 star | 9 | 0.4% | 0.1% | 0.6% | 0.6% |
| 7x7 cross + center 3x3 square (encoder constraint) | 10 | 0.2% | -0.1% | 0.2% | 0.1% |
| 9x7 cross (encoder constraint) | 10 | 0.2% | 0.4% | 0.5% | 1.3% |

|  | Number mults | V |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | AI-HE10 | RA-HE10 | LB-HE10 | LP-HE10 |
| HM-6.0 (9x7 cross + center 3x3 square) | 10 |  |  |  |  |
| 9x7 cross | 8 | 0.3% | 0.3% | 0.2% | 0.3% |
| 7x7 cross + center 3x3 square | 9 | 0.2% | 0.1% | 0.0% | -0.1% |
| 9x9 cross | 9 | 0.3% | 0.3% | 0.3% | 0.4% |
| 5x5 star | 9 | 0.4% | 0.1% | 0.6% | 0.3% |
| 7x7 cross + center 3x3 square (encoder constraint) | 10 | 0.1% | 0.0% | 0.1% | -0.3% |
| 9x7 cross (encoder constraint) | 10 | 0.3% | 0.3% | 0.4% | 0.4% |

One participant commented that it is worth considering the case with 7x7 cross and the square since because of the symmetric shape is symmetric and causes only small losses (0.1, 0.1, 01, 01) compared to the current filter shape.

The results were not tested subjectively.

No action is recommended by the group (neither regarding proposal I0289).

JCTVC-I0346 AHG6: Simplification of ALF filter coefficients coding [M. Budagavi (TI)]

Cross-check: JCTVC-I0488 AHG6: Cross-check of simplification of ALF filter coefficients coding (JCTVC-I0346) [T. Yamakage, T. Itoh, T. Watanabe (TOSHIBA)] [late]

This contribution presents three normative modifications to ALF filter coefficients coding, which reportedly result in simplification.

- It is proposed to use Exp-Golomb code instead of Golomb code for luma ALF filter coefficients.

  - When position independent EG(0) code is used instead of the HM-6.0 position dependent Golomb code, the average Y BD-Rate is reported to be in the range of 0.0% to 0.1% for region based ALF and 0.0% to 0.2% for block adaptive ALF.

  - When a position dependent EG(k) code is used, the average Y BD-Rate is reported to be 0.0% for both region based and block adaptive ALF. The proposed EG(k) scheme reportedly has worst case codeword length of 20 bits instead of 258 bits for Golomb(k) of HM-6.0. Short codeword VLCs are reported to be desirable than long codeword VLCs from parsing complexity reduction point of view.

- It is proposed to transmit information or only 9 ALF coefficients instead of 10 in HM-6.0 for both luma and chroma. The 10th coefficient is predicted from the remaining 9 such that the ALF filter has unit gain. The average Y BD-Rate is reported to be 0.0% for both region based and block adaptive ALF. The average U and V BD-Rates are reported to be in the range of 0.0% to -0.2% for region based ALF and 0.1% to -0.3% for block adaptive ALF.

When position independent EG(3) code is used in combination with second modification of transmitting only 9 ALF coefficients, the average Y BD-Rate is reported to be 0.0% for both region based and block adaptive ALF. The average U and V BD-Rates are reported to be in the range of 0.0% to -0.3% for region based ALF and 0.1% to -0.3% for block adaptive ALF

The proposal contains three technical changes (proposals).

Proposal 1

This proposal suggests using EG(k) instead of G(k) in order to decrease the worst case codeword length.

|  |  |  | HM-6.0 -Golomb (k) |  |
| --- | --- | --- | --- | --- |
| Syntax element | Filter coefficient | Coefficient value range | k | Max length (in bits) |
| alf_filt_coeff[0] | C0 | -256 to 255 | 2 | 68 |
| alf_filt_coeff[1] | C1 | -256 to 255 | 3 | 37 |
| alf_filt_coeff[2] | C2 | -256 to 255 | 3 | 37 |
| alf_filt_coeff[3] | C3 | -256 to 255 | 4 | 22 |
| alf_filt_coeff[4] | C4 | -256 to 255 | 3 | 37 |
| alf_filt_coeff[5] | C5 | -256 to 255 | 1 | 131 |
| alf_filt_coeff[6] | C6 | -256 to 255 | 2 | 68 |
| alf_filt_coeff[7] | C7 | -256 to 255 | 3 | 37 |
| alf_filt_coeff[8] | C8 | -256 to 255 | 4 | 22 |
| alf_filt_coeff[9] | C9 | 0 to 511 | 1 | 258 |

|  |  |  | Proposed - EG(k) |  |
| --- | --- | --- | --- | --- |
| Syntax element | Filter coefficient | Coefficient value range | k | Max length (in bits) |
| alf_filt_coeff[0] | C0 | -256 to 255 | 1 | 17 |
| alf_filt_coeff[1] | C1 | -256 to 255 | 2 | 16 |
| alf_filt_coeff[2] | C2 | -256 to 255 | 3 | 15 |
| alf_filt_coeff[3] | C3 | -256 to 255 | 4 | 14 |
| alf_filt_coeff[4] | C4 | -256 to 255 | 3 | 15 |
| alf_filt_coeff[5] | C5 | -256 to 255 | 1 | 17 |
| alf_filt_coeff[6] | C6 | -256 to 255 | 3 | 15 |
| alf_filt_coeff[7] | C7 | -256 to 255 | 3 | 15 |
| alf_filt_coeff[8] | C8 | -256 to 255 | 5 | 13 |
| alf_filt_coeff[9] | C9 | 0 to 511 | 0 | 20 |

Table below presents the BD-Rate results when EG(k) is used to code luma ALF coefficients in region based ALF. Anchor is ALF AhG Option2 software with RA coding using current picture information.

|  | AI-HE10 | RA-HE10 | LB-HE10 | LP-HE10 |
| --- | --- | --- | --- | --- |
| Y | 0.0% | 0.0% | 0.0% | 0.0% |
| U | 0.0% | 0.0% | 0.0% | 0.1% |
| V | 0.0% | 0.1% | -0.2% | -0.4% |

Table below presents the BD-Rate results when EG(k) is used to code luma ALF coefficients in block adaptive ALF. Anchor is ALF AhG Option2 software with BA coding using current picture information.

|  | AI-HE10 | RA-HE10 | LB-HE10 | LP-HE10 |
| --- | --- | --- | --- | --- |
| Y | 0.0% | 0.0% | 0.0% | 0.0% |
| U | 0.0% | 0.0% | 0.1% | -0.1% |
| V | 0.0% | -0.1% | 0.2% | 0.0% |

Proposal 2

In HM-6.0, a ΔC9 information is transmitted. ΔC9 is calculated as: C9 – (1.0 – 2*(C0+C1+C2+C3+C4+ C5+C6+C7+C8)).

The proposal suggests not sending information for ΔC9 and sending 9 filter coefficients only. It was reported that 97% of the time ΔC9 is zero for region based ALF and 97.5% of the time ΔC9 is zero for block based ALF for luma coefficients. The distribution for ΔC9 for chroma is reportedly similar to the luma case.

This makes the filter a unit gain filter. There was a comment from one participant that it might not be needed to constrain the filter to be a unit gain one. However, this part of the proposal had a general support from participants.

Region based ALF results when center coefficient is not transmitted. Anchor is ALF AhG Option2 software with RA coding using current picture information.

|  | AI-HE10 | RA-HE10 | LB-HE10 | LP-HE10 |
| --- | --- | --- | --- | --- |
| Y | 0.0% | 0.0% | 0.0% | 0.0% |
| U | -0.1% | -0.1% | -0.2% | 0.0% |
| V | -0.2% | -0.1% | 0.0% | -0.2% |

Block adaptive ALF results when center coefficient is not transmitted. Anchor is ALF AhG Option2 software with BA coding using current picture information.

|  | AI-HE10 | RA-HE10 | LB-HE10 | LP-HE10 |
| --- | --- | --- | --- | --- |
| Y | 0.0% | 0.0% | 0.0% | 0.0% |
| U | -0.2% | 0.0% | 0.1% | 0.0% |
| V | -0.3% | -0.2% | 0.0% | 0.1% |

Proposal 3

Use EG(3) code to encode all coefficients independently of their position.

One expert noticed that changing from Golomb to EG code is desirable but changing from EG(k) to EG(k)  might not be desirable because of worse filter adaptability.

It was also commented that EG coding may have interaction with proposal I0287.

Recommendation: The group recommends to adopt Proposal 2 (deriving the center coefficient and not sending ΔC9) and  Proposal 1 (use EG(k) instead of G(k)) to the DIS and the next version of HM.

JCTVC-I0381 ALF filter coefficients distribution and modifications to coding [M. Budagavi (TI)]

It is reported that distributions of chroma ALF coefficient values have peak at other than zero value and also the average behavior of chroma ALF is low-pass. The proposal suggests that instead of directly coding the chroma ALF coefficients (as in HM-6.0), the coefficients be coded after subtracting a fixed offset (corresponding to the peak of distribution). The different offset values used are:

Int chromaCenter[10] = {1, -11, 0, 34, 0, -3, 6, -13, 34, 0};

Chroma ALF coefficient distribution for region based ALF.

It is proposed to subtract the offset from the chroma ALF coefficient since the distributions have peaks at non-zero pozitions.

Table below presents the BD-Rate results when proposed technique is used to code chroma ALF coefficients in region based ALF. Anchor is ALF AhG Option2 software with RA coding using current picture information.

|  | AI-HE10 | RA-HE10 | LB-HE10 | LP-HE10 |
| --- | --- | --- | --- | --- |
| Y | 0.0% | 0.0% | 0.0% | 0.0% |
| U | 0.0% | -0.1% | -0.1% | -0.1% |
| V | 0.0% | -0.1% | -0.2% | -0.3% |

## Recommendation: BoG recommends no action this meeting cycle.

## RA/BA study

JCTVC-I0215 AHG6: ALF baseline Option 2 BA with reduced number of filters [P. Lai, F. C. A. Fernandes (Samsung)]

Cross-check: JCTVC-I0525 AHG6: Crosscheck on ALF baseline Option 2 BA with reduced number of filters (JCTVC-I0215) [T. Tsukuba, T. Ikai (Sharp)] [late]

This proposal investigates reducing of number of filters in ALF block-based filter adaptation with directional features (BA-DIR), in the context of ALF baseline Option 2 BA created in AHG6. Two approaches are tested:

1. Force encoder to merge initial BA-DIR classes into fewer than a maximum allowed number of filters (similar to the methods in CE2.c.1-1):

- Force to merge below 8 filters: 0.0% BD-rate changes.

- Force to merge below 4 filters: 0.1% BD-rate changes.

2. Modify the class mapping table (similar to the approach in CE2.c.1-2).

- Modify the class mapping table to 10 classes: AI / RA / LDB / LDP: 0.0% / 0.0% / 0.0% / 0.0%

- Modify the class mapping table to 7 classes: AI / RA / LDB / LDP: 0.0% / 0.0% / 0.0% / 0.0%

It is proposed to have only three quantization bins for variance. This results in only 7 filters in BA-ALF. Small losses (about 0.1%) have been observed for higher resolution sequences with the largest loss (0.3) for Nebuta.

It is proposed that if BA filter is further considered in the standard, 7 are used instead of 16.

The benefits of this are encoder complexity and less on-chip memory.

Both cross-checkers think that this feature is desirable for BA.

Recommendation: The BoG recommend including this change into the DIS text and HM provided that the BA ALF stays in the standard text.

JCTVC-I0363 AHG6: Comparison of block adaptive (BA) and region adaptive (RA) ALF [I. S. Chong, K. Sanjeev, M. Karczewicz (Qualcomm)]

The proposal shows comparison between BA and RA. The difference is coding gain (from 0.5 to 1.0 for 4 HE10 configurations). About 1 percent gain was reported for higher resolution sequences.

RA with 64 filters: no gain

BA with 8 filters (encoder only) no loss

Complexity analysis says that there is a problem storing 16 filters in the on-chip cache but it can be reduced to 7 or 8 using either encoder or normative solution.

It is claimed that the BA and RA are similar in row by row filtering loading

Reportedly, block activity results additionally in 1.56 additions/pixel.

The proponent has also demonstrated the visual test results. The proponent claims that lines and edges are preserved better with BA than RA. (At it also claimed that in SpinCalendar the difference is visible even in motion).

The suggestion is to include the Option2 BA into the Main Profile.

In case BA it is not included in the Main profile, it is suggested to keep the text of BA in the standard draft text and consider including it into higher profiles. The proponent also claims that the still picture quality is also important for decisions made broadcasting and content production companies. One participant agrees with these comments.

It was commented by one participant that BA can be more complex in hardware implementation because of filter switching. The proponent claims that the hardware complexity is similar for both BA and RA.

It was also noticed by one participant that the filter switching complexity is minor compared to other operations.

It was also noticed by one participant that RA has more flexibility in design (not loading filters into the register).

JCTVC-I0454 AHG6: Test results on region adaptive ALF with 64 regions [T. Yamakage, T. Itoh, T. Watanabe, T. Chujoh (TOSHIBA), C.-Y. Tsai, C.-Y. Chen, C.-M. Fu, Y.-W. Huang, S. Lei (MediaTek), I. S. Chong, M. Karczewicz (Qualcomm)] [late]

There were two modes in ALF: region adaptive (RA) and block adaptive (BA) ALF in HM5/WD5. In AHG6, it is reported in JCTVC-I0157 that BA shows better coding efficiency than RA (0.2-0.3% on average) especially on Class A sequences. However, in comparison with RA, BA requires (1) additional feature computation to switch filters and (2) faster switching of filters (BA switches filters in each 4 x 4-block, while RA switches filters by some LCUs).

In this contribution, the maximum number of regions in RA is increased from 16 to 64. It is reported that even when the maximum number of regions in RA is increased, almost no additional gain is observed.

The proponent suggested to skip the presentation since it has already been discussed in the CE report.

## Simplification of filter application

JCTVC-I0169 Grid displacements for ALF [S. Esenlik, M. Narroschke, T. Wedi (Panasonic)]

Cross-check: JCTVC-I0527 Cross-check of JCTVC-I0169 - Grid displacements for ALF [A. Fuldseth (Cisco)] [late] [miss]

The proponent was not present to present the proposal. The proposal addresses the problem of reducing the ALF filter coefficient line buffer chosen. The proposal is not relevant any more since ALF option 2 was chosen for adoption, which does not have this problem.

JCTVC-I0290 AHG6: Restriction on ALF filtering area [T. Ikai (Sharp)]

Cross-check: JCTVC-I0214 AHG6: Cross-check of JCTVC-I0290 on restriction on ALF filtering area [P. Lai, F. C. A. Fernandes (Samsung)] [late]

In this proposal, the ALF filtering area of a picture shall be equal to or less than the specified maximum ratio of the total area of picture as bitstream conformance. It can be level constraints or general constraints.

This proposed area restriction can be achieved by encoder only change using budget control and adaptive threshold technique as shown Fig 2. Firstly total_budget is set as picture_area * the maximum ratio. Then, LCU loop starts. In the loop, the filter decision of the target area is processed. The decision consists of three step1.

Encoder steps

[Step1] Budget control

If target_area + used_area exceeds the total_budget, then filtering shall be off.

[Step2] Adaptive threshold control

The benefit of the filtering is calculated by SSD_reduction by filtering divided by target_area. If the benefit is equal to or larger than an adaptive Threshold TH, the filtering is not restricted. Otherwise the filtering is restricted.

[Step 3] RD control

The BD-rate loss by the proposed restriction (anchor is HM5.0, 150frames for ClassA/B, )

|  | Maximum restriction area |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 30 % | 40% | 50% | 60% | 70% | 80% |
| IO10 | 0.7 | 0.5 | 0.4 | 0.3 | 0.2 | 0.2 |
| RA10 | 1.1 | 0.7 | 0.5 | 0.3 | 0.1 | 0.1 |
| LB10 | 0.6 | 0.4 | 0.2 | 0.1 | 0.1 | 0.0 |
| LP10 | 0.6 | 0.4 | 0.2 | 0.1 | 0.1 | 0.0 |

This is a profile/ level restriction.

It was commented by a participant that such a restriction maybe useful for the software decoder but it is not useful in the hardware implementation.

The proponent does not want to ask adopting the proposal at this meeting cycle but would like first to check its performance in combination with option 2 if option 2 is adopted.

## Unification of in-loop filtering (signaling)

JCTVC-I0171 Refinement for SAO and ALF syntax in the APS [S. Esenlik, M. Narroschke, T. Wedi (Panasonic)]

The proponents were not present to present the proposal.

The proposal in not relevant any more since the SAO syntax is removed from the APS (as well as the ALF syntax)

# References

- Y.W. Huang, "AHG6: Baseline options for ALF", JCTVC-I0157, Geneva, CH, 27 April – 7 May, 2012.

- M. Budagavi, "Crosscheck of Option 2 (RA) and independent ALF chroma control of AHG6: Baseline options for ALF (JCTVC-I0157)", JCTVC-I0493 Geneva, CH, 27 April – 7 May, 2012.

- F. Kossentini, M. Horowitz, H. Guermazi, N. Mahdi, "Non-CE2: Reduced coefficient precision for ALF complexity reduction", JCTVC-I0287 Geneva, CH, 27 April – 7 May, 2012.

- I. S. Chong, M. Karczewicz, "Non-CE2: Cross check of ALF filter modification (JCTVC-I0287, JCTVC-I0289) by Qualcomm",  JCTVC-I0424, Geneva, CH, 27 April – 7 May, 2012.

- F. Kossentini, M. Horowitz, H. Guermazi, N. Mahdi, Non-CE2: A cross 7x5 shape for ALF complexity reduction, JCTVC-I0289, Geneva, CH, 27 April – 7 May, 2012.

- I. S. Chong, M. Karczewicz, "Non-CE2: Cross check of ALF filter modification (JCTVC-I0287, JCTVC-I0289) by Qualcomm", JCTVC-I0424, Geneva, CH, 27 April – 7 May, 2012.

- M. Budagavi, "Study of ALF shapes", JCTVC-I0386, Geneva, CH, 27 April – 7 May, 2012.

- M. Budagavi, "AHG6: Simplification of ALF filter coefficients coding", JCTVC-I0346, JCTVC-I0424, Geneva, CH, 27 April – 7 May, 2012.

- T. Yamakage, T. Itoh, T. Watanabe, "AHG6: Cross-check of simplification of ALF filter coefficients coding (JCTVC-I0346)", JCTVC-I0488, Geneva, CH, 27 April – 7 May, 2012.

- M. Budagavi, "ALF filter coefficients distribution and modifications to coding", JCTVC-I0381, Geneva, CH, 27 April – 7 May, 2012.

- P. Lai, F. C. A. Fernandes, "AHG6: ALF baseline Option 2 BA with reduced number of filters," JCTVC-I0215, Geneva, CH, 27 April – 7 May, 2012.

- T. Tsukuba, T. Ikai, "AHG6: Crosscheck on ALF baseline Option 2 BA with reduced number of filters JCTVC-I0525", JCTVC-I0215, Geneva, CH, 27 April – 7 May, 2012.

- S. Chong, K. Sanjeev, M. Karczewicz, "AHG6: Comparison of block adaptive (BA) and region adaptive (RA) ALF", JCTVC-I0363, Geneva, CH, 27 April – 7 May, 2012.

- T. Yamakage, T. Itoh, T. Watanabe, T. Chujoh, C.-Y. Tsai, C.-Y. Chen, C.-M. Fu, Y.-W. Huang, S. Lei, I. S. Chong, M. Karczewicz, "AHG6: Test results on region adaptive ALF with 64 regions", JCTVC-I0454,  Geneva, CH, 27 April – 7 May, 2012.

- S. Esenlik, M. Narroschke, T. Wedi, "Grid displacements for ALF", JCTVC-I0169, Geneva, CH, 27 April – 7 May, 2012.

- A. Fuldseth, "Cross-check of JCTVC-I0169 - Grid displacements for ALF", JCTVC-I0527, Geneva, CH, 27 April – 7 May, 2012.

- T. Ikai, "AHG6: Restriction on ALF filtering area", JCTVC-I0290, Geneva, CH, 27 April – 7 May, 2012.

- P. Lai, F. C. A. Fernandes, "AHG6: Cross-check of JCTVC-I0290 on restriction on ALF filtering area", JCTVC-I0214, Geneva, CH, 27 April – 7 May, 2012.

- S. Esenlik, M. Narroschke, T. Wedi, "Refinement for SAO and ALF syntax in the APS", JCTVC-I0171, Geneva, CH, 27 April – 7 May, 2012.