| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0581 WG11 Number: m25214 |
| --- | --- |

| Title: | BoG report on high level parallelization |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): |  | Email: |  |
| Source: | BoG on high level parallelization |  |  |

_____________________________

This is contribution contains meeting notes from the break out group (BoG) on high level parallelization.

### High-level parallelism

JCTVC-I0113 High level syntax parsing issues [K. Suehring (HHI)]

Two high level syntax parsing issues have been discovered after the last JCT-VC meeting and been discussed on the JCT-VC email reflector: 1) a parsing order issue in the slice header (bug tracker issue #391) and 2) a parsing dependency between SPS and PPS (bug tracker issue #428). This contribution discusses possible solutions. For issue 1) the author suggests reordering the syntax elements (solution B) and for issue 2) the author suggest removing the tile parameter overwrite mechanism (solution A).

Issue 1 - Support voiced from multiple participants for solution B

Issue 2 – Reason for having in both SPS/PPS discussed.

Question: Does proposal allow tiles and WPP to co-exist in a sequence (frame by frame)?

Comment: Use of PPS signaling better supports load balancing

Comment: For issue 2, mandate that tiles_or_entropy_coding_sync_idc must have the same value for all PPS

Recommend: Adoption B for issue 1

Recommend: Adopt solution A for issue 2 w/ mandate that tiles_or_entropy_coding_sync_idc must have the same value within a coded video sequence.

JCTVC-I0056 Bitstream restriction flag to enable tile split [O. Nakagami, T. Suzuki (Sony)]

The contribution proposes to add 1-bit flag in VUI as tile_splittable_flag. The proposed flag represents bitstream restriction when tile coding is used. The flag enables decoders to decode tiles independently not only in picture level but also in bitstream level. When the flag is set to true, it is possible to extract any tile from the bitstream without entire decoding process. It is asserted that such flag enhances usability of tile coding in some application field. Eg. Frame packing stereo encoding, TV-conference systems etc.

Comment: Proposal disables inter-view prediction.  Concern expressed on coding efficiency impact.

Clarification that this is an encoder choice

Question: Why not just code two separate sequences or handle this at a higher level?

Concern over parsing dependencies by placing tile information in VUI

Question: Should this be located in an SEI message?

Concern expressed over use case size

No action recommended

JCTVC-I0158 Picture Raster Scan Decoding in the presence of multiple tiles [G. Clare, F. Henry, S. Pateux (Orange Labs)]

Picture raster scan single core decoding of frames encoded with multiple tiles is desirable in order to avoid the buffering of most of the picture before a single line of LCUs can be output. In the current design of HEVC, picture raster scan decoding requires bitstream jumping and CABAC state memorization/restore. The current contribution proposes to flush CABAC at the end of each LCU line inside a tile, so that CABAC state operations can be avoided and buffers can be eliminated. The impact on rate-distortion performance is +0.1% (Intra), +0.6% (Random Access), +1.5% (Low Delay) compared to current design when a large number of tiles is used (JCTVC-F335 tile configuration).

Impact: 40 bytes for Main profile

Question: Is this mandatory? Yes.

Comment: Encoder is responsible for delay already.

Coding efficiency: 4.9% loss for low delay B, Class E

Comment: Proposal focuses on low delay and results show larger impact for this class of sequences

Comment: Bit-rate variation and buffering also affect decoder delay

No action recommended

JCTVC-I0387 Cross verification of Picture Raster Scan Decoding in the presence of multiple tiles (JCTVC-I0158) [M. Coban (Qualcomm)] [late]

Cross check of JCTVC-I0158

JCTVC-I0520  “Parallel Scalability and Efficiency of WPP and Tiles” [C. C. Chi, M. Alvarez-Mesa, B. Juurlink, V. George, T. Schierl] [late]

This document presents a parallel scalability and efficiency analysis of the two main parallelization approaches being considered in HEVC, namely Tiles and Wavefront Parallel Processing (WPP). The two approaches have been implemented into HM4 and evaluated on an Intel Xeon/Westmere parallel machine with 12 cores running at 3.33 GHz. This document presents a comparison in terms of parallel scalability, processor usage efficiency and memory bandwidth.

Proponent updated loop filter of software to better match HM6

Boost library for high level parallelization functionality

Observation: For one slice per picture, RA, HE-profile both Tiles and WPP provide “significant” speedup for this implementation

CPU usage: Shows that tiles have higher CPU utilization for this experiment and implementation (higher CPU utilization is good)

Study of synchronization and memory access: For this implementation WPP has lower memory bandwidth compared to tiles.

Software can be made available

Comment: Loop filter is not implemented in same manner in WPP and tiles results reported here.

Comment: Deblocking tile by tile could have lower memory bandwidth

Comment: One participant reported implementing the loop filter for tiles in a different manner and observed different cache performance/locality and lower memory bandwidth.

Results here are very dependent on implementation

Discussion on performance saturation of the implementation and potential sources of serial bottlenecks.  Load balancing strategy of implementation.

Comment: May want to investigate cache conflicts for smaller images

Architecture considered is one specific architecture.  Different architectures may have significantly different performance.

Comment: For memory bandwidth results, participant observes higher memory bandwidth for single core point in results.  Question if this suggests implementation issue.

No action recommended

JCTVC-I0118 AHG4: Enable parallel decoding with tiles [M. Zhou (TI)]

Real-time UHD decoding can exceed the capability of a single core decoder. To enable parallel decoding on multi-core platforms, it is proposed to mandate evenly divided sub-pictures for high levels to guarantee pixel-rate balancing among cores when sub-pictures are processed in parallel. The key points of proposal are: 1) A picture is divided into a number of sub-pictures of equal size (in units of LCUs); 2) Sub-pictures are independent, only in-loop filters can be allowed cross the sub-picture boundaries; 3) Tiles, slices, entropy slices and WPP are contained in sub-pictures and cannot cross sub-picture boundaries; 4) The sub-picture partitioning information is signaled with tile syntax. If sub-pictures are mandated, tiles have to be uniformly spaced in vertical direction. 5) Sub-picture entries in bitstream are signaled in APS; 6) Sub-picture ID is signaled in slice header for low-latency applications. Finally, the limits for number of sub-pictures are also specified. The specification allows building a multi-core decoder by simply replicating the single core decoder without need of increasing the line buffer size.

Proposal: Mandate number of sub-pictures.  Here, a sub-picture is independent from another sub-pictures except that loop filtering between sub-pictures is allowed

Motivation: Minimize cross-core communication

Multiplexing is at higher layer

Question: What is effect on picture quality by dividing image into independent regions?

Question: Can slices be used instead with a maximum number of CTBs?

Response: Memory requirement is higher for slice solution.

Suggestion: Should we have separate levels with mandated sub-pictures/tiles and without mandated sub-pictures/tiles?  This would allow applications to select a higher level that does not contain sub-tiles.

Comment: Without mandating sub-pictures, a decoder cannot depend on parallelization

Comment: CANNB has a comment to not mandate partitioning of a picture

Clarification: Motion compensation allowed across sub-pictures

Comment: Wavefront is not supported completely in sub-picture in syntax.

Comment: Could this be done with constraints on tiles?

Comment: Recognition of implementation issue

Intention is to not allow slices to cross sub-picture boundaries.

Comment: Prefer approach that is general and not for a specific architecture

Comment: Sub-picture comment is asserted to be a general concept and not specific to an architecture

Comment: One expert commented that within a sub-picture other parallelization tools could be used.  Note that currently WPP are not allowed together, but this could be changed with sufficient evidence.

Consensus: General support for the concept.  The group likes the concept of uniformly spaced (like tiles) sub-pictures given that we impose no additional constraints beyond the sub-picture locations.  This can be possibly achieved with existing syntax and appropriate constraints.

Recommendation: Discuss profile/level issues (above – adding additional levels without subpictures/tiles) in larger group

JCTVC-I0233 AHG4: Enabling decoder parallelism with tiles [R. Sjöberg, J. Samuelsson, J. Enhorn (Ericsson)]

This contribution identifies a number of problems regarding tiles: There is currently no mechanism for an encoder to guarantee that a coded video sequence can be decoded in parallel, the tile syntax is replicated in both SPS and PPS, there is no semantics for the PPS tile syntax, there is a dependency between SPS and PPS, no tile index is signaled when entry point offsets are used for tiles, the semantics for tile_idx_minus_1 is incomplete, and the tile parameter derivation text is currently in the tile semantics section. A revision 1 (r1) version of this document was uploaded late. The r1 changes consist of changes to the abstract and editorial corrections to the proposed WD semantics for use_tile_info_from_pps_flag.

This proposal claims to address these tile problems by proposing the following changes:

1) To make a separate tile_info syntax table that is shared between SPS and PPS

2) To merge the two PPS flags, tile_info_present_flag and tile_control_present_flag into one flag: tile_info_present_in_pps_flag

3) To add a flag in the slice header, use_tile_info_from_pps_flag, to control whether the tile info from the SPS or the PPS shall be used. The flag is only present if there is both SPS and PPS tile info.

4) To add an SPS flag, tiles_fixed_structure_flag, to indicate that the tile info from the SPS is always used. If set to one, we do not parse use_tile_info_from_pps_flag.

5) To add two SPS flags to indicate that all tiles do have entry point offsets or entry point markers and to include tile id with entry point offsets and markers only if the corresponding flag is set equal to 0.

6) To only send tile_idx_minus_1 for entry point markers if tiles are used (not send them in case of WPP) and change its name to tile_id_marker_minus1

7) To specify the length and value of tile_idx_minus_1

8) To add a tile id syntax element, tile_id_offset_minus1, for every tile entry point offset

9) To move tile parameters derivation text, currently in the semantics section, to a new subclause in the decoding process

10) To clarify the semantics for entry_point_offset

NOTES:

1) To make a separate tile_info syntax table that is shared between SPS and PPS

2) To merge the two PPS flags, tile_info_present_flag and tile_control_present_flag into one flag: tile_info_present_in_pps_flag

3) To add a flag in the slice header, use_tile_info_from_pps_flag, to control whether the tile info from the SPS or the PPS shall be used. The flag is only present if there is both SPS and PPS tile info.

Comment: Tile information is no longer in SPS and PPS with adoption of  JCTVC-I0113.

4) To add an SPS flag, tiles_fixed_structure_flag, to indicate that the tile info from the SPS is always used. If set to one, we do not parse use_tile_info_from_pps_flag.

Proposal: Signal tiles_fixed_structure_flag in VUI (given other recommendation to signal tiles syntax in PPS.)  Inferred to be 0 if not present.

Recommend: Adopt

5) To add two SPS flags to indicate that all tiles do have entry point offsets or entry point markers and to include tile id with entry point offsets and markers only if the corresponding flag is set equal to 0.

Proponent: It is OK if group mandates entry points for all tiles.

Entry points mandated for all tiles

6) To only send tile_idx_minus_1 for entry point markers if tiles are used (not send them in case of WPP) and change its name to tile_id_marker_minus1

Markers were removed in other recommendation

7) To specify the length and value of tile_idx_minus_1

Note: Confirm with software

Markers were removed in other recommendation

8) To add a tile id syntax element, tile_id_offset_minus1, for every tile entry point offset

Proponent: Mandate for all entry points is OK

Entry points mandated for all tiles in other recommendation

9) To move tile parameters derivation text, currently in the semantics section, to a new subclause in the decoding process

Recommend: Adopt (remove [X] to reflect recommendations above)

10) To clarify the semantics for entry_point_offset

Recommend: Consult with editors but maintain intent

JCTVC-I0356 Support of independent sub-pictures [M. Coban, Y. -K. Wang, M. Karczewicz (Qualcomm)]

This contribution presents the the concept of supporting sub-pictures in HEVC. Currently tiles provide encoder and decoder side parallelism without restrictions on loop filtering across tiles and referencing of pixel and motion information from outside the tile boundaries. In order provide more flexible parallelism for UHD video decoding the concept of independent sub-pictures within HEVC framework is proposed. Sub-pictures prohibit referencing from outside of sub-picture boundaries and disables loop-filtering across sub-picture boundaries.

Comment: Similar to JCTVC- I0056

No action recommended

JCTVC-I0078 AHG4: Single-core decoder friendly WPP [Hendry, B. Jeon (LG)]

It is assessed that the current ordering of coding tree in the bitstream when WPP is used might not be friendly for single-core decoder since it has to jump forth and back within the bitstream to the correct location for parsing. One way to avoid this problem is to force the number of WPP substream to be maximum, that is, one LCU line is one WPP substream, so that the order of coding tree is in the normal picture raster scan order. However, such hard constraint to always force using maximum number of substream might not be always desired as it is further assessed that the current coding tree order is useful if the bitstream is really intended for multi-core decoder.

This contribution proposes to add a flag either in SPS or PPS to indicate whether or not coding tree is reordered when WPP is used. It is suggested by proponent that the flag gives flexibility to encoder to determine to which side the coded bitstream will be friendlier to, that is, if the flag is set, then the coded bitstream is friendlier to multi-core decoder, else then the coded bitstream is friendlier to single-core decoder.

Proponent: Prefers not to mandate one row of LCUs per sub-stream to address single core performance

Comment: Bit-stream jumping may not be a significant issue for implementation.

Comment: Requires an encoder to have knowledge of decoder architecture (i.e., if a bit-stream jump is difficult) and also parallelization factor of that decoder

Comment: Other proposals mandate one row of LCUs per sub-stream

No action recommended

JCTVC-I0079 AHG4: Simplified CABAC Initialization for WPP [Hendry, B. Jeon (LG)]

Currently when WPP is used, CABAC probability table of the first LCU, starting from the second LCU rows, is initialized from that of the 2nd LCU of the previous LCU row. It is assessed that this initialization mechanism requires a buffer for storing the states of CABAC probability table before it is used. This contribution reports a study on possibility to reset CABAC probability table of the 1st LCU of every LCU row when WPP is used in order to avoid the need to provide buffer for storing the states of CABAC probability table. It is reported that resetting CABAC probability table at every first LCU causes luma loss at average 0.1% for AI-MAIN, 0.1%Y for AI-HE10, 0.2% for RA-MAIN, 0.2% for RA-HE10, 0.7% for LB-MAIN, and 0.7% for LB-HE10.

It is suggested by the proponent that the idea proposed in this contribution can be combined with the idea proposed in I0078 – AHG4: Single-core decoder friendly WPP, that is, reset CABAC probability table of the 1st LCU of every LCU row when the proposed ctb_reordering_flag is not set so that the coded bitstream is even friendlier for parsing and decoding with single-core decoder. Further, the proponent would support the inclusion of this version of WPP (i.e., reset CABAC probability table of the 1st LCU of every LCU row and mandate that ctb_reordering_flag is not set) to the main profile of HEVC.

Comment: Current CABAC initialization is trained for test set.  Additional information is in JCTVC-I0463 that shows performance CABAC syncro on different sequences.  Results are AI: .2-.8%; RA: .3-1.0%; LDB: .3-1.7% loss for sequences outside of test set when disabling the CABAC syncro.

Comment: Additional overhead is also incurred for WPP by other parts of the system.

Comment: Size of CABAC buffer may be smaller in actual implementation.  (Some context models in HM are not used.)

No action recommended

JCTVC-I0463 Crosscheck of AHG4: Simplified CABAC Initialization for WPP (JCTVC-I0079) [G. Clare, F. Henry (Orange Labs)] [late]

See notes for JCTVC-I0079.

JCTVC-I0139 Syntax on wavefront information [S. Jeong, T. Lee, C. Kim, J. Kim, J. Park (Samsung)]

The current syntax design for Tile and WPP supporting parallel processing is not unified in the location of sending the detailed information and it is not efficient. It is proposed to signal WPP information as the same level of parameter set as Tiles, which is SPS level having overriding flag in PPS level.

Comment: Tiles information recommended to be removed from SPS.

No action recommended.

JCTVC-I0360 Wavefront parallel processing simplification [Y.-K. Wang, M. Coban (Qualcomm), F. Henry (Orange Labs)]

This document proposes to simply the wavefront parallel processing (WPP) design by mandating one substream per LCU line, in order to preserve bitstream causality and providing maximum level of parallelism capability. Simulation results comparing to the current design without this simplification are provided in the attachment of this document.

Comment: This may simplify decoder use of WPP, since the encoder does not have to target a specific decoder parallelization.

Comment: Provides maximum parallelization to WPP decoder

Concern: Coding efficiency loss may be significant for larger picture sizes

Comment: Functionality outweighs the coding efficiency loss.

Recommend: Adopt

JCTVC-I0361 Restriction on coexistence of WPP and slices  [M. Coban, Y.-K. Wang (Qualcomm)]

This document proposes to limit the co-existence of WPP and slices similarly as the co-existence of tiles and slices.

Proposal: Use same restriction for slices-wpp as slices-tiles.  This means multiple slices can be in a CTB row or multiple CTB rows can be in a slice.  Other combinations are not allowed.

Comment: May be related to JCTVC-I0229.

Revisited after JCTVC-I0229.

Two proposals – proposal 1 and proposal 2 in presentation.

Comment: MTU size matching may be less efficient with the proposed method

Comment: WPP coding efficiency improvements require multiple sub-streams per slice

Comment: Support that problem considered should be addressed

Comment: Should not bound smallest size of possible slice

Recommend: Adopt solution 2 in the presentation (subject to text)

JCTVC-I0229 Dependent Slices [T. Schierl, V. George, A. Henkel, D. Marpe (HHI)]

Wavefront parallel processing (WPP) structures the picture into substreams, which have dependencies to each other. Those substreams, e.g., if applied as one substream per row, may be contained in a single slice per picture. In order to allow an immediate transport after encoding such a substream, each substream would need to be in its own slice. The concept of dependent slices, as proposed in this contribution, allows for information exchange between slices for both the parsing and reconstruction process. This enables low delay coding and transmission with a minimum bit rate overhead for WPP.

Motivation: Allow for parsing and reconstruction to cross slice boundaries

Additionally, allows for implicit entry point signaling for WPP.  Asserted to allow handling sub-stream entry points at a higher level due to NAL unit header.

Compared to one substream per row, the increase is asserted to be about .8%.  However, the comparison is approximate due to different HM versions.

Comment: Seems more generic than application to parallel tools.  May be useful for reducing latency.

Question: What are gains compared to using “regular” slices?  Proponents results show gains of about 13-15% coding efficiency improvements compared to “regular” slices.

Comment: Support for both proposal and use case

Question: What is complexity and resource increase?  No increase compared to WPP.

Question: Can we use fragmentation at the packetization layer?  Asserted that proposal provides lower latency.

Comment: Lower delay from proposal comes at ~10% bit-rate cost for Class E.

Comment: Lower delay is worth bit-rate cost; support expressed for proposal

Question: How does this effect slice rate?  Does it increase the rate?

Comment: Concern expressed about decoder implementation

Maybe related to I427, I159

Recommendation: Review in larger group

JCTVC-I0501 Crosscheck of Dependent Slices (JCTVC-I0229) [G. Clare, F. Henry (Orange Labs)] [late]

Results agreed.

Wavefront implementation only.

Software and document agreed.  Noted that document only described 1 row per dependent slice configuration.

JCTVC-I0138 Syntax on entropy slice information [S. Jeong, T. Lee, C. Kim, J. Kim, J. Park (Samsung)]

In current HEVC design, the usage of Tile and WPP is signaled by the index named by “tiles_or_entropy_coding_sync_idc” in Sequence Parameter Set (SPS). However, in the case of Entropy Slice, decoder knows the usage of Entropy Slice only after parsing the syntax “entropy_slice_flag” in Slice Header. It is proposed that the syntax “tiles_or_entropy_coding_sync_idc” has to indicate the case of Entropy Slice as other parallel processing support tools like Tile and WPP. This syntax design is also effective to write syntax bits related to Entropy Slice information in Slice Header.

Comment: This is addressed in the text

Comment: Propose to change name of syntax element (editorial)

Recommend: Adopt (text may need improvement; consult with editors)

JCTVC-I0141 Intra mode prediction at entropy slice boundary [B. Li, H. Li (USTC), H. Yang (Huawei)]

Entropy slice is a light-weight parallel mechanism which breaks the entropy decoding status. The intra sample prediction and motion prediction can cross the entropy slice boundary. This contribution discusses the possibility of also making the intra mode prediction across the entropy slice boundary.

Comment: Possible that there is still parsing dependencies for intra-mode

Comment: Logical approach as long as parsing dependency is not present

Recommendation: Adoption conditioned on confirming that parsing dependency is not presentJCTVC-I0237 Specifying entry points to facilitate different decoder implementations [W. Wan, P. Chen (Broadcom)]

This proposal recommends mandating the entry point of every tile and every wavefront substream be signaled instead of the definition in the present draft of the standard which allows an encoder to selectively choose which entry points to transmit.  It claims that different decoder implementations may expect or require the entry points of every tile or wavefront substream to facilitate efficient decoding in their architecture.  An example is given where a single core decoder performing raster scan decoding of tiles would need every entry point to facilitate efficient decoding.  Another example is provided where a multi-core decoder may have difficulties decoding a stream generated with a number of entry points that is not well matched to the number of cores it has available for decoding.  Changes to the text are provided to mandate transmission of every entry point as well as general cleanup of tile processing syntax and semantics.

Proposal: Mandate entry point of every tile/wavefront substream in a bitstream be explicitly signaled.

Multiple participants voiced support for mandating entry points

Comment: Concern about coding efficiency impact

Comment: Mandate is OK if offset information is in slice header

Recommend: Adoption

Editorial action item in entry_point_offset[k-1] and general cleanup

Proposal2: Location of entry points in the bitstream (for example at the beginning of a slice or beginning of a picture).  Example given to include in first slice header.

Proposal2 is withdrawn due to other recommendations.

Addressed as part of recommendation (JCTVC-I359)

JCTVC-I0077 AHG4: Correcting description of bitstream pointer for decoding WPP substreams [Hendry, B. Jeon (LG)]

Proposal assumes interleaved sub-streams.  No longer needed due to recommendation to adopt JCTVC-I0360

JCTVC-I0080 AHG4: Unified marker for Tiles’ and WPP’s entry points [Hendry, B. Jeon (LG)]

Currently, entry points of tiles and WPP substreams can be signalled in the same way by using offset in slice header. In addition to that, entry points to tiles can also be signalled by using special byte pattern as marker within slice data.

This contribution proposes:

- to allow marker to be also used for signalling entry points of WPP substreams.

- to constrain signalling entry point in one location only, either in slice header or in slice data, by adding ‘entry_points_location_flag’ in SPS. The proponent sees no benefit of using both mechanisms at the same time.

- to add offset information after entry point marker.

Proposal 1: Allow WPP to use markers to indicate entry points

Comment: This is already allowed in the text

Not necessary due to other recommendation

Proposal 2: Signal in PPS if markers or entry points are used

Question: Should we allow not signaling any entry information? Makes it difficult for single core decoder.

Comment: Should we allow signaling both entry point information?  This might useful hypothetically.

Not necessary due to other recommendation

Proponent: Want encoder to not mix entry_point information, for example send entry_point_offsets for some tiles/partitions and markers for other tiles/partitions

Proposal 3: Add offset after marker

Comment: It would be possible to have the offset without the marker and this might be more efficient.

Comment: Markers provide enhancement for error resilience

Comment: Not sure if this is needed.

Comment: Concern about hybrid approach of sending offsets in the bitstream

Not necessary due to other recommendation

JCTVC-I0357 Tile entry point signalling [M. Coban, Y. -K. Wang, M. Karczewicz (Qualcomm)]

In the current HEVC specification, tile entry points can be signalled by two different methods. First one being the NAL structure entry offsets signalled in the slice header, the other one being tile start code markers before a tile. This proposal addresses issues with the existing scheme and proposes methods to address the issues in signalling and parsing of tile entry points.

Proposal:

+ Entry points signaled in the slice header should be RBSP offsets that are relative from the previous tile entry point, starting from the end of the slice header, and data should be in RBSP

Comment: Addresses circular issue in determining offset locations

Comment: Previous implementations also included this approach

Recommendation: Adopt moving offsets relative to end of slice header

Recommendation: Revisit RBSP offsets in larger group and after off-line discussion

+ If entry points are signaled then TileID should be present for every tile with entry points

Comment: May not be necessary if entry points for all tiles are mandated

Not necessary due to other recommendation

+ If tile entry markers (0x00002) are used they should be present for every tile

Comment: Signaling all the entry points may be helpful for multiple applications

Not necessary due to other recommendation

+ Presence of entry point offsets in the slice header or tile start code markers are signaled in SPS (PPS because of other recommendation)

Not necessary due to other recommendation

Comment: TileID may provide improved error resilienceJCTVC-I0154 AHG4: Syntax to disable tile markers [C.-W. Hsu, C.-Y. Tsai, Y.-W. Huang, S. Lei (MediaTek)]

In HEVC CD, two methods are provided to locate tile start points in the bitstream. One is tile entry point offset in the slice header. The other is tile entry point maker within the slice data. Tile entry point offset in slice header can be easily disabled by setting num_entry_point_offsets to zero, while tile entry point markers are always sent as long as the number of tiles is greater than 1. In this contribution, we propose a syntax design that can disable tile markers if not necessary.

Similar to JCTVC-I0357, JCTVC-I0080

Propose a flag to disable signaling of tile markers in the PPS

Proponent: Allow signaling entry points and markers at the same time

Spirit to adopt this kind of functionality (I0154, I0357, I0080)

No longer necessary due to other recommendations

JCTVC-I0147 AHG4: Parallel Processing Entry Point Indication For Low Delay Applications [S. Worrall (Aspex)]

To permit parallel decoding of tile or wavefront substreams it is necessary to include indicators in the bitstream, so that the decoder is able to access these substreams. Two approaches currently exist in the Committee Draft [1]: an entry point offset table in the slice header, and tile markers. The entry point offset table approach in general requires fewer bits, but incurs delay. Tile markers allow low delay encoding, but require a 24 bit marker code to be inserted before each substream. This proposal introduces a technique that claims to have lower delay than the entry point table scheme, and requires less overhead than the marker code scheme. The technique is compatible with both tiles and wavefront parallel processing, and it is recommended that this technique is used to replace the two separate schemes that currently exist in the CD.

Proposal: Provide entry point marker for second substream, Followed by offsets interleaved in the bit-stream

+ Replace ue(v)  with fixed length offset bit indicator.

Results compare existing method to proposal. 0.0% for AI, 0.2% for RA and 0.4% for LDB (1.1% for Class E)

Comment: This may be similar to JCTVC-I0080

Comment: The fixed length offset bit indicator does not result in a multiple of 8-bits

Concern: This may create an issue when number of cores of encoder or decoder are not matched.   The amount of computations is larger and also dependent on how the bit-stream is constructed.

Concern: Mixing RBSP and NAL referencing may make this difficult for architectures that handle emulation prevention and decoding as independent stages.  This would require interaction between these operations.

Concern: Reduces latency at encoder only when all sub-streams finish at the same time.

Concern: There are stalls with this method even for single core.

JCTVC-I0579

Crosscheck reports there is a 1 or 2 byte per frame penalty for I0147.  Additionally there is a 1 byte per frame penalty required to signal last offset in slice.

Report that I0159 may be more efficient than I0147.  Using coding scheme of I0159 in I0147 reported to provide better coding efficiency.

JCTVC-I0159 Proposals on entry points signaling [Gordon Clare, Félix Henry, Stéphane Pateux]

Currently, signalling of entry points for tiles and wavefront substreams are done with offsets or markers. Offsets can be used for tiles or wavefront entry points, and are written in the slice header. This contribution proposes that offsets are written at the start of each substream or tile instead. It is asserted that the proposed modifications reduce encoder delay for parallel and single core scenarios. This contribution also proposes that offsets are byte aligned. It is asserted that this byte alignment facilitates offset and substream concatenation. This contribution also proposes that TileID is written after a marker only when tiles_or_entropy_coding_sync_idc is equal to 1 since this syntax element is not used otherwise. Finally, this contribution proposes that one offset per tile is mandated. It is asserted that this modification is necessary to allow picture raster scan decoding of LCUs when multiple tiles are used. The proposed modification of offset entry points produce BD-rate modifications of 0.0%, 0.0%, +0.1% (using WPP) and 0.0%, 0.0%, 0.0% (using tiles) compared to anchor in Intra, Random Access and Low Delay configurations.

Three aspect, each of which is similar to other proposals

First point – do not send tileID when WPP is used

Third point – request for mandatory offsets for tiles to enable picture raster scan decoding

Second point – write the offset at the start of tile/substream and then offsets and the beginning of the following tiles/substreams.  Additionally, the offsets are byte aligned

A difference between JCTVC-I0159 and JCTVC-I0147 is that the offset for the first tile is sent at the beginning.

Comment: Latency may be larger than JCTVC-I0147

Concern: Delay may not be improved for a parallel encoder (delay is already one sub-stream)

Comment: Similar to JCTVC-I0080.  JCTVC-I0080 suggests uses u(v) and not byte aligning

Comment: The problem of encoder delay (motivated here) can also be addressed using markers at some potential expense of R-D performance

Results for WPP (one WPP per CTB line) is 0.0% to 0.1% and with tiles are 0.0% (max of 0.1% for one class)

Proponent: Coding efficiency loss in the results may increase because the size of last tile/substream is not provided in bit-stream.  (This is necessary for the current design.)

JCTVC-I0267 Crosscheck report for Orange's proposal I0159 [Hendry, B. Jeon (LG)] [late]

Results matched.

JCTVC-I0427 AHG4: Category-prefixed data batching for tiles and wavefronts [S. Kanumuri, G. J. Sullivan, Y. Wu, J. Xu (Microsoft)]

This contribution proposes a modification to the formatting of entropy-coded bitstream data in HEVC for use with the tile and wavefront coding features, as originally proposed in JCTVC-G815. The same concept could also apply to PIPE/V2V/V2F entropy coding or other such schemes that include the need to convey different categories of data. In the current HEVC draft design that uses a single method of entry point signalling for tiles and wavefronts (JCTVC-H0556), an index table is used in the slice header to identify the location of the starting point of the data for each entry point. The use of these indices increases the delay and memory capacity requirements at the encoder (to batch up all of the data before output of the index table and the subsequent sub-streams) and at the decoder (to batch up all of the input data in every prior sub-stream category while waiting for the data to arrive in some other category).

This contribution proposes, rather than using the current index table approach, for the different categories of data to be chopped up into batches, and for each batch to be prefixed with a batch type identifier and a batch size indicator. The different categories of data can then be interleaved with each other in relatively-small batches instead of being buffered up for serialized storage into the bitstream data. Since the encoder can emit these batches of data as they are generated, and parallelized decoders can potentially consume them as they arrive, the delay and buffering requirements are asserted to be reduced. It is also asserted that the decoder can skip scanning for start codes within the batch which reduces complexity. Furthermore, if the decoder is not interested in consuming a particular category of data, it is asserted that the decoder can skip the removal of emulation prevention bytes in data corresponding to that category. The contribution also reports a bug in HM 6.1 and proposes that it be fixed as recommended on the HEVC issue tracker.

The average BD bit rate impact, comparing the proposal to HM 6.1 as the reference, is asserted to be 0.0% for a representative All-Intra configuration, 0.1% for a representative Random Access configuration and 0.2% for a representative Low Delay configuration.

Proposal: inter-leave the data from multiple tiles/sub-streams within the bit-stream.  Categories represent one or more tiles (or one or more substreams).

This proposal was previously proposed as JCTVC-G0815.

Bit-rate comparison:

For Tiles: 0.0% for AI, .2% for RA, .3% for LDB, .3% for LP compared to current method (slice header)

For WPP: 0.0/0.1% for AI, 0.2% for RA, .3% for LDB and LP

Compared to tile markers: 0.0% change for all sequences (with bug fix 490)

Concern: How to deal with MTU size matching?  Solution would require adding delay to address this situation and proposal may not improve latency in that situation.

Comment: This changes the bit-stream order of CTBs in the bit-stream.  This may create issues for a single core decoder

Concern: This may not be useful for WPP processing.  Asserted that a constraint could address the issue by ensuring CTBs are ordered in the bit-stream appropriately.

Comment: Number of batches is restricted.  Possible to address this in a future proposal.

Comment (multiple): Is this better handled at the system layer?  Asserted to be better to handle in the VCL for decoder parallelization.

Comment: Should only push functionality to a system layer that is specific to that system layer.  If functionality is applicable to multiple system layer systems then it (the functionality) should be in the video coding specification.

Comment: Without slice size limits, the proposal is friendly for encoders.  With slice size limits, the proposal does not provide additional functionality.  (Asserted by proponent to not be true.)  Discussion to continue off-line.

Comment: Relationship with ASO in H.264/AVC.  Appears similar but ASO is in slice level.  Might be good to have ASO capability in new specification.

Question: Are results available for 1 CTB or sub-stream?

Concern (multiple): This increases difficulties for a single core decoder.  Proposal requires additional demuxer or stich/processing to reassemble data before sending to a CABAC engine.

Comment: Other proposals would be preferable

No action recommended

NOTES below relate to discussion of inter-leaved signaling (something like JCTVC-I0159):

Comment: Need further description of use cases and latency needs

Comment: If we don’t know we need something better, keep the current design of transmitting all the offsets in a slice in the slice header

Comment: Useful to keep the offset information together (in the slice header).

Comment: May need a tile/sub-stream id for an entry point if all of the tile/sub-stream locations are not sent

Consensus in the room is that the benefits of interleaved offsets require more study and better understanding of the application needs for latency reduction and benefits.

Comment: The need for reduced latency for this application is not well established given total system design (packetization, etc.)

Comment: Applications that need to be sub-slice and will use the parallelization tools is unclear

Comment: Packetization is slice based in the vast majority of applications

Comment: One participant has observed that extremely low latency applications do not run over a network that requires packetization (such as RTP)

Comment: At least one participant did not fully agree with the previous comment.

Comment: Rewriting the slice header does not hurt the latency of video transmission over RTP

Comment: If you have packetization, there is a delay due to packetization.  This allows a system to put information in the slice header without additional delay.

Consensus in the room is that any application needs for low latency (as currently addressed by entry point markers) should be dealt with at the slice level.

Note: Other proposals at this meeting address the problem in this slice level manner (JCTVC-0070, JCTVC-0229)

Recommendation: Remove entry point markers (specifically the technology signaled with 0x000002 in the CD) from the CD

JCTVC-I0448 AHG4: Cross-verification of JCTVC-I0427 entitled category-prefixed data batching for tiles and wavefronts [M. Zhou (TI)] [late]

JCTVC-I0456 Cross-check of AHG4: Category-prefixed data batching for tiles and wavefronts (JCTVC-I0427) [M. Horowitz, S. Xu (eBrisk) [late]

JCTVC-I0070 Nested hierarchy of tiles and slices through slice header prediction [M. M. Hannuksela, A. Hallapuro (Nokia)]

It is observed in this document that the primary difference between a tile-shaped slice and a tile included in a slice (as one of many tiles included in the same slice) is the presence or absence of the slice header. In the HEVC CD, a tile may contain one or many complete slices, or a slice may contain one or many complete tiles.

This contribution proposes the following items:

- A picture delimiter NAL unit may carry a slice header, which may be used for decoding of more than one slice of the picture.

- A slice header beyond the slice address need not be provided for any slice.

- A slice header may be selectably predicted from the previous slice in scan order or from a slice header carried within the picture delimiter NAL unit.

- The tile marker is proposed to be removed from the slice_data( ) syntax. For similar purposes as a tile marker was earlier used, a slice (typically with a short header) can be used.

Revision 1 of the contribution includes source code that implements the proposed changes and provides simulation results. When a slice size of about 36 LCUs of size 64x64 was used, the proposed slice header prediction provided about 1.5% BD-rate reduction on average in low-delay B main configuration when compared to HM6.0. When compared to HM6.0 with one slice per picture and a tile size about 6x6 LCUs of size 64x64, the proposal provided about 0.6% BD-rate increase on average in low-delay B main configuration.

Revision 2 attempts to clarify the relation of the proposal to tiles and tile markers. The proposed changes in slice_data( ) were updated.

Proposal – picture delimiter NAL unit may carry a slice header may be used for decoding of more than one slice of the picture

Proposal – slice header beyond the slice address need not be provided for any slice

Results show approximately 1.5% reduction (5% for LD, Class E)

Proposal – slice header may be selectively predicted from previous slice

Proposal – tile marker removed (not discussed in detail because of previous recommendation)

Results show that use of short headers compared to tile markers provides a coding efficiency loss of 0.6% on average for LDB

Recommendation: Review in larger group

JCTVC-I0362 Virtual line buffer model and restriction on asymmetric tile configuration [S. Kumar, G. Van der Auwera, M. Coban, Y.-K. Wang, M. Karczewicz (Qualcomm)]

It is proposed to restrict asymmetry of tile configurations in order to reduce loop filtering (Deblocking, Sample Adaptive Offset, Adaptive Loop Filter) line buffer requirement based on a proposed Virtual loop filter line buffer model.

Proposal: Encoder constraint on the width or height of tiles

Currently have a restriction of 384 pixels for tile width

Proposes to have a “total virtual line buffer size” bound.  For a 4k-by-2k picture, line buffer savings are more than 6KB.

Question: Are there examples for the restriction?  Yes.

Question: Is there a case where a system could not use a specific number (or larger) of tiles? Possibly.

Question: Is it possible to divide picture into N column tiles?

For vertical tiles, restriction is on tile width

Restriction is on number of LCUs

Comment: May need additional study.  General support for motivation to reduce implementation cost.

Comment: Needs additional information and support to make the concept clear

Recommendation: Further study encouraged

JCTVC-I0462 AHG4: Profile Requirements For Facilitation Of Parallel Tile Decoding [S. Worall (Aspex)] [late]

The current CD defines a single restriction on tiles: tiles should be greater than 384 pixels in width. The current restriction can only be used by applications that include capability negotiation (e.g. videoconferencing), so that the encoded bitstream can be tailored to the number of cores on the encoder and decoder. To allow parallel decoding for streaming and broadcast applications, tighter restrictions are needed to facilitate flexible encoding and decoding with different numbers of cores. This contribution proposes some restrictions on tile sizes, which are intended to allow parallel decoding by as wide a range of applications as possible.

Two options

- Specify large tile sizes

- Specify smaller, fixed tile sizes

Option two is preferred to better enabled parallel decoding, load balancing for example.

Comment: Uniform spacing may not be good for load balancing in a software encoderConfirmation: Goal of proposal to guarantee a specific tile portioning for a frame size

Comment: Tiles can be disabled currently.  Even given that tiles can be disabled, there is support for the concept since some decoders can exploit the behavior.

Comment: There is some flexibility in the tile locations (confirmed)

Comment: Constraint on tile height is not desired

Comment: Agree that constraints are necessary for enabling parallel decoding

Comment: Some participants prefer a minimum number of tiles

Recommendation: Review in larger group

JCTVC-I0198 On Tiles and Wavefront tools for parallelism [J.-M. Thiesse, J. Viéron (Ateme)]

Not presented.

Recommendation: Review in larger group