| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0582 |
| --- | --- |

| Title: | Performance evaluation of DST in intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kemal Ugur Oguz Bici | Tel: Email: | +358 50 4860857 kemal.ugur@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This document presents performance results of several configurations of utilizing DST in intra prediction with the aim of simplifying the current design. We tested two simplifications for this purpose. It is asserted that experimental results show that significant simplifications are possible for HEVC without changing coding efficiency.

# Introduction

To the 9th JCT-VC meeting, there had been proposals to send different quantization matrices for DST transform [1][2]. The reason for transmitting separate quantization matrices is that DST has different energy compaction properties than DCT, so using the same non-flat quantization matrices for both DCT and DST could have visual artifacts.

The HEVC CD supports utilizing DST for 4x4 intra prediction blocks. The configurations for DST are as follows:

- DCT in both directions (DCT-DCT)

- DST in horizontal and DCT in vertical directions (DST-DCT)

- DST in vertical and DCT in horizontal directions (DCT-DST)

- DST in both directions  (DST-DST)

These configurations basically mean that the residual of a 4x4 intra block could be transformed in four different ways. This complicates the decoder design for both software and hardware implementations. Also, three additional quantization matrices are proposed to be signalled in [1][2], which complicates the design.

During the discussion of these proposals, some experts questioned the usefulness of using DCT-DST adaptation in HEVC. In order to evaluate the performance of DST and potential simplifications we tested some simplifications.

Simplification 1: We use DST for all intra prediction modes of 4x4 blocks. The benefits of this are:

- No need to support DCT-DST and DST-DCT transforms.

- As transform type does not depend on intra prediction mode, simplification in decoder is achieved.

- The visual artifacts of using quantization matrices and DST are removed.

The experimental results show that this configuration shows 0.1% BDR difference on average for all intra case.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.1% | -0.3% | -0.2% | -0.1% | -0.3% | -0.3% |
| Class B | 0.0% | -0.2% | -0.2% | 0.0% | -0.2% | -0.2% |
| Class C | 0.2% | -0.2% | -0.2% | 0.2% | -0.2% | -0.1% |
| Class D | 0.2% | -0.1% | -0.2% | 0.3% | -0.1% | -0.1% |
| Class E | 0.0% | -0.2% | -0.2% | 0.0% | -0.3% | -0.2% |
| Overall | 0.1% | -0.2% | -0.2% | 0.1% | -0.2% | -0.2% |
|  | 0.1% | -0.2% | -0.2% | 0.1% | -0.2% | -0.2% |

Simplification 2: In this simplification, we always use DST-DST for all prediction modes except DC prediction (i.e. transform type does not depend on prediction direction). The experimental results show that this configuration shows 0.0% BDR difference on average for all intra case.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.1% | -0.2% | -0.2% | -0.1% | -0.2% | -0.2% |
| Class B | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| Class C | 0.1% | -0.1% | -0.1% | 0.1% | -0.1% | -0.1% |
| Class D | 0.1% | -0.1% | -0.1% | 0.1% | -0.1% | -0.1% |
| Class E | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% |
| Overall | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
|  | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |

# WD Text

Simplification 1:

Depending on PredMode and IntraPredMode, the following applies:

–	If PredMode is equal to MODE_INTRA, Log2(nW*nH) is equal to 4, and cIdx is equal to 0, the variables horizTrType and vertTrType are set equal to 1. specified as Table 8-2 with IntraPredMode as input.

–	Otherwise, the variables horizTrType and vertTrType are set equal to 0.

Table 8-1 – Specification of horizTrType and vertTrType

| IntraPredMode | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| vertTrType | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| horizTrType | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

| IntraPredMode | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| vertTrType | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| horizTrType | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Simplification 2:

Depending on PredMode and IntraPredMode, the following applies:

–	If PredMode is equal to MODE_INTRA, Log2(nW*nH) is equal to 4, and cIdx is equal to 0, and IntraPredMode is not equal to INTRA_DC the variables horizTrType and vertTrType are set equal to 1. specified as Table 8-2 with IntraPredMode as input.

–	Otherwise, the variables horizTrType and vertTrType are set equal to 0.

Table 8-2 – Specification of horizTrType and vertTrType

| IntraPredMode | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| vertTrType | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| horizTrType | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

| IntraPredMode | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| vertTrType | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| horizTrType | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

# References

[1] JCTVC-I0268, “Quantization matrices for 4x4 DSTs in HEVC”, J. Lou, L. Wang (Motorola Mobility)

[2] JCTVC-I0419, "Syntax for supporting 4x4 DST quantization matrices", R. Cohen, A. Vetro (MERL)

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).