| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0584 |
| --- | --- |

| Title: | BoG Report: Memory Bandwidth Reduction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Teruhiko Suzuki 2-10-1, Osaki, Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2740 teruhikos@jp.sony.com |
| Source: | BoG on memory bandwidth reduction |  |  |

_____________________________

# Introduction

This BoG was established in to study contributions related to memory bandwidth issues in HEVC. During the discussion in JCTVC meeting, it was general agreed that restrictions for memory bandwidth reductions are desirable, and can be achieved for reasonable penalty. This BoG is formed to further discuss the proposals and suggest one or two preferred solutions.

# Summary of proposals

The proposals can be categorized into two groups.

## Group 1: MV restrictions

JCTVC-I0075 (Toshiba)

- Eencoder only trick, no syntax change

- Restrict to chroma integer position

- 4x8 bi-pred is prohibited

- For 8x4, One MV of L0/L1 is restricted to integer position of chroma

- DIS text is available. It includes normative restrictions in Annex A

JCTVC-I0351 (Qualcom)

- No syntax change, no parsing change, decoder change

- Case 1: vertical MV is restricted to integer position

  - Apply 8x4 and 4x8 bi-pred

- Case 1.1: The same technique is applied to both horizontal and vertical MV

- Case2: The same technique is apply to 8x8 (8x8 only vertical)

- MV rounding in MC process

- DIS text is available

## Group 2: PU restrictions

JCTVC-I0107 (Sony and Sharp)

- Level dependent small PU restriction

- When restricted, encoder don’t choose merge bi-pred as candidate

- Syntax change (on level dependent signalling) and decoding process change

- change derivation process of merge candidate

- if no level dependent change, no syntax change is necessary

- DIS text is available. It includes changes in derivation process of merge candidate

JCTVC-I0297 (JVC)

- Case 1: birepd is changed to uni pred for 4x8 and 8x4 PU

- This change is applied just after reconstructing MV for merge and AMVP

- Storage of MV is not needed

- MV is transmit for both L0/L1, but discarded if not necessary

- No change in MC process

- Case 2: The conversion is applied in MC process

- No syntax change

- DIS text is available

JCTVC-I0425 (TI&Broadcom)

- Comparison of 216, 107, 425 was shown

- Proposal is close to I0297 (Case 1)

- The only one difference is that inter pred direction flag is not encoded. (It is encoded in I0297)

- DIS text is available

# Discussion

## Group 1: MV restrictions

JCTVC-I0075:

- Good point is no additional complexity of decoder is needed.

- Two experts express the concern on encoder side constraint. In AVC, sometimes encoder doesn’t apply such constrain. Decoder may not be able to decode such bitstreams. It is very hard to detect if bitstream include such constraints or not.

- In case of AVC, bitstream analyzer maybe difficult to implement because constraint is for two continuous MBs. It is an issue how easily detect evil bitstream. This can be achieved by a flag.

- If decoding process changes is specified, encoder is more difficult to generate illegal bitstream. Then bad encoder can be reduced. From this point of view, I0075 method has less advantage compared with other proposals.

JCTVC-I0351 (Qualcom)

- One expert expressed interest, because loss is minimum among all proposals.

- Concern that it may cause on visual quality, especially vertical pan sequence.

- Such degradation may not be serious.

- uni-pred can be used, then problem can be avoided.

- In AVC, to restrict fractional MV, jerkiness can be observed. (not confirmed for HEVC yet)

- No consensus on visual quality concern, nobody checked the visual quality yet.

- Same concern on restricting small block size was expressed. (for all proposals)

- Encoder time is still the same, while encoder restricts MV to integer.

- ME is the same as HM. After ME, MV is rounded if necessary. Bi-pred ME, but no subpel refinement for 8x4 and 4x8 is used, while other proposal doesn’t use only use uni-pred ME for subpel refinement.

## Group 2: PU restrictions

JCTVC-I0107 (Sony and Sharp)

- Interleave of merge candidate derivation and restriction is not clean solution.

- Difference between I0297 and I0107 is the place to change merge candidate. In I0107, merge candidate is changed during derivation process, while I0297 proposal changes just before MC process.

- BD loss is almost the same, but slightly lower than I0297 (case 1).

- Due to this, experts prefer I0297 rather than I0107.

- This proposal does not interleaved too much, but proponent can be agree to use I0297 (case 1).

JCTVC-I0425 (TI&Broadcom)

- The only difference from I0297 is coding of inter pred direction flag.

- If people prefer solution without syntax change, I0297 is better solution.

- Proponent agree an prefer in I0297 (case 1), because it is more simple than case 2.

JCTVC-I0297 (JVC)

- Case 1 is better solution

## Suggested Solutions

BoG agreed I0351 (case 1.1) is the best among group 1 and I0297 (case 1) is the best among group 2. BoG continue to discuss which methods is better.

Group 1: MV restrictions

JCTVC-I0351 (case 1.1)

1 experts supported this, because

- BD rate loss is smaller than I0297

Group 2: PU restrictions

JCTVC-I0297 (case 1)

3 experts supported this, because

- Change of the text is smaller than I0351

- BD rate loss is reasonable (smaller than other proposals except I0351)

- Number of access to small block (e.g. 4x8, 8x4) is smaller (no consensus on this comment)

| Proponents |  | JVC (JCTVC-I0297) | Qualcomm (JCTVC-I0351) |
| --- | --- | --- | --- |
|  |  | Case1 | Case 1.1 |
| Ave. gain (Luma) | A | 0.05% | 0.02% |
|  | B | 0.18% | 0.06% |
|  | C | 0.40% | 0.15% |
|  | D | 0.58% | 0.36% |
|  | E | 0.15% | 0.08% |
|  | All | 0.30% | 0.14% |
| Time | Enc | 93.50% | 99.80% |
|  | Dec | 99.25% | 100.50% |
| Worst case (2D4x2) |  | -36% | -36% |
| Worst case (2D 8x2) |  | -44% | -44% |
| Worst case (1D 4x1) |  | -31% | -31% |
| Worst case (1D 8x1) |  | -50% | -44% |
| Bandwidth Avg. |  | -40% | -39% |

Question:

What is the worst case coding loss in I0351 and I0297 ? (Loss in main)

I0351:

BQ sequare (class D) LBmain 0.8 %

I0297:

BQ squeare (class D) LBmain 0.9 %

During the BoG discussion, people prefer to bring those two options to JCTVC meeting and make the final decision in JCTVC main meeting, not in BoG.

# Conclusion

BoG recommends to review two options I0351 (case 1.1) and I0297 (case 1) as candidate of the memory bandwidth reduction method. One of two should be included in DIS.