| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0585 |
| --- | --- |

| Title: | Report on subjective viewing of ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Tomoo Yamakage | Tel: Email: | tomoo.yamakage@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

Two subjective viewing results for ALF are reported. The first viewing is about JCTVC-I0169 (CE2.f) and the second viewing is about ALF off versus Option2_RA_ALF on (JCTVC-I0157). Results of the first viewing (Viewing1) showed that the proposal of JCTVC-I0167 is not better than the HM6 anchor. Results of the second viewing (Viewing2) showed that Option2_RA_ALF showed better subjective picture quality for six test points out of forty test points and one test point showed ALF off is better.

# Viewing conditions

Viewing conditions for both sessions 1 and 2 are as follows:

- April 29, 2012 at T072 in ITU-T tower building

- Some monster PC (Windows 7 – 64 bit)

- Dell 3008 LCD (30 inches, 2560x1600)

- Qp = 32, 37

- 5-grade scale

-2: A is worse than B

-1: A is slightly worse than B

0: Equal

1: A is slightly better

2: A is better

- 18 viewers

- Viewing order is randomized

# Viewing1 results (CE2.f, JCTVC-I0169[1])

The following sequences are used:

- BasketballDrill, HE10 Low Delay B, Qp32

- BasketballDrill, HE10 Low Delay B, Qp37

- Johnny, HE10 Low Delay B, Qp37

Anchor is HM6 HE10.

The following table shows the results for all 18 viewers:

If the proposal is better than the anchor, Mean value is positive and confidence interval will not cross the value of 0.

Two test points showed that the confidence interval cross the value of 0, therefore, these test points did not indicate that the proposal was better than the anchor.

One test point showed that the proposal was worse than the anchor (i.e., Mean value is minus and confidence interval does not cross the value of 0).

# Viewing2 results (ALF_off versus Option2_RA_ALF, JCTVC-I0157[2])

The following sequences are used:

1:	KristenAndSara (1280x720),		Low Delay B,		Qp37

2:	Johnny (1280x720), 			Low Delay B,		Qp37

3:	FourPeople (1280x720),	 		Low Delay B,		Qp37

4:	SpinCalendar (1280x720),	 	Low Delay B,		Qp37

5:	RiverBed (1920x1080), 		Low Delay B,		Qp37

6:	Kimono (1920x1080), 			Low Delay B,		Qp37

7:	ParkScene (1920x1080),		Low Delay B,		Qp37

8:	Cactus (1920x1080), 			Low Delay B,		Qp37

9:	BasketballDrive (1920x1080), 		Low Delay B,		Qp37

10:	BQTerrace (1920x1080),		Low Delay B,		Qp37

11:	Traffic (2560x1600),			Random Access,	Qp37

12:	PeopleOnStreet (2560x1600), 		Random Access,	Qp37

13:	NebutaFestival (2560x1600), 		Random Access,	Qp37

14:	SteamLocomotive (2560x1600),		Random Access,	Qp37

15:	RiverBed (1920x1080), 		Random Access,	Qp37

16:	Kimono (1920x1080), 			Random Access,	Qp37

17:	ParkScene (1920x1080), 		Random Access,	Qp37

18:	Cactus (1920x1080), 			Random Access,	Qp37

19:	BasketballDrive (1920x1080), 		Random Access,	Qp37

20:	BQTerrace (1920x1080),		Random Access,	Qp37

21:	KristenAndSara (1280x720),		Low Delay B,		Qp32

22:	Johnny (1280x720), 			Low Delay B,		Qp32

23:	FourPeople (1280x720),	 		Low Delay B,		Qp32

24:	SpinCalendar (1280x720),	 	Low Delay B,		Qp32

25:	RiverBed (1920x1080), 		Low Delay B,		Qp32

26:	Kimono (1920x1080), 			Low Delay B,		Qp32

27:	ParkScene (1920x1080),		Low Delay B,		Qp32

28:	Cactus (1920x1080), 			Low Delay B,		Qp32

29:	BasketballDrive (1920x1080), 		Low Delay B,		Qp32

30:	BQTerrace (1920x1080),		Low Delay B,		Qp32

31:	Traffic (2560x1600),			Random Access,	Qp32

32:	PeopleOnStreet (2560x1600), 		Random Access,	Qp32

33:	NebutaFestival (2560x1600), 		Random Access,	Qp32

34:	SteamLocomotive (2560x1600),		Random Access,	Qp32

35:	RiverBed (1920x1080), 		Random Access,	Qp32

36:	Kimono (1920x1080), 			Random Access,	Qp32

37:	ParkScene (1920x1080), 		Random Access,	Qp32

38:	Cactus (1920x1080), 			Random Access,	Qp32

39:	BasketballDrive (1920x1080), 		Random Access,	Qp32

40:	BQTerrace (1920x1080),		Random Access,	Qp32

Anchor is HM6 Main (ALF off).

The following table shows the results for all 18 viewers:

If the proposal is better than the anchor, Mean value is positive and confidence interval will not cross the value of 0.

Sequences with red circle showed that Option2_RA_ALF is better than the anchor. Six test points out of forty test points showed better subjective picture quality of Option2_RA_ALF than the anchor.

Sequences with brown circle showed that Option2_RA_ALF is worse than the anchor. Only one test point out of forty test points showed worse subjective picture quality of Option2_RA_ALF than the anchor.

Sequences with green circle are marked as informational purposes. These sequences showed the lowest confidence interval value was larger than -0.1.

The following table shows the file size of each bistream:

Five test points exceeded two percent tolerance of file size. However, these test points are not included in the six test points that Option2_RA_ALF is better than the anchor.

# Patent rights declaration(s)

TOSHIBA Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- S. Esenlik, M. Narroschke, T. Wedi, “Grid displacements for ALF,” Document of Joint Collaborative Team on Video Coding, JCTVC-I0169, April, 2012.

- C.-Y. Tsai, C.-Y. Chen, C.-M. Fu, Y.-W. Huang, S. Lei, T. Yamakage, T. Itoh, T. Watanabe, T. Chujoh, I. S. Chong, M. Karczewicz, “AHG6: Baseline options for ALF,” Document of Joint Collaborative Team on Video Coding, JCTVC-I0157, April, 2012.