| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0586-r3 |
| --- | --- |

| Title: | BoG report on I_PCM / lossless loop filtering unification |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Keiichi Chono Geert Van der Auwera Matthias Narroschke Byeungwoo Jeon Hiroya Nakamura Félix Henry | Tel: Email: | chono@ct.jp.nec.com geertv@qualcomm.com Matthias.Narroschke@eu.panasonic.com bjeon@skku.edu nakamura.hiroya@jvckenwood.com felix.henry@orange-ftgroup.com |
| Source: | BoG on I_PCM / lossless loop filtering unification |  |  |

_____________________________

# Summary and recommendation

Mandate

- Harmonize deblocking operation in I_PCM case with the solution suggested in JCTVC-I0529 base text on JCTVC-I0030.

Item

- (Editorial) Fix the text bug of the deblocking of the original lossless coding in Ticket 476: Missing text regarding bypassing deblocking filter process in lossless coding.

- (Editorial) Update the bypass descriptions of lossless coding in deblocking/SAO/ALF according to the new syntax elements (i.e., replace "qpprime_y_zero_transquant_bypass_flag is equal to 1 and QP’Y is equal to 0" with "transquant_bypass_enable_flag is equal to 1 and cu_transquant_bypass_flag equal to 1.").

- (Technical) Unify QP derivation in deblocking process of I_PCM and lossless coding according to the meeting notes on JCTVC-I0529.

Discussion

All attendees (including non proponents) agreed using the average of QPY regardless of block types in deblocking process.

Through the editorial work, it was discovered that JCTVC-I0529 introduces a new functionality to the current I_PCM design. Specifically, cu_transquant_bypass_flag can be used to control loop filter on/off on I_PCM regions. The proponent of JCTVC-I0529 does not intend to introduce such a new functionality to the current I_PCM design. BoG decided to have the following constraint in the semantics in pcm_flag to avoid having the new functionality (an alternative description is attached in the appendix.):

pcm_flag specifies whether the associated coding unit with PART_2Nx2N is coded by I_PCM: If the pcm_flag is equal to 1, the associated coding unit with PART_2Nx2N is coded by I_PCM. When the pcm_flag is not present, it shall be infered to be equal to 0. It is a requirement of bitstream conformance that when pcm_flag is equal to 1, the value of cu_transquant_bypass_flag of the associated coding unit shall be equal to pcm_loop_filter_disable_flag.

There was comment from non proponent that such a restriction is unnecessary since the spec is stable regardless of the restriction.

Recommendation

- Adopt using the average of QPY regardless of block types in deblocking process (JCTVC-I0529). (Text is attached. Changes to JCTVC-I0030 are highlighted with magenta.)

- Decide whether the constraint in the combination of cu_transquant_bypass_flag, pcm_flag, and pcm_loop_filter_disable_flag is necessary or not.

Appendix: Alternative description of pcm_flag constraint

pcm_flag specifies whether the associated coding unit with PART_2Nx2N is coded by I_PCM: If the pcm_flag is equal to 1, the associated coding unit with PART_2Nx2N is coded by I_PCM. When the pcm_flag is not present, it shall be infered to be equal to 0. It is a requirement of bitstream conformance that when the value of cu_transquant_bypass_flag of the associated coding unit is not equal to pcm_loop_filter_disable_flag, pcm_flag shall be equal to 0.